# Submission ID #: 61671

# Scriptwriter Name: Bridget Colvin

Project Page Link: https://www.jove.com/account/file-uploader?src=18809703

Title: Controlled Strain of 3D Hydrogels under Live Microscopy Imaging

Authors and Affiliations: Avraham Kolel1, Avishy Roitblat Riba2, Sari Natan3, Oren Tchaicheeyan3, Eilom Saias,2 and Ayelet Lesman3,4

1Department of Biomedical Engineering, Faculty of Engineering, Tel-Aviv University

2Department of Material Science and Engineering, Faculty of Engineering, Tel-Aviv University

3School of Mechanical Engineering, Faculty of Engineering, Tel-Aviv University

4 Center for the Physics and Chemistry of Living Systems

# Corresponding Author:

# Avraham Kolel

# avrahamkolel@mail.tau.ac.il

# Co-Authors:

avishee@gmail.com

sazor8@gmail.com

toren101@gmail.com

eilom122@gmail.com

ayeletlesman@tauex.tau.ac.il

## Author Questionnaire

1. Microscopy: Does your protocol require the use of a dissecting or stereomicroscope for performing a complex dissection, microinjection technique, or similar?  N

2. Software: Does the part of your protocol being filmed demonstrate software usage?  Y

If Yes, we will need you to record using screen recording software to capture the steps. If you use a Mac, QuickTime X also has the ability to record the steps. Please upload all screen captured video files to your project page as soon as reasonably possible.

3. Interview statements: Considering the Covid-19-imposed mask-wearing and social distancing recommendations, which interview statement filming option is the most appropriate for your group? Please select one.

☒ 	Interviewees wear masks until the videographer steps away (≥6 ft/2 m) and begins filming. The interviewee then removes the mask for line delivery only. When the shot is acquired, the interviewee puts the mask back on. Statements can be filmed outside if weather permits.

4. Filming location: Will the filming need to take place in multiple locations (greater than walking distance)?   N

Protocol Length

Number of Shots: 43

# Introduction

- Introductory Interview Statements

REQUIRED:

  - Avraham Kolel: Tissues are grown in 3-dimensional soft environments, through which cells are exposed to various mechanical cues. Our method allows the stretching of hydrogels in a 3-dimensional manner for the investigation of biomechanical responses [1].

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera

REQUIRED:

  - Avraham Kolel: This technique enables the uniform stretching of hydrogels along their thickness while live imaging is performed. Additionally, the hydrogel’s geometry can be manipulated to any size or shape [1].

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera

OPTIONAL:

  - Ayelet Lesman: Knowledge about how cells and the extracellular matrix respond to forces is important for revealing how tissues develop and diseases progress, leading to the potential development of cancer therapies and tissue engineering models [1].

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera

# Protocol

- Silicone Strip Preparation

  - Begin by using a laser or manual cutter to create a rubber strip with a hole in the center [1].

    - WIDE: Talent cutting pieces

  - Cut a rectangle of hydrophobic film so that it is wider than the silicone strips [1] and wash a plastic dish with 70% ethanol [2].

    - Talent cutting layer(s)

    - Talent washing dish, with ethanol container visible in frame

  - After drying with lint free wipes, place the sealing film into the dish [1] place one silicone strip with the plastic wrap removed from one side into the center of each piece of sealing film [2].

    - Talent placing film into dish

    - Talent removing plastic and placing strip into dish

- Fibrin Gel Polymerization

  - For fibrin gel formation, uniformly deliver 2.5 microliters of cooled, labeled fibrinogen [1] into the silicone cut-out of each strip so that the entire circumference of each cut-out is in contact with fibrinogen, taking care not to allow any air-pockets or bubbles to form anywhere in the solutions [2].

    - WIDE: Talent loading pipette with fibrinogen Videographer: Important/difficult step

    - Fibrinogen being added to cutout, with fibrinogen container visible in frame Videographer: Important/difficult step

  - Immediately add 2.5 microliters of cooled thrombin directly to each fibrinogen solution [1] and quickly mix the solutions with careful pipetting, moving the tip around the entire mixture to create as homogenous a solution as possible [2].

    - Thrombin being added to fibrinogen, with thrombin container visible in frame Videographer: Important/difficult step

    - Mixture being pipetted Videographer: Important/difficult step

  - After mixing, place the covered dish in the incubator for 30 minutes for gel polymerization [1].

    - Talent placing dish into incubator

  - At the end of the incubation, add enough PBS to the dish to submerge the gel-silicone constructs [1] and carefully lift each sample from the dish, making sure the sealing film layer remains adhered to the strip [2].

    - Construct being submerged

    - Sample being lifted

  - Slowly peeling from one end of the silicone to the other, carefully detach the sealing film from each piece of silicone [1-TXT].

    - Film being peeled from silicone Videographer: Important step TEXT: Avoid pulling areas close to the cut-out where stress concentrations may exist

  - Then place the strip back into the dish [1] and place the dish on the stage of a light microscope to assess the condition of the sample [2].

    - Talent placing strip into dish

    - Talent placing dish onto stage

- Sample Loading

  - To load the sample onto the stretching device, fill the bath with PBS [1] and place the silicone strip containing the sample gel across the top of the bath so the ends of the strip are sitting on each side of the bath [2].

    - WIDE: Talent filling bath

    - Talent placing strip across bath

  - Place and tighten the clamps and fabric strips so that all of the pieces are connected to form one straight strip with the cut-out in the center [1].

    - Clamps and strips being tightened

  - Place an aluminum liquid well into the stretching device [1] and use a sealing solution to secure a glass coverslip to the bottom of the well [2].

    - Talent placing well into device

    - Talent coverslip into material in well, with material container visible in frame

  - Fill the well with 1-2 milliliters of PBS/cell medium [1] and place the strip-fabric-gel construct into the device [2].

    - Talent filling well, with solution container visible in frame

    - Talent placing construct into device

  - Clamp the fabric strip into the bracket so that the cut-out of the gel is in the center [1] before carefully placing and locking the pin-down insert into place in the device [2].

    - Talent clamping strip in bracket Videographer: Important step

    - Talent placing/locking down insert in device Videographer: Important step

  - Place and lock other fabric side into the spindle [1] and place the stretching device and the attached sample onto the stage of a confocal microscope [2].

    - Talent locking fabric side into spindle

    - Talent place device onto microscope stage

  - Use a USB cable to connect the microcontroller to the computer [1] and connect the servomotor to the microcontroller [2].

    - Talent connecting microcontroller to computer

    - Talent connecting servomotor to microcontroller

  - Then open the stretching device control module on the computer to image the sample [1].

    - Talent opening module, with monitor visible in frame

- Sample Adequacy Confirmation

  - To determine whether the sample is adequate for experimentation, use low-resolution live imaging to scan the entire gel [1] and determine the lowest Z-position at which full adhesion to the inner walls of the cut-out is apparent with no tears or bubbles [2].

    - WIDE: Talent scanning gel, with monitor visible in frame

    - SCREEN: To be provided by Authors: Shot of lowest Z-position at which full adhesion apparent

  - After noting the Z-location of the microscope, scan the interface of the fluorescent label of the gel and silicone strip to determine the full adhesion of the gel to the silicone throughout its circumference [1].

    - SCREEN: To be provided by Authors: Interface being scanned

  - After scanning, move the stage in the Z-direction until there is no longer continuity in the gel and note the upper limit of the Z-position [1].

    - SCREEN: To be provided by Authors: Stage being moved until no continuity

  - Then subtract the upper limit of the Z-direction from the lower limit to calculate the sample thickness [1-TXT].

    - BLACK TEXT WHITE BACKGROUND: Zο=Zu-Zl Video Editor: please emphasize Z-U with “upper limit”, Z-L with “lower limit”, and Z-o with “sample thickness”

- Stretching Device Operation, Stretching, and Imaging

  - To determine the pre-stretched position of the sample, click Go To Zero Servo Pos in the module software to make sure the servomotor is at its zero position [1] and attach the servomotor to the stretching device, taking care not to put any strain on the sample [2].

    - WIDE: Talent clicking button, with monitor visible in frame

    - Talent attaching servomotor to device

  - While imaging, click the plus 1 button to move the motor one degree at a time in the clockwise direction [1]. When the right side of the cut-out moves, click the minus 1 button to reverse the sample to the penultimate position to maintain the sample under minimal tension [2].

    - SCREEN: To be provided by Authors: +1 button being clicked

    - Right side moving, then minus 1 button being clicked/position being reversed

  - Click Set Min Servo Position to set the reference position [1-TXT] and capture a 40x magnification, high resolution, single Z-slice tile image of the entire gel area as a baseline reference for the post-processing analysis [2].

    - SCREEN: To be provided by Authors: Set Min Servo Position being clicked TEXT: Click Go To Min Servo Position to return to reference position at any time

    - SCREEN: To be provided by Authors: Image being acquired

  - Capturing a single Z-slice image in the middle of the gel thickness is also recommended to facilitate returning to approximately the same Z-position after stretching [1-TXT].

    - SCREEN: To be provided by Authors: Middle of gel image being acquired TEXT: High-resolution tile images required 20-30 min

  - When all of the images have been acquired, advance the servomotor one degree per second until the desired stretch magnitude is reached [1].

    - SCREEN: To be provided by Authors: Sample being stretched/servomotor position being adjusted

  - At each stretch magnitude for which an analysis is desired, verify that the gel has not detached from the silicone at any point throughout its circumference by scanning the interface between the gel and the silicone [1]. Then, capture a high-resolution tile Z-stack (Z-thickness - Zo) image set of the entire gel area for post-processing analysis [2].

    - SCREEN: To be provided by Authors: Stretched Z-slice image

    - Use 6.6.1. Video Editor: please emphasize red gel and black interface when mentioned

## Protocol Script Questions

A. Which steps from the protocol are the most important for viewers to see? Please list 4 to 6 individual steps.

3.1., 3.2., 3.5., 4.5.

B. What is the single most difficult aspect of this procedure and what do you do to ensure success? Please list 1 or 2 individual steps from the script above.

3.1., 3.2. Creating homogenous gels that completely attach to the silicone is critical for the success of this experiment. To ensure success, use cold fibrin components and perform the mixing protocol quickly while moving the pipette tip to cover the entire geometric cut-out.

# Results

- Results: Representative Gel Strain Stretching and Gel Fiber Alignment Analysis

  - Zooming in and manually tracking bead aggregates during gel stretching [1] allows calculation of the local gel strains in the axial and perpendicular directions [2].

    - LAB MEDIA: Figures 9A and 9B Video Editor: please zoom into the dashed square for one image from Figure 9A to show corresponding magnification from Figure 9B

    - LAB MEDIA: Figures 9B and 9C Video Editor: please keep showing zoomed image from 8.1.1., add Figure 9C, and emphasize Egel text and arrow

  - Typically, the axial strains propagate relatively linearly from the silicone cut-out edge to the center of the gel and are larger than the compressive perpendicular strains [1].

    - LAB MEDIA: Figure 9D Video Editor: please emphasize red data line in Figure 9D

    - LAB MEDIA: Figure 9D Video Editor: please emphasize green data line in Figure 9D

  - Here high-magnification images [1] of a typical un-stretched and relatively isotropic hydrogel [2] and a hydrogel under high 80% cut-out strain, depicting highly aligned fibers in the stretch direction, are shown [3].

    - LAB MEDIA: Figures 11A and 11B

    - LAB MEDIA: Figures 11A and 11B Video Editor: please emphasize Figure 11A

    - LAB MEDIA: Figures 11A and 11B Video Editor: please emphasize Figure 11B

  - Analysis of the fiber reorientation [1] revealed an approximately linear dependence between the fiber alignment and the external strain on the cut-out up to strains of 40% [2], with moderate saturation at strains above 40% [3].

    - LAB MEDIA: Figures 11A, 11B, and 11D

    - LAB MEDIA: Figures 11A, 11B, and 11D Video Editor: please emphasize data lines from 0-40%

    - LAB MEDIA: Figures 11A, 11B, and 11D Video Editor: please emphasize data lines from 40-80%

  - As illustrated in this fiber alignment analysis [1], as the external strain on the cut-out increases [2], the fiber alignment increases, following an overall non-linear curve [3].

    - LAB MEDIA: Figure 12

    - LAB MEDIA: Figure 12 Video Editor: please sequentially emphasize top left graph and green data line together then bottom left graph and blue data line together

    - LAB MEDIA: Figure 12 Video Editor: please emphasize blue data point in Figure 12C

  - Note that the gel response to stretch is relatively uniform along its Z-thickness [1].

    - LAB MEDIA: Figure 12B Video Editor: please emphasize the colored straight and horizontal lines of Figure 12B

  - As illustrated in this preliminary finite element simulation performed on 2D continuous material [1], the color-map ranges from 38-42%, indicating that the gel strains are relatively homogenous throughout the circular gel area [2].

    - LAB MEDIA: Figure 13

    - LAB MEDIA: Figure 13 Video Editor: please emphasize heat map distribution in image from center to top and bottom of circle

# Conclusion

- Conclusion Interview Statements

  - Avraham Kolel: Take care that sample the gels achieve complete adhesion to the inner walls of the geometric cut-out, as a complete sample adhesion and continuity is crucial for a reliable sample stretching [1].

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera (3.1., 3.6., 5.1.-5.4.)

  - Avraham Kolel: Altering the shape of the hydrogel by modifying the cut-out geometry enables the generation of programmable gradients in stretch and fiber alignment for the analysis of cell responses to these gradients [1].

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera