Summary of Model Reconstruction

in Agisoft Metashape Pro

- Import/Add photos

  - Make sure orientation of photos are consistent.

- Align Photos

  - One option is to begin using the settings listed below. Then, troubleshoot as needed.

    - Accuracy:  High

- Generic Preselection

- Reset current alignment (only check ON if previously aligned photos)

Advanced:

Key point limit – 40,000

Tie Point limit – 4,000

Apply Masks to – usually does not apply, will be greyed out.

- Adaptive camera fit modeling

- Gradual Selection

  - To help reduce the number of potential error points used.

    - Reprojection Error - 0.4

    - Reconstruction Uncertainty - 200

    - Projection Accuracy - 20

- Add Markers, Scale References, and Elevation information

  - Add markers in photos with the GPS coordinates added in the References Pane.

  - Add in depths (z-values) for elevation.

  - Create scale references.

- Optimize Cameras

  - Prior to optimizing cameras, in the Reference Pane, check ON any GPS markers and/or scale references if they wish to be used to help optimize cameras.

  - Use default settings in Optimize Camera settings.

- Build Dense Cloud

  - Quality:  Medium

Depth filtering:  Mild

- Calculate point colors

  - Edit dense cloud to delete any obvious points that will affect DEM generation.

- Build DEM

  - Source data:  Dense Cloud

Interpolation:  Enabled (default)

Point Classes:  All

- Export DEM

  - Use default settings.

- Build Orthomosaic

  - Surface:  DEM

Blending Mode:  Mosaic

Pixel Size:  Change accordingly. For example: 0.001 m is for 1 mm/pixel resolution.

- Export Orthomosaic

  - Export as TIFF. May need to check the box for BigTIFF since file size may be too large for regular TIFF export.

- Build 3D mesh model

  - Source data: Depth Maps

  - Surface type: (greyed out Arbitrary 3D)

  - Quality: Medium

  - Face count: Low

  - Interpolation: Enabled (default)

- Calculate vertex colors

- Build texture

  - Texture type: Diffuse map

  - Source data: Images

  - Mapping mode: Generic

  - Blending mode: Mosaic (default)

  - Texture size/count: 4092 x 1

- Enable hole filling