# Submission ID #: 61911

# Scriptwriter Name: Bridget Colvin

Project Page Link: https://www.jove.com/account/file-uploader?src=18882903

Title: An Improved Assay and Tools for Measuring Mechanical Nociception in Drosophila Larvae

Authors and Affiliations: Roger Lopez-Bellido1 and Michael J. Galko1,2,3

1Department of Genetics, UT MD Anderson Cancer Center

2Neuroscience Graduate Program, Graduate School of Biomedical Sciences, The University of Texas M.D. Anderson Cancer Center

3Genetics and Epigenetics Graduate Program, Graduate School of Biomedical Sciences, The University of Texas M.D. Anderson Cancer Center

# Corresponding Authors:

# Michael J. Galko

# mjgalko@mdanderson.org

# Roger Lopez-Bellido

rlopez5@mdanderson.org

## Author Questionnaire

1. Microscopy: Does your protocol require the use of a dissecting or stereomicroscope for performing a complex dissection, microinjection technique, or similar?  Y, Zeiss Stemi 2000 stereomicroscope and Leica MZ6 stereomicroscope

2. Software: Does the part of your protocol being filmed demonstrate software usage?  N

3. Interview statements: Considering the Covid-19-imposed mask-wearing and social distancing recommendations, which interview statement filming option is the most appropriate for your group? Please select one.

X 	Interview Statements are read by Authors outside. If the weather does not permit this, interview statements will be read by JoVE’s voiceover talent.

4. Filming location: Will the filming need to take place in multiple locations (greater than walking distance)?   N

Protocol Length

Number of Shots: ~24

# Introduction

- Introductory Interview Statements

REQUIRED:

  - Michael J. Galko or JoVE Voiceover Talent: This protocol is significant because it allows the user to build a tool that can be used to precisely measure mechanical nociception responses in genetically tractable Drosophila larvae [1].

    - INTERVIEW: Named talent says the statement above in an interview-style shot outside, looking slightly off-camera OR Use script footage

REQUIRED:

  - Roger Lopez-Bellido or JoVE Voiceover Talent: The main advantage of this technique is that it uses simple materials to build custom tools that can be used to measure mechanical nociception from the sub-threshold to fully responsive range [1].

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera OR Use 2.1., 2.4., and/or 2.6.

OPTIONAL:

  - Michael J. Galko or JoVE Voiceover Talent: These tools and methods can be used to measure baseline mechanical nociception and nociceptive sensitization following injury or disease [1].

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera OR Use 4.3.2. Probe being applied

OPTIONAL:

  - Roger Lopez-Bellido: Probing larvae with custom Von Frey filaments takes practice. Users should be able to generate reproducible dose-response curves with control larvae before attempting an experiment [1].

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera OR Use 4.4.1. Talent recording response

# Protocol

- Mechanical Probe Construction

  - To construct a mechanical probe, use a small wire cutter to cut each nitinol filament to the specified length perpendicular to its long axis [1].

    - WIDE: Talent cutting filament

  - The filaments come in three different pre-set diameters [1].

    - LAB MEDIA: Figure 1B

  - Examine the tip of the filaments under a stereomicroscope [1] to ensure that no sharp or irregular edges remain that could cause tissue damage to the larva skin and interfere with the calibration [2].

    - Talent placing filaments under microscope

    - SCOPE: Shot of filament tip(s)

  - Then use a sharpening stone to manually smooth the edges of the probe until no sharp irregularities persist [1].

    - Probe being sharpened Videographer: Important step

  - Next, use a hypodermic needle to poke a hole near the end of a wooden popsicle stick [1].

    - Hole being made

  - Apply wood glue to a single nitinol filament [1] and insert the glue-coated filament into the needle slot [2-TXT].

    - Glue being applied Videographer: Important step

    - Filament being inserted Videographer: Important step TEXT: Allow glue to dry approximately 5 h

  - When the glue has dried, press the probe against a scale until the probe bends to determine the maximum force that can be recorded in grams [1-TXT].

    - Probe being pressed against scale, with scale readout visible in frame Videographer: Important step

  - Use the formula to convert the recorded mass to force in millinewtons [1-TXT]. Then divide the measured force [2] by the surface area of the filament tip [3] to convert the calculated force to kilopascals of pressure [4].

    - BLACK TEXT WHITE BACKGROUND: f = ma

    - LAB MEDIA: Figure 1MVideo Editor: please emphasize force column

    - LAB MEDIA: Figure 1M Video Editor: please emphasize area column

    - LAB MEDIA: Figure 1M Video Editor: please emphasize pressure column

  - Preparing multiple probes using filaments of different diameters and lengths will generate a full set spanning the responsive range for Drosophila larvae [1-TXT].

    - Shot of range of probes OR LAB MEDIA: Figure 1N TEXT: Check probes every 3-4 wks; Replace when pressure deviates  3% from original measure

- Preparation of Larvae

  - To prepare Drosophila larvae for an experiment, raise the larval progeny on standard food in a 25-degree Celsius incubator for about 96 hours [1].

    - WIDE: Talent placing progeny into incubator

  - When the larvae reach the third instar stage, pour the soft fly food contents into a clean standard size Petri dish [1] and use forceps to gently sort the medium-sized, mid third instar larvae from the smaller, second and early third instar and larger, late or wandering third instar larvae [2].

    - Talent pouring food contents into dish

    - Larvae being sorted Video Editor: please emphasize smaller second/early third and larger late/wandering larvae when mentioned

  - Then use the forceps to transfer 20-30 mid third instar larvae into a small Petri dish containing a small plug of fly food moistened with water at room temperature [1].

    - Talent adding larvae to dish with fly food plug

- Mechanical Nociception Assay

  - To perform a mechanical nociception assay, use forceps to place a mid-third instar larva onto a dark, thin, vinyl pad under a bright field stereomicroscope [1-TXT] and arrange optical fiber lights between the microscope objective lenses and the pad [2].

    - WIDE: Talent placing larva onto pad Videographer: Important step TEXT: Discard larvae that do not exhibit normal locomotion following pad transfer

    - Talent arranging lights Videographer: Important step

  - Use a paper towel to wipe away any excess water surrounding the larva [1] and move the pad to orient the head and mouth of the larva toward the non-dominant hand of the researcher [2].

    - Water being wiped Videographer: Important step

    - Pad/larva being oriented Videographer: Important step

  - Select a mechanical probe [1] and apply the probe to the posterior dorsal side of the larva at approximately abdominal segment A8 for 1-2 seconds [2], carefully compressing the larvae into the underlying pad at the point of probe contact until the probe bends and elicits the previously measured amount of pressure [3].

    - Talent selecting probe Videographer: Important step

    - Probe being applied Videographer: Important/difficult step

    - LAB MEDIA: Video 3: 00:08-00:12 Videographer: Important/difficult step

  - A positive nociceptive response is indicated if the larva shows a complete corkscrew roll of 360 degrees along the axis of its body within 3 seconds [2]. Record the behavioral response for each larva [1].

    - Talent at computer, recording response, with monitor visible in frame NOTE: Place 4.4.2 before 4.4.1.

    - LAB MEDIA: Video 2: 00:06-00:11

  - Then discard the tested larva [1] and prepare the next larva for the assay [2-TXT].

    - Talent discarding larva

    - Talent placing larva onto pad TEXT: Repeat 3-6 sets of 10 larvae/probe

## Protocol Script Questions

A. Which steps from the protocol are the most important for viewers to see? Please list 4 to 6 individual steps.

2.4., 2.6., 2.7., 4.1.-4.3.

B. What is the single most difficult aspect of this procedure and what do you do to ensure success? Please list 1 or 2 individual steps from the script above.

4.3. User has to be patient coming onto the larva with the probe, especially for the lower pressure ones that can be a little wobbly.

# Results

- Results: Representative Mechanical Nociception and Hypersensitivity

  - These customized mechanical probes with nitinol filaments can be used to elicit mechanically evoked behaviors [1] and to generate a full behavioral dose response curve using both innocuous and noxious mechanical probes of varying intensity [2].

    - LAB MEDIA: Figure 2D

    - LAB MEDIA: Figure 2D Video Editor: please emphasize dose response curve

  - As these behavioral assay results demonstrate, probes that exert pressures of 200 kilopascals or less [1] do not provoke an aversive rolling response in Drosophila larvae [2].

    - LAB MEDIA: Figure 1M Video Editor: please emphasize P1 and P2 probe rows

    - LAB MEDIA: Video 3: 00:04-00:16

  - As expected, these subthreshold, or non-noxious, mechanical probes do not elicit visible neuronal tissue damage [1].

    - LAB MEDIA: Figure 2E Video Editor: please emphasize 200 kPa image

  - Conversely, suprathreshold, or noxious, probes elicit an augmented behavioral response [1] as well as tissue damage to the peripheral sensory neurons themselves in a dose dependent manner [2].

    - LAB MEDIA: Figure 2D Video Editor: please emphasize data points from 462-5116

    - LAB MEDIA: Figure 2E Video Editor: please emphasize 462-5116 kPa images

  - The probes can also be used to measure mechanical hypersensitivity after injury [1].

    - LAB MEDIA: Figure 3B

  - Approximately 20% of larvae respond with aversive rolling as early as 2 hours after UV treatment [1], while 50% respond at 4 hours [2] compared to mock UV-irradiated animals [3]. Because the probe used for this analysis was a normally subthreshold 200 kilopascal, this response was considered to be mechanical allodynia [4].

    - LAB MEDIA: Figure 3B Video Editor: please emphasize 2h UV data bar

    - LAB MEDIA: Figure 3B Video Editor: please emphasize 4h UV data bar

    - LAB MEDIA: Figure 3B Video Editor: please emphasize 2 and 4 h mock data bars

    - LAB MEDIA: Figure 3B

  - At later time points, the behavioral response of the UV-treated larvae is slightly increased but not statistically different than that of the mock-irradiated control group [1].

    - LAB MEDIA: Figure 3B Video Editor: please add/emphasize ns and brackets over 8, 16, and 24 h data bars

  - Larvae probed with a 462-kilopascal probe at 4, 8, and 16 hours following UV treatment exhibit a significant increase in the aversive rolling response [1], with 4 hours being the peak of the behavioral hypersensitivity [2]. As this stimulus was initially noxious, the response was considered to be mechanical hyperalgesia [3].

    - LAB MEDIA: Figure 3C Video Editor: please add/emphasize P= texts and brackets over 4, 8, and 16 h data bars

    - LAB MEDIA: Figure 3C Video Editor: please emphasize 4h UV data bar

    - LAB MEDIA: Figure 3C

# Conclusion

- Conclusion Interview Statements

  - Roger Lopez-Bellido or JoVE Voiceover Talent: The probe preparation and larva selection and compression are critical to the success of the procedure [1].

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera (2.4, 2.6., 2.7., 4.1-4.3.) OR Use any of 2.4, 2.6., 2.7., 4.1-4.3.

  - Roger Lopez-Bellido or JoVE Voiceover Talent: Using these probes, both the baseline nociception and mechanical nociceptive hypersensitivity can be measured following injury [1].

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera OR Use 4.3.2. Probe being applied

  - Michael J. Galko or JoVE Voiceover Talent: Using these tools and assay, the molecular and genetic bases of mechanical nociception and nociceptive hypersensitivity are able to be measured in the genetically tractable Drosophila model [1].

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera OR LAB MEDIA: Figure 2D