# Submission ID #:  61917

# Scriptwriter Name: Domnic Colvin

# Project Page Link: https://www.jove.com/account/file-uploader?src=18884733

# Title:   Electric-Field-Induced Neural Precursor Cell Differentiation in Microfluidic Devices

# Authors and Affiliations:

Hui-Fang Chang1, Shih-En Chou1, Ji-Yen Cheng1,2,3,4

1Research Center for Applied Sciences, Academia Sinica, Taipei11529, Taiwan

2Institute of Biophotonics, National Yang-Ming University, Taipei 11221, Taiwan

3Department of Mechanical and Mechatronic Engineering, National Taiwan Ocean University, Keelung 20224, Taiwan

4College of Engineering, Chang Gung University, Taoyuan 33302, Taiwan

# Corresponding Authors:

# Ji-Yen Cheng

# jycheng@gate.sinica.edu.tw

# Email Addresses for All Authors:

# emily16@gate.sinica.edu.tw

# angel30711@gmail.com

# jycheng@gate.sinica.edu.tw

## Author Questionnaire

1. Microscopy: Does your protocol require the use of a dissecting or stereomicroscope for performing a complex dissection, microinjection technique, or something similar?  No

2. Software: Does the part of your protocol being filmed include step-by-step descriptions of software usage?  No

3. Interview statements: Considering the COVID-19-imposed mask-wearing and social distancing recommendations, which interview statement filming option is the most appropriate for your group? Please select one.

☒ 	Interview Statements are read by JoVE’s voiceover talent.

4. Filming location: Will the filming need to take place in multiple locations?   Yes

If Yes, how far apart are the locations? 27.22 meters

Current Protocol Length

Number of Steps:  21

Number of Shots:  39

# Introduction

- Introductory Interview Statements

REQUIRED:

  - This study presents a protocol for the differentiation of neural precursor cells solely induced by direct current pulse stimulation in a microfluidic system.

    - 4.3.1.

  - This technique provides a beneficial approach for reducing the experimental setup size, sample volume, and reagent volume. Furthermore, the MOE chip is suitable for confocal microscopy observations.

    - 4.4.4.

OPTIONAL:

  - The simple direct current pulse treatment can control mouse neural precursor cells’ fate and could be used to develop therapeutic strategies for nervous system disorders.

    - 5.1.1, Figure 7.

# Protocol

- Design and fabrication of the multichannel, optically transparent, electrotactic chip and coating poly-L-lysine on the substrate in the cell culture regions

  - To begin, draw patterns for individual PMMA layers and the double-sided tape using appropriate software [1-TXT].

    - Talent drawing patterns using AutoCAD software. TEXT: PMMA- polymethyl methacrylate

  - Switch on the carbon dioxide laser scriber [1] and connect it to a personal computer [2]. Open the designed pattern file using the software [3].

    - Talent switching on the laser scriber.

    - Talent connecting the laser scriber to the computer.

    - Talent opening the desired file in the software.

  - Place the PMMA sheets or double-sided tape on the platform of the laser scriber [1], then focus the laser onto the surface of the PMMA sheets or the double-sided tape using the auto-focus tool [2].

    - Talent placing the PMMA sheets or the tape on the platform of laser scriber.

    - Talent focusing the laser on the surface of the tape and sheets.

  - Select the laser scriber as the printer, then use the laser scriber to start the direct ablation on the PMMA sheet or double-sided tape [1].

    - Talent selecting the laser scriber as printer.

  - Remove the protective film from the PMMA sheets [1], and clean the surface using nitrogen gas [2].

    - Talent removing the protective film from the PMMA sheets.

    - Talent cleaning the surface of the film using nitrogen gas.

  - For bonding together multiple layers of PMMA sheets, stack three pieces of 1-millimeter PMMA sheets [1] and bond them under a pressure of 5 kilograms per square centimeter in a thermal bonder for 30 minutes at 110 degrees Celsius to form the flow or electrical stimulation channel assembly [2].

    - Talent stacking the layers of PMMA sheets.

    - Talent bonding the layers in thermal bonder.

  - Adhere 12 pieces of adaptors to the individual openings in Layer 1 of the MOE chip assembly with fast-acting cyanoacrylate glue [1-TXT].

    - Talent adhering 12 pieces of adaptors with glue. TEXT: MOE- multichannel, optically transparent, electrotactic

  - Disinfect the 1-millimeter PMMA substrates, the double-sided tape, and the 3-millimeter optical grade PMMA using ultraviolet irradiation for 30 minutes before assembling the chip [1].

    - Talent disinfecting the substrates, tape, and PMMA using UV.

  - Adhere the 1-millimeter PMMA substrates on the 3-millimeter optical grade PMMA with the double-sided tape to complete the PMMA assembly [1]. Adhere the cleaned cover glass to the PMMA assembly with the double-sided tape [2]. Videographer: This step is important!

    - Talent adhering the substrates on PMMA sheets with tape.

    - Talent adhering the cover glass to the PMMA assembly.

- Coating poly-L-lysine on the substrate in the cell culture regions and preparation of the salt bridge network

  - Seal the openings of the agar bridge adaptors with white finger-tight plugs [1]. Connect the flat-bottom connector to the MOE chip assembly via the medium inlet and outlet adaptors [2-TXT], then connect the cone-Luer adaptor to the 3-way stopcocks [3].

    - Talent sealing the openings of the agar bridge adaptors with white finger tight plugs.

    - Talent connecting the connector to MOE chip assembly. TEXT: MOE- multichannel, optically transparent, electrotactic

    - Talent connecting the cone-Luer adaptor to the stopcocks.

  - Add 2 milliliters of 0.01% PLL solution using a 3-milliliter syringe that connects to the 3-way stopcock of the medium inlet [1-TXT] and connect an empty 3-milliliter syringe to the 3-way stopcock of the medium outlet [2].

    - Talent adding PLL solution to the 3-way stopcock. TEXT: PLL- poly-L-lysine

    - Talent connecting empty syringe to the 3-way stopcock.

  - Fill the cell culture regions with the PLL solution [1] and slowly pump the coating solution back and forth [2]. Close the two 3-way stopcocks to seal the solution inside the culture regions [3]. Videographer: This step is important!

    - Talent filling the PLL solution in cell culture regions.

    - Talent pumping the coating solution back and forth.

    - Talent closing both the 3-way stopcocks.

  - Incubate the MOE chip at 37 degrees Celsius overnight in an incubator filled with 5% carbon dioxide atmosphere [1].

    - Talent incubating the MOE chip in the incubator.

  - Open the two 3-way stopcocks [1] and flush away bubbles in the channels by manually pumping the coating solution back and forth in the channel using the two syringes [2].

    - Talent opening the two 3-way stopcocks.

    - Talent pumping the coating solution in the channel.

  - Draw 3 milliliters of complete medium into a 3-milliliter syringe that connects to the 3-way stopcock of the medium inlet [1] and add it to replace the coating solution in the cell culture regions [2].

    - Talent drawing complete medium into syringe.

    - Talent adding the complete medium in cell culture regions to replace the coating solution.

  - Replace the white finger-tight plug with the Luer adaptor [1] and inject 3% hot agarose to fill the adaptor [2]. Videographer: This step is difficult and important!

    - Talent replacing the tight plug with Luer adaptor.

    - Talent injecting agarose in the adaptor.

  - Connect the Luer lock syringe to the Luer adaptor [1] and inject 3% hot agarose through the black rubber bung to fill the Luer lock syringe. Allow 10 to 20 minutes for the agarose to cool down and solidify [2]. Videographer: This step is important!

    - Talent connecting Luer lock syringe to the adaptor.

    - Talent injecting hot agarose to the Luer lock syringe.

- Setup of the microfluidic system for DC pulse stimulation

  - Install the cell-seeded MOE chip onto the transparent ITO heater that is fastened on a programmable X-Y-Z motorized stage maintained at 37 degrees Celsius [1-TXT].

    - Talent installing the MOE chip onto ITO heater. TEXT: ITO-Indium-Tin-Oxide

  - Infuse the mNPCs by manual pumping into the MOE chip via the medium outlet [1-TXT], then incubate the cell-seeded MOE chip on the ITO heater for 4 hours [2]. Videographer: This step is important!

    - Talent infusing the mNPCs into the MOE chip. TEXT: mNPCs- mouse neural stem and progenitor cells

    - Talent incubating the MOE chip on ITO heater.

  - Use electrical wires to connect an EF multiplexer to the MOE chip via the electrodes on the chip [1-TXT]. Connect an EF multiplexer and a function generator to an amplifier to output square-wave DC pulses [2-TXT].

    - Talent connecting EF multiplexer to MOE chip. TEXT: EF-electric fields

    - Talent connecting an EF multiplexer and a function generator to an amplifier. TEXT: 300 mV/mm magnitude, 100 Hz frequency at 50% duty cycles (50% time-on and 50% time-off)

  - Remove all the flat bottom connector and adaptors on the MOE chip [1] and fill the adaptors with PBS [2], then seal the adaptor openings with parafilm [3]. After immunostaining, observe the cells using a confocal fluorescence microscope [4]. Videographer: This step is important!

    - Talent removes all of the Flat bottom connector and Luer adaptor on MOE chip.

    - Talent fills the PBS into the adaptors.

    - Talent sealing the openings of adaptors with parafilm.

    - Talent observing the cells under fluorescence microscope.

# Results

- Differentiation of the mNPC cells in the control group and in the DC pulse stimulation group at DIV 3, 7, and 14

  - The DC pulse stimulation resulted in simultaneous mNPCs differentiating into neurons, astrocytes, and oligodendrocytes in stem cell maintenance medium [1].

    - LAB MEDIA: Figure 7

  - The percentages of neurons, astrocytes, and oligodendrocytes in the control group [1] and in the stimulation group are shown here [2].

    - LAB MEDIA: Figure 7A-7C.

    - LAB MEDIA: Figure 7D-7F.

  - After the DC pulse treatment, the mNPCs expressed significantly high numbers of neurons at DIV 7 [1]. Astrocytes at DIV 3 [2] and oligodendrocytes at DIV 7 and DIV 14 were present at relatively higher levels in the stimulation groups than in the control group [3].

    - LAB MEDIA: Figure 7B and 7E. Video editor focus on the yellow region in pie chart.

    - LAB MEDIA: Figure 7A and 7D. Video editor focus on the grey region in pie chart.

    - LAB MEDIA: Figure 7B, 7C, 7E and 7F. Video editor focus on the blue region in pie chart.

# Conclusion

- Conclusion Interview Statements

  - Neurospheres formed by 30 to 40 cells are preferred for initiating mNPC differentiation. Overgrowth of mNPCs will impair cell survival during the differentiation process.

    - 4.2.1.