# Submission ID #:  61978

# Scriptwriter Name: Domnic Colvin

# Project Page Link: https://www.jove.com/account/file-uploader?src=18903338

# Title: A Versatile Kit Based on Digital Microfluidics Droplet Actuation for Science Education

# Authors and Affiliations:

Yu-Hao Guo, Chih-Hsien Lee, Ya-Tang Yang

Electrical Engineering, National Tsing Hua University, Hsinchu, Taiwan

# Corresponding Authors:

# Ya-Tang Yang		(ytyang@ee.nthu.edu.tw)

# Email Addresses for All Authors:

# S104035891@m104.nthu.edu.tw

# love50816@gmail.com

# ytyang@ee.nthu.edu.tw

## Author Questionnaire

1. Microscopy: Does your protocol require the use of a dissecting or stereomicroscope for performing a complex dissection, microinjection technique, or something similar?  No

2. Software: Does the part of your protocol being filmed include step-by-step descriptions of software usage?  No

3. Interview statements: Considering the COVID-19-imposed mask-wearing and social distancing recommendations, which interview statement filming option is the most appropriate for your group? Please select one.

☒ 	Interview Statements are read by JoVE’s voiceover talent.

4. Filming location: Will the filming need to take place in multiple locations?   No

Current Protocol Length

Number of Steps:  12

Number of Shots:  28

# Introduction

- Introductory Interview Statements

REQUIRED:

  - We present a digital microfluidics educational kit based on commercially sourced printed circuit board that allows the user to get hands-on experience with digital microfluidics. This is a viable, low-cost solution for education, provided that digital PCB design files can be shared.

    - Figure 1 a, then b.

  - We propose to use digital microfluidics as an educational tool because droplets are manipulated on generic electrode-array platforms. Users can leverage on an extended set of electronic components, now highly accessible for do-it-yourself applications, to electronically interface with the droplets.

    - 2.3.1, 2.3.2.

# Introduction of Demonstrator on Camera

  - Demonstrating the procedure will be Yu Hao Guo, a graduate student from the Yang laboratory.

    - INTERVIEW: Author saying the above.

    - The named demonstrator(s) looks up from workbench or desk or microscope and acknowledges the camera.

# Protocol

- Assembling the digital microfluidics kit and preparation of insulator on the electrode array

  - Begin by soldering the surface mount resistors, transistors, and light-emitting diodes onto the PCB board [1-TXT]. Videographer: This step is difficult and important!

    - Talent soldering the electrical components on PCB board. TEXT: PCB- Power circuit board

    - NOTE: Same as 2.1.1, different angle.

  - Connect the output of the high-voltage power supply board to the PCB board with the soldered components [1-3], then connect the battery to the voltage booster board to boost the voltage from 6 volts to 12 volts [4-6]. Videographer: This step is important!

    - Added shot: Solder the power supply board to PCB

    - Talent connecting the PCB board to the power supply.

    - NOTE: may be an added shot here, not sure based on the author’s notes. Maybe the previous shot was split in two.

    - Talent connecting battery to the voltage booster.

    - Added shot: Connecting 3 wires to booster board

    - Added shot: Solder to XL6009

  - Connect the humidity sensor [1], the ultrasonic piezo atomizer, and the atomizer driver board to the microcontroller board [2-3]. Videographer: This step is important!

    - Talent connecting the humidity sensor to the microcontroller board. Videographer focus on the humidity sensor and microcontroller board.

    - Talent connecting the atomizer and atomizer driver board to the microcontroller board.

    - Added shot: A shot on full assembly

  - Turn on the microcontroller using the provided supplementary code [1]. Adjust the variable resistor of the high-voltage board [2] and use the digital multimeter to measure the voltage of the EWOD electrode [3-TXT].

    - Talent turning on the microcontroller.

    - Talent adjusting the variable resistor of the high-voltage board.

    - Talent measuring voltage of EWOD electrode using multimeter. TEXT: Output voltage should be ~230 V

  - Wear clean nitrile gloves [1] and apply 10 microliters of 5 centistokes Silicone oil on the electrode area using a micropipette [2]. Spread the oil evenly on the electrode area using a finger [3].

    - Talent wearing nitrile gloves.

    - Talent applying Silicone oil to the electrode area.

    - Talent spreading the oil.

  - Cut a piece of food wrap with dimensions 2.5 by 4 centimeters [1] and place it on top of the electrode [2]. Apply silicone oil on the electrode area using a micropipette [3] and spread it evenly [4].

    - Talent cutting piece of food wrap.

    - Talent placing food wrap on the electrode area.

    - Talent applying silicone oil on the electrode area.

    - Talent spreading the oil using finger.

- Chemiluminescence, fluorescent imaging, and long-term droplet actuation experiment

  - To perform a chemiluminescence experiment, place 2 to 5 microliters of luminol solution on the target electrode using a micropipette [1]. Place 10 microliters of 0.1% potassium ferricyanide on the electrode, which can be moved as a droplet for electrowetting [2].

    - Talent placing luminol solution on the target electrode.

    - Talent placing potassium ferricyanide on the electrode.

  - Turn on the microcontroller so that the potassium ferricyanide droplet merges with the luminol [1].

    - Talent turning on the microcontroller. NOTE: May have been split in 2 shots.

  - For fluorescent imaging, cut a 1 square centimeter piece of semi-transparent tape [1] and place it between the excitation light-emitting diode and EWOD electrodes [2].

    - Talent cutting a piece of semi-transparent tape.

    - Talent placing the tape between light-emitting diode and EWOD electrodes.

  - Attach the emission glass filter on the camera of the smartphone with a tape [1] and place 10 microliters of the potassium ferricyanide solution on the electrodes [2]. Record the video of the droplet actuation using a smartphone [3].

    - Talent attaching emission glass filter on the camera of smartphone.

    - Talent placing potassium ferricyanide on the electrode.

    - Talent recording the video of droplet actuation.

  - For long-term droplet actuation, place 1 milliliter of water onto the ultrasonic atomizer [1-TXT]. Videographer: This step is important!

    - Talent placing water on the atomizer. TEXT: Maintain humidity level over 90%

  - Place a droplet of potassium ferricyanide [1] and turn on the microcontroller [2], then immediately close the lid of the enclosure [3]. Check the droplet actuation after an hour [4].

    - Use shot 3.1.2

    - Talent turning on the microcontroller.

    - Talent closing the lid.

    - Talent checking the droplet actuation.

# Results

- The analysis of the droplet movement in fluorescent imaging and under the control of ultrasonic atomizer

  - For the chemiluminescence experiment, the droplet of ferricyanide is actuated to move and mix with the pre-deposited luminol droplet on the target electrode at 12 seconds [1].

    - LAB MEDIA: Figure 3. Video editor focus on the burst of blue light at t=12s.

  - A schematic setup of an LED serving as the light source for excitation, a semi-transparent clear office tape as a light diffuser, and the emission filter directly attached to the smart phone camera is shown here [1].

    - LAB MEDIA: Figure 4a.

  - Fluorescent imaging of the droplet containing fluorescein isothiocyanate in the dark is seen as a result of the semi-transparent tape serving as the diffuser to evenly distribute the excitation light [1].

    - LAB MEDIA: Figure 4b.

  - For a long-term experiment, successful droplet actuation can be observed [1]. Representative humidity data under the action of an ultrasonic atomizer is shown here [2].

    - LAB MEDIA: Figure 5a.

    - LAB MEDIA: Figure 5b.

# Conclusion

- Conclusion Interview Statements

  - This protocol can be used to develop an educational kit based on digital microfluidics. A luminol-based chemiluminescence experiment is reported as a specific example. The kit can be assembled within a short period of time and with minimal training in electronics.

    - 2.3.3.

  - The simplified experiment described here can be extended to other experiments. For example, a paper test kit can be used by moving the droplet to the paper to be adsorbed. A microcontroller with interface logic circuit can also be added to provide more sophisticated digital control and programmability.

    - 3.6.1.

  - This protocol can benefit non-professional enthusiasts to learn and apply electronics to further advance their knowledge of the field.

    - 3.6.4.