SUPPLEMENTAL FILE

TITLE:

Lumped-Parameter and Finite Element Modeling of Heart Failure with Preserved Ejection Fraction

AUTHORS AND AFFILIATIONS:

Luca Rosalia1,2, Caglar Ozturk1, Ellen T. Roche1,2,3

1Institute for Medical Engineering and Science, Massachusetts Institute of Technology, Cambridge, Massachusetts 02139, USA

2Health Science and Technology Program, Harvard/Massachusetts Institute of Technology, Cambridge, Massachusetts 02139, USA

3Department of Mechanical Engineering, Massachusetts Institute of Technology, Cambridge, Massachusetts 02139, USA

L.R. and C.O. contributed equally to this work.

Luca Rosalia		(lros@mit.edu)

Caglar Ozturk		(cozturk@mit.edu)

Corresponding author:

Ellen T. Roche		(etr@mit.edu)

Lumped-parameter hydraulics (isothermal) library: Details on the elements used in the lumped-parameter simulation can be found here.

Table S1: Geometric and mechanical parameters of baseline lumped-parameter simulation.

|  | Density [kg m-3]1 | Kinematic viscosity [mm2 s-1]1 | Bulk modulus [GPa]2 |
| --- | --- | --- | --- |
| Blood | 1060 | 3.55 | 2.2 |
|  | Diameter [cm]3-6 | Length [cm]3-5,7-9 | Compliance [m Pa-1] |
| Left ventricle | 4.4 | 7 | 0.002–1 × 10-6 |
| Right ventricle | 4.8 | 5 | 1.8–5.5 × 10-5 |
| Left atrium | 5 | 7.5 | 0.8–1.7 × 10-5 |
| Right atrium | 5.6 | 5 | 1.9–2.3 × 10-5 |
| Ascending aorta | 2.65 | 8.8 | 4.0 × 10-7 |
| Descending aorta | 2.15 | 6.9 | 7.0 × 10-7 |
| Thoracic aorta | 1.92 | 33.2 | 1.7 × 10-7 |
| Pulmonary artery | 3 | 5 | 2.2 × 10-7 |
| Superior vena cava (SVC) | 2.1 | 7.1 | 5.0 × 10-6 |
| Abdominal inferior vena cava (IVC) | 1.4 | 17 | 4.4 × 10-6 |
| Thoracic IVC | 1.4 | 2.5 | 6.4 × 10-7 |
|  | Area [cm2]10-13 | Opening time [s] | Closing time [s] |
| Aortic valve | 4 | 0.00 | 0.43 |
| Pulmonary valve | 5 | 0.00 | 0.47 |
| Mitral valve | 4.2 | 0.53 | 1.00 |
| Tricuspid valve | 5 | 0.43 | 1.00 |
|  | Resistance [Pa s m-3]14,15 | Volume [L]14 | Compliance [m Pa-1] |
| Upper body | 0.31–5.2 × 108 | 0.89 | 6.7 × 10-8 |
| Abdominal | 0.15–2.3 × 108 | 2.02 | 3.6 × 10-8 |
| Lower body | 0.40–4.7 × 108 | 0.51 | 4.1 × 10-8 |
| Pulmonary | 0.50–5.0 106 | 0.55 | 6 × 10-7 |

Table S2: Extensive set of parameters of baseline lumped-parameter simulation.

|  | Relative amount of trapped air |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Custom hydraulic fluid | 0.005 |  |  |  |  |  |
|  | Laminar friction constant | Aggregate equivalent length [m] | Internal surface roughness height [µm] | Laminar flow upper Reynolds limit | Turbulent flow upper Reynolds limit | Specific heat ratio |
| Hydraulic pipeline | 64 | 0 | 15 | 2000 | 4000 | 1.4 |
|  | Viscoelastic time constant [s] |  |  | Specific heat ratio |  |  |
| Constant volume hydraulic chamber | 0.01 |  |  | 1.4 |  |  |
| Variable-compliance compliance chamber | 0.01 |  |  | 1.4 |  |  |
|  | First approximating coefficient [Pa kg-1 m3] | Second approximating coefficient [Pa s kg-1] | Third approximating coefficient [Pa s2 kg-1 m-3] | Fourth approximating coefficient [Pa s2 kg-1 m-3] | Correction factor | Pump design delivery [L min-1] |
| Centrifugal pump | 326.8 | 3.104 × 104 | 1.097 × 107 | 2.136 × 105 | 0.8 | 130 |
|  | Reference angular velocity [rpm] | Reference density [kg m-3] | Leak resistance [Pa m3 s-1] | Drive shaft torque [N m] | Torque-pressure coefficient [N m Pa-1] | Angular speed threshold for flow reversal [rad s-1] |
| Centrifugal pump | 1770 | 1060 | 7 × 106 | 0.1 | 1 × 10-6 | 1 × 10-9 |
|  | Maximum passage area [m2] | Crack pressure [Pa] | Maximum opening pressure [Pa] | Flow discharge coefficient | Leakage [m2] | Laminar flow pressure ratio |
| Check valve | 0.1 | 0.01 | 0.1 | 1 | 1 × 10-12 | 0.999 |
|  | Flow discharge coefficient |  | Minimum area |  | Laminar flow pressure ratio |  |
| Variable area hydraulic orifice | 1 |  | 1 × 10-15 |  | 0.999 |  |

Figure S1: Input signals for (A) centrifugal pump, (B) left ventricle, (C) right ventricle, (D) left and right atria for the lumped-parameter simulation.

Table S3: Fluid cavities values in the mechanical finite element analysis (FEA) model3.

| Boundary Name | Pressure (MPa) |
| --- | --- |
| CAV-ARTERIAL-COMP | 1.06 × 10-2 |
| CAV-VENOUS-COMP | 2.66 × 10-4 |
| CAV-PULMONARY-COMP | 1.06 × 10-3 |
| CAV-LV | 5.33 × 10-4 |
| CAV-RV | 2.66 × 10-4 |
| CAV-AORTA | 1.06 × 10-2 |
| CAV-PULMONARY_TRUNK | 1.06 × 10-3 |
| CAV-RA | 2.66 × 10-4 |
| CAV-LA | 5.33 × 10-4 |
| CAV-SVC | 2.66 × 10-4 |

Table S4: Boundary conditions of fluid exchange links for the finite element analysis (FEA) model.

| Boundary Name | Viscous Resistance Coefficient Value (1/mm2) |
| --- | --- |
| Link-Aortic-Valve | 112896 |
| Link-Body-Resistance | 59965.5 |
| Link-Mitral-Valve | 2359.04 |
| Link-Pulmonary-Resistance | 4750 |
| Link-Pulmonary-Valve | 423.47 |
| Link-Tricuspid-Valve | 429.5 |
| Link-Venous-Resistance | 429.5 |

FEA simulations were performed by the FEA simulation software. Table S5 shows the required model files to run the simulation. More details can be found in the User Guide3.

Table S5: The required simulation files for the finite element analysis (FEA) model3.

| File Name | Description |
| --- | --- |
| LH-Human-Model-Beta-V2_1.cae | FEA simulation software GUI File |
| nodes_AORTIC_ARCH.inp | Initial nodal coordinates of aortic arch |
| nodes_VENTRICLES.inp | Initial nodal coordinates of right and left ventricles |
| nodes_TRICUSPIDVALVE_MESH.inp | Initial nodal coordinates of tricuspid valve |
| nodes_PULMONARY_VALVE_CUT.inp | Initial nodal coordinates of pulmonary valve |
| nodes_R_ATRIUM.inp | Initial nodal coordinates of the right atrium |
| nodes_PULMONARY_TRUNK.inp | Initial nodal coordinates of the pulmonary cavity |
| nodes_MITRALVALVE_MESH.inp | Initial nodal coordinates of the mitral valve |
| nodes_L_ATRIUM.inp | Initial nodal coordinates of the left atrium |
| nodes_ASSEMBLY.inp | Initial nodal coordinates of the assembly connections |
| nodes_AORTIC_VALVE_CUT.inp | Initial nodal coordinates of the aortic valve |
| nodes_CHORDAE_TENDINEAE_MITRAL.inp | Initial nodal coordinates of the MV chordae tendineae |
| nodes_CHORDAE_TENDINEAE_TRICUSPID.inp | Initial nodal coordinates of the TV chordae tendineae |
| DF-Aortic_Arch.inp | Discrete field file of the aortic arch for the fiber orientations |
| DF-L_Atrium.inp | Discrete field file of the left atrium for the fiber orientations |
| DF-MITRALVALVE_MESH.inp | Discrete field file of the mitral valve for the fiber orientations |
| DF-Pulmonary_Trunk.inp | Discrete field file of the pulmonary cavity for the fiber orientations |
| DF-R_Atrium.inp | Discrete field file of the right atrium for the fiber orientations |
| DF-TRICUSPIDVALVE_MESH.inp | Discrete field file of the tricuspid valve for the fiber orientations |
| DF-Ventricles.inp | Discrete field file of the ventricles for the fiber orientations |
| mech-mat-ARTERY.inp | Material definition input for the artery in mechanical simulation |
| mech-mat-LA_ACTIVE.inp | Material definition input for the active left atrium components in mechanical simulation |
| mech-mat-LV_ACTIVE.inp | Material definition input for the active left ventricle components in mechanical simulation |
| mech-mat-PASSIVE.inp | Material definition input for the passive cardiac tissue components in mechanical simulation |
| mech-mat-RA_ACTIVE.inp | Material definition input for the active right atrium components in mechanical simulation |
| mech-mat-RV_ACTIVE.inp | Material definition input for the active right ventricle components in mechanical simulation |
| mech-mat-VALVE.inp | Material definition input for the valves in mechanical simulation |
| elec-mat-L_Atrium.inp | Material definition input for the left atrium in electrical simulation |
| elec-mat-PurkinjeL.inp | Material definition input for the Prukinje fiber in electrical simulation |
| elec-mat-PurkinjeR.inp | Material definition input for the Prukinje fiber in electrical simulation |
| elec-mat-R_Atrium.inp | Material definition input for the right atrium in electrical simulation |
| elec-mat-Resistors.inp | Material definition input for the conduction elements in electrical simulation |
| elec-mat-Ventricles.inp | Material definition input for the ventricles in electrical simulation |
| standardU.dll | FEA sImulation software library and subroutine files |
| heart-elec-std.obj |  |
| explicitU-D.dll |  |
| heart-mech-xplD.obj |  |
| libstandardU.so |  |
| heart-elec-std.o |  |
| libexplicitU-D.so |  |
| heart-mech-xplD.o |  |
| abaqus_v6.env |  |
| calcNodeCoords.py | FEA simulation software toolbox files |
| inversePreliminary.py |  |
| straight_mv_chordae.py |  |
| genPartFile.py |  |
| genAssemblyFile.py |  |

The fluid flow between heart chambers is modeled using fluid exchange definitions. The fluid exchange module (FEA Simulation software) was employed to create the lumped-parameter model of blood flow depicted in Figure 2B. This model utilizes the conventional RC circuit analogy, which, in its simplest form, can be written as:

| i=VR | (S1) |
| --- | --- |

where 𝑖 is current, V is voltage and R is resistance. In the hydraulic domain, Equation S1 can be rewritten as:

| Q=∆PR | (S2) |
| --- | --- |

where Q represents the volume of blood per unit time, ∆𝑃 is pressure gradient across a conduit and R is resistance to flow. In the FEA model, fluid exchange is defined as fluid flow through porous media as per Darcy’s Law (Equation S3). Flow is assumed to be laminar, and it is characterized by viscous loss coefficients in the linear flow direction16:

| Q=-kAμ∆PL | (S3) |
| --- | --- |

where Q is the volumetric flow rate, k is the permeability of porous media, µ is the dynamic viscosity, A is the cross-sectional area, and L is the length over which pressure drop occurs. This can be simplified and rewritten as in Equation S4:

| ∆P=ραAQ | (S4) |
| --- | --- |

where 1/α is equal to the viscous resistance coefficient Cv16, yielding:

| ∆P=ρCvAQ | (S5) |
| --- | --- |

In the FEA model, the viscous resistance coefficient Cv is implemented as an input parameter to calculate the flow and pressure drop between fluid cavities.

Table S6: Parameters for the aortic-stenosis lumped-parameter simulation.

|  | Area [cm2] | Opening time [s] | Closing time [s] |
| --- | --- | --- | --- |
| Aortic valve | 1.2 | 0.00 | 0.43 |

Table S7: Fluid exchange link definitions in the finite element analysis (FEA) model3.

| Fluid Exchange Name | Effective Exchange Area (mm2) |  |
| --- | --- | --- |
|  | Normal Heart | Aortic Stenosis |
| LINK-ARTERIAL-VENOUS | 441.15 | 441.15 |
| LINK-LA-LV | 1039.8 | 1039.8 |
| LINK-LV-ARTERIAL | 440 | 132 |
| LINK-PULMONARY-LA | 434.9 | 434.9 |
| LINK-RA-RV | 1722.2 | 1722.2 |
| LINK-RV-PULMONARY | 434.9 | 434.9 |
| LINK-VENOUS-RA | 441.15 | 441.15 |

Table S8: Parameters for the HFpEF lumped-parameter simulation.

|  | Diameter [cm] | Length [cm] | End-Diastolic Compliance [m Pa-1] |
| --- | --- | --- | --- |
| Left ventricle | 4.4 | 7 | 2.2 × 10-12 |
|  | Area [cm2] | Opening time [s] | Closing time [s] |
| Aortic valve | 1.2 | 0.00 | 0.43 |
|  | Leak resistance [Pa m3 s-1] |  |  |
| Centrifugal pump | 18 × 106 |  |  |

Figure S2: (A) Aortic and (B) mitral flow signals for the baseline, stenosis, and HFpEF profiles, obtained by FEA. Abbreviations: E = early relaxation phase; A = atrial contraction; FEA = finite element analysis; HFpEF = heart failure with preserved ejection fraction.

REFERENCES

1	van Wyk, S., Prahl Wittberg, L., Bulusu, K. V., Fuchs, L., Plesniak, M. W. Non-Newtonian perspectives on pulsatile blood-analog flows in a 180° curved artery model. Physics of Fluids. 27 (7), 71901–71901 (2015).

2	Carew, T. E., Vaishnav, R. N., Patel, D. J. Compressibility of the arterial wall. Circulation research. 23 (1), 61–68 (1968).

3	Abaqus Dassault, S. SIMULIA living heart human model user documentation. (2017).

4	Cuomo, F. et al. Effects of age-associated regional changes in aortic stiffness on human hemodynamics revealed by computational modeling. PLoS ONE. 12 (3), e0173177 (2017).

5	Sonavane, S. K. et al. Comprehensive imaging review of the superior vena cava. RadioGraphics. 35, 1875–1892 (2015).

6	Patil, S. et al. Assessment of inferior vena cava diameter by echocardiography in normal Indian population: A prospective observational study. Indian Heart Journal. 68 (Suppl 3), S26–S30 (2016).

7	Dotter, C. T., Roberts, D. J., Steinberg, I. Aortic length: angiocardiographic measurements. Circulation. 2 (6), 915–920 (1950).

8	Ghosh, S. K., Paul, S. Anatomy of the retrohepatic segment of the inferior vena cava and the ostia venae hepaticae with its clinical significance. Surgical and Radiologic Anatomy. 34, 347–355 (2012).

9	Mozes, G., Gloviczki, P. The vein book. Bergan, J. J.(ed), Academic Press, 15–25 (2007).

10	González-Mansilla, A. et al. Valve area and the risk of overestimating aortic stenosis. Heart. 105 (12), 911–919 (2019).

11	Capps, S. B., Elkins, R. C., Fronk, D. M. Body surface area as a predictor of aortic and pulmonary valve diameter. The Journal of thoracic and cardiovascular surgery. 119 (5), 975–982 (2000).

12	Soeki, T. et al. Mitral inflow and mitral annular motion velocities in patients with mitral annular calcification: evaluation by pulsed Doppler echocardiography and pulsed Doppler tissue imaging. European Journal of Echocardiography. 3 (2), 128–134 (2002).

13	Bhatia, A. Transesophageal echocardiography evaluation of tricuspid and pulmonic valves. Annals of Cardiac Anaesthesia. 19 (Supplement), S21–S25 (2016).

14	Heldt, T., Shim, E. B., Kamm, R. D., Mark, R. G. Computational modeling of cardiovascular response to orthostatic stress. Journal of Applied Physiology. 92 (3), 1239–1254 (2002).

15	Lilly, L. S. Pathophysiology of heart disease: a collaborative project of medical students and faculty. Lippincott Williams & Wilkins (2012).

16	Zeng, Z., Grigg, R. A criterion for non-Darcy flow in porous media. Transport in porous media. 63 (1), 57–69 (2006).