Table S1: Geometric and mechanical parameters of baseline lumped-parameter simulation.

|  | Density [kg m-3]1 | Kinematic viscosity [mm2 s-1]1 | Bulk modulus [GPa]2 |
| --- | --- | --- | --- |
| Blood | 1060 | 3.55 | 2.2 |
|  | Diameter [cm]3-6 | Length [cm]3-5,7-9 | Compliance [m Pa-1] |
| Left ventricle | 4.4 | 7 | 0.002–1 × 10-6 |
| Right ventricle | 4.8 | 5 | 1.8–5.5 × 10-5 |
| Left atrium | 5 | 7.5 | 0.8–1.7 × 10-5 |
| Right atrium | 5.6 | 5 | 1.9–2.3 × 10-5 |
| Ascending aorta | 2.65 | 8.8 | 4.0 × 10-7 |
| Descending aorta | 2.15 | 6.9 | 7.0 × 10-7 |
| Thoracic aorta | 1.92 | 33.2 | 1.7 × 10-7 |
| Pulmonary artery | 3 | 5 | 2.2 × 10-7 |
| Superior vena cava (SVC) | 2.1 | 7.1 | 5.0 × 10-6 |
| Abdominal inferior vena cava (IVC) | 1.4 | 17 | 4.4 × 10-6 |
| Thoracic IVC | 1.4 | 2.5 | 6.4 × 10-7 |
|  | Area [cm2]10-13 | Opening time [s] | Closing time [s] |
| Aortic valve | 4 | 0.00 | 0.43 |
| Pulmonary valve | 5 | 0.00 | 0.47 |
| Mitral valve | 4.2 | 0.53 | 1.00 |
| Tricuspid valve | 5 | 0.43 | 1.00 |
|  | Resistance [Pa s m-3]14,15 | Volume [L]14 | Compliance [m Pa-1] |
| Upper body | 0.31–5.2 × 108 | 0.89 | 6.7 × 10-8 |
| Abdominal | 0.15–2.3 × 108 | 2.02 | 3.6 × 10-8 |
| Lower body | 0.40–4.7 × 108 | 0.51 | 4.1 × 10-8 |
| Pulmonary | 0.50–5.0 106 | 0.55 | 6 × 10-7 |