Table S2: Extensive set of parameters of baseline lumped-parameter simulation.

|  | Relative amount of trapped air |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Custom hydraulic fluid | 0.005 |  |  |  |  |  |
|  | Laminar friction constant | Aggregate equivalent length [m] | Internal surface roughness height [µm] | Laminar flow upper Reynolds limit | Turbulent flow upper Reynolds limit | Specific heat ratio |
| Hydraulic pipeline | 64 | 0 | 15 | 2000 | 4000 | 1.4 |
|  | Viscoelastic time constant [s] |  |  | Specific heat ratio |  |  |
| Constant volume hydraulic chamber | 0.01 |  |  | 1.4 |  |  |
| Variable-compliance compliance chamber | 0.01 |  |  | 1.4 |  |  |
|  | First approximating coefficient [Pa kg-1 m3] | Second approximating coefficient [Pa s kg-1] | Third approximating coefficient [Pa s2 kg-1 m-3] | Fourth approximating coefficient [Pa s2 kg-1 m-3] | Correction factor | Pump design delivery [L min-1] |
| Centrifugal pump | 326.8 | 3.104 × 104 | 1.097 × 107 | 2.136 × 105 | 0.8 | 130 |
|  | Reference angular velocity [rpm] | Reference density [kg m-3] | Leak resistance [Pa m3 s-1] | Drive shaft torque [N m] | Torque-pressure coefficient [N m Pa-1] | Angular speed threshold for flow reversal [rad s-1] |
| Centrifugal pump | 1770 | 1060 | 7 × 106 | 0.1 | 1 × 10-6 | 1 × 10-9 |
|  | Maximum passage area [m2] | Crack pressure [Pa] | Maximum opening pressure [Pa] | Flow discharge coefficient | Leakage [m2] | Laminar flow pressure ratio |
| Check valve | 0.1 | 0.01 | 0.1 | 1 | 1 × 10-12 | 0.999 |
|  | Flow discharge coefficient |  | Minimum area |  | Laminar flow pressure ratio |  |
| Variable area hydraulic orifice | 1 |  | 1 × 10-15 |  | 0.999 |  |