62181 ScreenShot_Summary

5.3. From the software’s File menu, open the image file and convert it to greyscale, adjust

brightness and contrast, and remove outlier noise if necessary [1].

5.3.1. SCREEN: Image opened, converted to greyscale, then brightness, contrast, and

noise corrected. 00:00-00:19

5.4. Draw a rectangle around the first lane with the Rectangle Selection tool and select it

through the Analyze, Gels, and Select First Lane command or by using the keyboard

shortcut [1].

5.4.1. SCREEN: Rectangle drawn and selected. 00:19-00:27

5.5. Use the mouse to click and hold in the middle of the rectangle on the first lane and drag it over to the next lane. Go to Analyze, Gels, and Select Next Lane. Repeat this step for all remaining lanes [1].

5.5.1. SCREEN: Rectangle moved and lane selected. 00:27-00:44

5.6. After all the lanes are highlighted and numbered, go to Analyze, Gels, and Plot Lanes

to draw a profile plot of each lane [1].

5.6.1. SCREEN: Lane profile plots created.00:44-00:50

5.7. Use the Straight-line selection tool to draw a line across the base of each peak

corresponding to each band, leaving out the background noise [1]. After all the peaks

from every lane have been closed off, select the Wand tool and click inside each peak.

Measurements should pop up in the Results window for each highlighted peak [2].

5.7.1. SCREEN: Line drawn across a peak. 00:50-1:26

5.7.2. SCREEN: Wand tool used to create measurements. 1:26-1:53

5.8. Sum the intensity from all 3 bands for each sample and calculate the percentage for

each isoform relative to the total [1]. Plot the percentages of each isoform or the

differences in the percentage relative to untreated samples [2]. 1:53-2:18

5.8.1. SCREEN: Intensity summed and percentage calculated.

5.8.2. SCREEN: Percentages plotted.