# Submission ID #:  62257

# Scriptwriter Name: Madhulika Pathak

# Supervisor Name: Anastasia Gomez

# Project Page Link: https://www.jove.com/account/file-uploader?src=18988433

# Title:   Single Myofiber Culture Assay for the Assessment of Adult Muscle Stem Cell Functionality Ex Vivo

# Authors and Affiliations:

Sören S. Hüttner1, Christian Hayn1, Hellen E. Ahrens1, Manuel Schmidt1, Henriette Henze1, Julia von Maltzahn1

1Leibniz Institute on Aging, Fritz-Lipmann-Institute, Beutenbergstrasse 11, 07745 Jena, Germany

# Corresponding Authors:

# Julia von Maltzahn 	(julia.vonmaltzahn@leibniz-fli.de)

# Email Addresses for All Authors:

soeren.huettner@leibniz-fli.de

christian.hayn@leibniz-fli.de

hellen.ahrens@leibniz-fli.de

manuel.schmidt@leibniz-fli.de

henriette.henze@leibniz-fli.de

julia.vonmaltzahn@leibniz-fli.de

## Author Questionnaire

1. Microscopy: Does your protocol require the use of a dissecting or stereomicroscope for performing a complex dissection, microinjection technique, or something similar?  Yes

If Yes, can you record movies/images using your own microscope camera?

No

If your protocol involves microscopy but you are not able to record movies/images with your microscope camera, JoVE will need to use our scope kit (through a camera port or one of the oculars). Please list the make and model of your microscope.

Leica Stemi 200-CS with holder for a camera

2. Software: Does the part of your protocol being filmed include step-by-step descriptions of software usage?  No

3. Interview statements: Considering the COVID-19-imposed mask-wearing and social distancing recommendations, which interview statement filming option is the most appropriate for your group? Please select one.

☒ 	Interview Statements are read by JoVE’s voiceover talent.

4. Filming location: Will the filming need to take place in multiple locations?   No

Current Protocol Length

Number of Steps:  17

Number of Shots:  39

# Introduction

- Introductory Interview Statements

REQUIRED:

  - Muscle stem cells remain associated with their endogenous niche and can be manipulated easily, for instance through siRNA transfection.

    - 4.1.3.

  - This method makes it possible to analyze muscle stem cells in culture in a setting which resembles the in vivo situation more closely than standard 2D culture models by preserving the niche.

    - 5.3.1, 5.3.2.

Ethics Title Card

  - Procedures involving animal subjects have been performed in accordance with the guidelines of the Leibniz Institute on Aging - Fritz Lipmann Institute and the European Union (EU) directive 2010/63/EU.

# Protocol

- Preparing pipettes for trituration of the collagenase-digested muscles

  - Begin by cutting sterile Pasteur pipettes with a diamond pen [1].

    - Talent cutting the Pasteur pipette using diamond pen

  - For each mouse, use one large bore pipette with an opening of about 0.3 centimeters and a length of about 10 to 12 centimeters and a second glass pipette with a small opening of about 0.1 centimeter and a length of approximately 22 centimeters [1].

    - Short and large pipette

  - Smoothen the edges of both pipettes by holding the pipette tips in the flame of a Bunsen burner for 5 to 10 seconds with gentle movement [1].

    - Talent flaming the pipette using burner

  - Immediately before use, coat both pipettes with sterile Horse serum by filling the whole pipette with 2 milliliters of Horse serum for 5 minutes [1], then ejecting the Horse serum [2] and allowing the pipettes to dry for 5 minutes at room temperature [3].

    - Talent filling the pipette with serum and then holding it

    - Talent ejecting the serum

    - Talent placing the pipettes down to dry

- EDL muscle isolation and collagenase digestion

  - Spray all equipment [1] and the hindlimbs of the mouse with 70% ethanol [2]. Use hardened fine curved scissors and fine forceps to remove the skin and expose the underlying muscles [3-TXT].

    - Talent spraying 70% ethanol on all the equipment

    - Talent spraying ethanol on the hindlimbs of mouse

    - Talent dissecting the mouse to remove the skin TEXT: Scissors: 24 mm Forceps: Dumont 7

  - Remove the surrounding fascia with the fine curved forceps without damaging the underlying muscles [1].

    - Talent removing the fascia with forceps

  - To remove the tibialis anterior, or TA, grab the distal TA tendon with forceps and cut it with fine scissors [1-TXT]. While holding the TA at the tendon, pull it towards the knee and cut the muscle close to the knee to expose the EDL muscle [2].

    - Talent removing the TA TEXT: Vannas spring scissors: 5 mm cutting edge, 0.35 mm tip diameter

    - Talent cutting the muscle close to the knee and exposing the EDL muscle.

  - Lift the distal EDL tendon with curved forceps and cut with fine Vannas spring scissors [1]. Expose the proximal EDL tendon by carefully pulling the EDL towards the knee, then cut the proximal tendon with scissors [2]. Incubate the EDL muscles in the reaction tube at 37 degrees Celsius in a circulating water bath [3].

    - Talent cutting the distal EDL tendon Videographer: This step is important!

    - Talent pulling and cutting the proximal EDL tendon Videographer: This step is difficult and important!

    - Talent putting the muscle containing tube in water bath

  - Stop the digestion when muscles loosen up and single myofibers are visible [1]. Carefully transfer the muscles with the large bore pipette to the first well of the prewarmed 12-well plate containing myofiber isolation medium [2].

    - Talent displaying the tube with loosen muscles and single myofibers

    - Talent transferring the muscle with pipette to the 12-well plate

- Myofiber dissociation and culture

  - Working under a stereo binocular microscope equipped with a heating plate, flush the muscles with warm isolation medium using the large bore pipette [1]. Dissociate the muscles with the large bore pipette [2] until the desired number of myofibers are floating freely in the solution [3].

    - SCOPE: Talent flushing the muscles with the medium

    - SCOPE: Talent continuously dissociating the muscles Videographer: This step is important!

    - SCOPE:  Freely floating muscle fibers in the solution

  - To wash the debris, use the small-bore glass pipette to transfer non-contracted myofibers to the second well filled with isolation medium [1], then transfer 50 to 100 non-contracted myofibers to one well of a 24 well plate filled with myofiber culture medium [2].

    - SCOPE: Talent transferring myofibers to the second well.

    - SCOPE: Talent transferring myofibers to the well of 24 well plate

  - Incubate the myofibers at 37 degrees Celsius and 5 percent Carbon dioxide for 72 to 96 hours [1].

    - Talent incubating the plate

- siRNA transfection, fixation and IF staining

  - Transfect myofiber associated muscle stem cells 4 hours after myofiber isolation.  Combine 25 microliters of Opti-MEM with the respective volume of siRNA with 25 microliters Opti-MEM containing 1.5 microliters of transfection reagent [1-TXT]. Incubate the reaction mix for 5 minutes [2] and add it to the cells in the 24-well plate [3].

    - Talent adding Opti-MEM with siRNA to Opti-MEM with transfection reagent. TEXT: Final concentration: 5 pmol siRNA

    - Talent incubating the reaction mix

    - Talent adding reaction mix to a well with myofibers.

  - To perform immunofluorescence staining, carefully discard the myofiber culture medium while leaving some solution in the well [1]. Add 500 microliters of 2 percent paraformaldehyde to fix the myofibers with their adjacent muscle stem cells [2].

    - Talent discarding the myofiber culture medium.

    - Talent adding PFA to the plate

  - Incubate the plate for 5 minutes at room temperature followed by incubation with antibodies for the immunofluorescence staining [1]. Remove the supernatant carefully [2-TXT] and wash the myofibers three times with PBS [3], then cover the plate with tinfoil [4-TXT].

    - Talent incubating the plate

    - Talent removing the supernatant

    - Talent washing the myofibers with PBS TEXT: PBS- pH 7.4, 500 µL for 5 min at RT each

    - Talent covering 24 well plate with tinfoil. TEXT: Perform immunofluorescence under light reducing conditions

  - Use a hydrophobic pen to draw a circle on a microscopic glass slide [1], then transfer the myofibers in the smallest volume possible to the slide [2] and disperse them [3].

    - Talent drawing the circle on the slide

    - Talent transferring the myofibers on glass slide

    - Talent dispersing the myofibers on the slide

  - Remove the residual liquid with the small-bore Pasteur pipette or a 200 microliter pipette [1]. Use two drops of aqueous mounting medium and cover the myofibers with a coverslip [2], then allow the slides to dry [3] and store them at 4 degrees Celsius in the dark followed by microscopic analysis [4].

    - Talent removing the residual liquid using pipette

    - Talent mounting myofibrils and putting the coverslip on slides

    - Talent letting the slides to dry

    - Talent sitting at the microscope analyzing the slides

# Results

- Results: Culture and transfection of murine EDL muscle-derived single myofibres

  - This protocol demonstrates derivation and culture of single myofibers from murine EDL muscles. Immunofluorescence staining for Pax7 was used to identify nuclei in muscle stem cells [1].

    - LAB MEDIA: Figure 3 Video editor: please highlight figure 3A

  - The magnified area exposed the myofiber with its adjacent muscle stem cells and demonstrated the Pax7 immunofluorescence signal in the nucleus of a muscle stem cell [2].

    - LAB MEDIA: Figure 3 Video editor: please highlight figure 3B

  - Muscle stem cells myogenic progression can be analyzed by marker expression. Presence of Pax7 and absence of MyoD expression is associated with quiescent muscle stem cells of freshly isolated myofibers [1]. MyoD expression can be observed in proliferating muscle stem cells [2].

    - LAB MEDIA: Figure 4 Video editor: please highlight 0 h Column of the figure

    - LAB MEDIA: Figure 4 Video editor: please highlight 42 h Column of the figure

  - After 72 hours, muscle stem cells form clusters of progenies with different myogenic states, which is paralleled by expression of different myogenic markers [1]. Pax7 only cells are self-renewing stem cells [2]. Pax7 and MyoD double positive cells are proliferating [3], whereas MyoD only cells are differentiating cells [4].

    - LAB MEDIA: Figure 4 Video editor: please highlight figure 72 h Column of the figure

    - LAB MEDIA: Figure 4 Video editor: please highlight Pax 7 row and 72 h Column of the figure and highlight red arrow pointed cell

    - LAB MEDIA: Figure 4 Video editor: please highlight DAPI+MERGE row and 72 h Column of the figure and highlight red + green arrow pointed cell

    - LAB MEDIA: Figure 4 Video editor: please highlight MyoD row and 72 h Column of the figure and highlight green arrow pointed cell

  - The siRNA transfection of muscle stem cells showed accumulation of cytoplasmic siRNA in a granule-like fashion, indicating efficient uptake [1]. Quantification of transfected Pax7 positive cells per myofiber revealed that the number of transfected cells increased up to 74% after 30 hours [2] with no adverse effect on muscle stem cell numbers [3].

    - LAB MEDIA: Figure 5 Video editor: please highlight figure 5A

    - LAB MEDIA: Figure 5 Video editor: please highlight figure 5B

    - LAB MEDIA: Figure 5 Video editor: please highlight figure 5C

# Conclusion

- Conclusion Interview Statements

  - When attempting this protocol, it is of utmost importance to carefully dissect the EDL without causing any damage.

    - 3.3.2, 3.4.2 and 3.5.1

  - In addition to siRNA transfection, the analysis of transgenic animals or the incubation with recombinant proteins is possible for further functional analyses of muscle stem cells on their adjacent myofibers.

    - 5.5.4.