Supplemental Information

- Items for making an inoculation box

- Stainless steel grid mesh screen, 48-micron, Size: 24" X 48" from Beyondsupply, Amazon.

- Medium density fiberboard (1/4" x 2' x 4'); Model# 1508104 from Home Depot.

- 1-5/8" cabinet door magnetic catch white, Internet #202101901, Model #P110-W from Home Depot.

- 3" Corner clamp, SKU 63653, 1852, 60589 from Harbor Freight Tools. Need 8 each.

- 2" steel zinc-plated corner brace (4-Pack); Internet #203170052, Model #13611, Store SKU #339695 from Home Depot. Buy one pack of 4. Four are used to support the left side panel.

- #8-32 x ½" machine screws, flat washers and nuts from Home Depot.

- 4-7/8" zinc-plated light duty door pull handles. Internet #202034068, Model #15184, Store SKU #241768 from Home Depot.

- Kelleher 3/4" x 3/4" x 36" wood square dowel from Home Depot. Buy 4 pieces.

- Items for making an inoculation chamber

  - 0.250 thick clear extruded acrylic film-masked sheet (SACR.250CEF), Size: 18" X 30" @ +/- 0.015 [Qty: 2] from www.professionalplastics.com. For front and back sides of enclosure.

  - 0.250 thick clear extruded acrylic film-masked sheet (SACR.250CEF), Size: 18" X 20" @ +/- 0.015 [Qty: 1] from www.professionalplastics.com. For left side of enclosure.

  - 0.250 thick clear extruded acrylic film-masked sheet (SACR.250CEF), Size: 17 ½" X 20" @ +/- 0.015 [Qty: 1] from www.professionalplastics.com. For right side of enclosure which is the door that pulls down. Needs ½" clearance at bottom.

  - 0.250 thick clear extruded acrylic film-masked sheet (SACR.250CEF), Size: 20" X 29 ½ " @ +/- 0.015 [Qty: 1] from www.professionalplastics.com. For top side of enclosure.

  - 3/4" steel zinc plated corner brace (4-Pack); Internet #202950157, Model #13542, Store SKU #527580 from Home Depot. Buy one pack of 4. Two are used as bottom hinges for the right-side access door. Use a nylon lock nut and do not tighten all the way and leave some gap.

  - #8-32 x ½" machine screws, flat washers and nuts from Home Depot, for all corner braces and door hinge. The hinge will need a nylon insert nut.

  - #6-32 zinc plated nylon lock nut (4-Pack) from Home Depot. Need two for the right door hinge.

  - 1-5/8" cabinet door magnetic catch white (also see items g and h); Internet #202101901; Model #P110-W from Home Depot.

  - #6-32 x ¾" machine screws, flat washers and nuts from Home Depot, to secure magnet door latch.

  - #6-32 x 3/8” Phillips flat head zinc plated machine screw (8-Pack) and flat washers from Home Depot.

- Items for inoculation

  - Fine fan-blender brushes from Michaels Store. Buy multiple.