Supplementary File 1: Millisect Pseudocode

function Millisect_save_pseudo(slidepath)

%MILLISECT_SAVE_PSEUDO attempts to crop whitespace around tissue, creating

%one single PNG export with an mask of all ROIs present on a

%slide.  This function calls several custom APIs in order to access image

%data and ROI coordinates.  An end user will need to customize this access

%for their particular system.

%% create mask of ROIS and keep them

% Queries external viewer where ROIs are created.  IF there are no ROIs

% present on a slide, skip making an overlay image

AllRegions = VectorAPI(slidepath, 'GETALL'); %get all ROI ID's to be stamped

% if no ROIs, then skip the rest

if size(AllRegions, 2) > 0

% get pixel data at a low mag power, high mag is overkill for image alignment

imgin = CaptureAPI(slidepath,magpower);

%% crop whitespace to focus on just regions with tissue

% this step is optional, but can help the downstream alignment of the image

% on the Millisect

%create binary mask using global intensity threshold to roughly id tissue

tmask = ~imbinarize(rgb2gray(imgin), intensity_thresh) ;

% use alternating morphological operations of increasing size to remove

% small isolated noise and non-tissue

tmask = imclose(tmask, d3);

tmask = imopen(tmask, d3);

tmask = imclose(tmask, d5);

tmask = ~(bwareaopen(~tmask, 1000)); % closes small holes in tissue

tmask = imopen(imclose(tmask, d14), d9);

tmask = bwareaopen(tmask, 3000); % removes small pieces of tissue

tmask = imopen(tmask, d14);

tmask = imclose(tmask, d30);

tmask = bwareaopen(tmask, 10000); % removes small pieces of tissue

tmask = ~bwareaopen(~tmask, 10000); % closes large holes

%find convex hull to make one object, dilate to add a little around edges

ss_mask = bwconvhull(tmask);

ss_mask = imdilate(ss_mask, d14);

% get x-y coordinates of bounding box to crop tissue

hstats = regionprops(ss_mask, 'BoundingBox') ;

%% get ROIs

% query an external viewing platform to return an array of vectors

% containing x-y coordinates for ROIs made on a slide, and return a binary

% mask of the union of them all at the same size as the raw image

add_rois = gSlide_API_ROIs_to_mask(slidepath ,lowmagpower );

%crop raw image and binary mask

imgin = imcrop(imgin , hstats.BoundingBox) ;

add_rois = imcrop(add_rois , hstats.BoundingBox);

%% process low-res image to make it a manageable size

dimy = size(imgin, 1);

dimx = size(imgin, 2);

%if longest dimension is greater than 1000 pixels, resize image so longest side is

%less than 1000

if ( dimy > 1000 || dimx > 1000 )

%find longest side

if dimy > dimx

resize_factor = 1 / (dimy / 1000) ;

else

resize_factor = 1 / (dimx / 1000) ;

end

imgin = imresize(imgin, resize_factor);

add_rois = imresize(add_rois, [size(imgin,1) size(imgin, 2)], 'nearest');

end

%% create overlays and save

% create filename with .jpg extension

[~, savename] = fileparts(slidepath);

savename = fullfile(savename, '.jpg');

%create image overlay of input image with solid green wherever add_rois

%binary mask is present

im2 = imoverlay(imgin, add_rois, [1 1 0 ]) ;

imwrite(im2, savename, 'jpg');

end

end