# Submission ID #:  62470

# Scriptwriter Name: Mithila Boche

# Supervisor Name: Anastasia Gomez

# Project Page Link: https://www.jove.com/account/file-uploader?src=19053398

# Title:   Practical Aspects of Sample Preparation and Setup of 1H R1ρ Relaxation Dispersion Experiments of RNA

# Authors and Affiliations:

Hannes Feyrer, Judith Schlagnitweit, Katja Petzold

Department of Medical Biochemistry and Biophysics, Karolinska Institutet, 171 77 Stockholm, Sweden

# Corresponding Authors:

Katja Petzold			katja.petzold@ki.se

# Email Addresses for All Authors:

hannes.feyrer@ki.se

# judith.schlagnitweit@ki.se

# katja.petzold@ki.se

## Author Questionnaire

1. Microscopy: Does your protocol require the use of a dissecting or stereomicroscope for performing a complex dissection, microinjection technique, or something similar?  NO

2. Software: Does the part of your protocol being filmed include step-by-step descriptions of software usage?  YES

If Yes, we will need you to record using screen recording software to capture the steps. If you use a Mac, QuickTime X also has the ability to record the steps. Please create screen capture videos for all shots labeled SCREEN and upload them to the project page: https://www.jove.com/account/file-uploader?src=19053398.

3. Interview statements: Considering the COVID-19-imposed mask-wearing and social distancing recommendations, which interview statement filming option is the most appropriate for your group? Please select one.

☒ 	Interviewees wear masks until videographer steps away (≥6 ft/2 m) and begins filming, then the interviewee removes the mask for line delivery only. When take is captured, the interviewee puts the mask back on. Statements can be filmed outside if weather permits.

4. Filming location: Will the filming need to take place in multiple locations?   Yes (for mentioned b-roll)

If Yes, how far apart are the locations? 500 meters

Current Protocol Length

Number of Steps:  12

Number of Shots:  39

# Introduction

- Introductory Interview Statements

REQUIRED:

  - Hannes Feyrer: This protocol describes RNA sample preparation and NMR setup for proton R1rho relaxation dispersion measurements to examine conformational exchange in RNA molecules.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-Roll: 2.8.2 for “NMR set up”

  - Hannes Feyrer: Since protons are probed, the method does not need isotopic labelling and can access structural features like shifted base-pairing directly.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

OPTIONAL:

  - Hannes Feyrer: The pipeline is quite modular, in the sense that R1rho relaxation dispersion can be measured even if the sample is produced with a different method.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-Roll: 2.7.4 for “sample is produced”

# Protocol

- NMR Sample Preparation

  - Begin by preparing the plasmid sample and setting up an in vitro transcription reaction as described in the text manuscript [1]. Incubate the reaction at 37 Celsius for 1 hour [2], then dilute 1 microliter of the sample 10-fold in loading solution [3] and run 1 microliter on a denaturing PAGE gel [4].

    - WIDE: Establishing shot of talent adding reaction components. TEXT: Refer to text for details

    - Talent removing reaction tubes from an incubator.

    - Talent diluting the sample.

    - Talent runs the sample on the gel and proceeds for imaging.

  - If transcription is successful, scale the reaction to the desired volume and run it overnight, then test it for completion with a denaturing PAGE gel [1].

    - Talent imaging the resulting gel.

  - In case of incomplete cleavage, heat the reaction solution in a conventional microwave at 450 watts for 15 seconds to reanneal the RNA and the cleavage guide [1]. Then, cool the solution slowly to 37 degrees Celsius for 40 minutes [2] and note the formation of a new precipitate [3].

    - Talent placing reaction solution in microwave and setting time. Videographer: This step is important!

    - Talent taking out solution from microwave and placing the solution in an incubator.

    - Shot of precipitate being formed.

  - Add more inorganic pyrophosphatase and RNase H [1], incubate the reaction for 1 to 3 hours at 37 degrees Celsius [2], then confirm completion of the cleavage reaction with denaturing PAGE [3].

    - Talent adding IPPAse and RNase to reaction mixture.

    - Talent putting the reaction in the incubator.

    - Talent adding sample to PAGE.

  - When the RNase H cleavage reaction is completed [1], quench the reaction by adding EDTA to a 50 millimolar final concentration [2] and vortex thoroughly [3]. Filter the solution through a 0.2-micron syringe filter [4] and concentrate the solution to a volume injectable into an HPLC system for purification [5].

    - Talent removing sample.

    - Talent adding EDTA.

    - Talent vortexing the solution.

    - Talent injecting the sample into HPLC system.

  - Before adding the concentrated and purified sample to the NMR tube, clean the tube by flushing with abundant water [1], then RNase decontamination reagent, water, and 95% ethanol [2]. After the final rinse with water, leave it to dry [2].

    - WIDE: Talent cleaning NMR tube with water.

    - Talent cleaning NMR tube with RNase decontamination reagent/ water/ 95% ethanol.

    - Shot of tube drying.

  - Rinse the plunger with water [1] and use a lint-free wipe to clean with RNase decontamination reagent and 95% ethanol [2]. After drying the tube, add 10% D2O (D-2-O) to the NMR sample [3]. Use a large pipette tip to transfer the RNA sample into the NMR tube, flowing it along the side of the tube wall [4].

    - Talent cleaning the plunger.

    - Talent wiping the plunger.

    - Talent adding water to the sample.

    - Talent filling the sample into the tube.

  - Insert the plunger into the tube by pushing it down with a fast-twisting motion to remove air bubbles, then pull the plunger up slowly without creating new air bubbles [1] and fix it with paraffin wax film [4].

    - Talent inserting, pushing the plunger and pulling it up. Videographer: This step is important!

    - Talent fixing the plunger with wax and places the sample in the spectrometer.

- 1H R1𝜌 Relaxation Dispersion

  - Create a new data set based on an aromatic proton-carbon-HSQC data set used on fully labeled RNA samples for RNA assignment [1]. Authors: Please create screen capture videos for all shots labeled SCREEN and upload them to the project page: https://www.jove.com/account/file-uploader?src=19053398.

# Videographer: Film the screen for all SCREEN shots as a backup

    - WIDE: Talent at the computer. Videographer: Obtain a few shots of talent clicking the mouse and typing on the keyboard to use as b-roll throughout the video.

  - Set the general parameters and RD-specific parameters [1-TXT], then set proton spin lock power to 1.2 kilohertz for testing [2]. Generate a test vd list with only one entry, 0 milliseconds, set TDF1 to 1, and update D30 to run a test spectrum [3].

    - SCREEN: To be provided by authors: Parameters being set. TEXT: Refer to Table 2 in Manuscript Videographer: This step is important!

    - SCREEN: To be provided by authors: 1H SL power being set. Videographer: This step is important!

    - SCREEN: To be provided by authors: Test vd list being generated and D30 updated, then test spectrum being run.

  - After setting up a test vd list [1], update D30 and TDF1 [2-TXT] and plot the intensity of the peak for different the spin lock lengths.  Identify the spin lock length at which the intensity of the original peak decreases to 1/3 [3].

    - SCREEN: To be provided by authors: setting up a test vd-list. TEXT: six entries: 0 m, 5 m, 10 m, 20 m, 30 m, 40 m

    - SCREEN: To be provided by authors: D30 and TD[F1] being updated and experiment being run. TEXT: Example, D30 = 42 ms and TDF1 = 6.

    - SCREEN: To be provided by authors: Intensity of the peak vs. SL length being plotted.

  - From the results of the test runs, create the final vd list to be used in the experiment [1]. Select the number of scans so that the weakest peak of the list has a signal-to-noise ratio of at least 10 [2].

    - SCREEN: To be provided by authors: Results being observed/ the final vd list being created.

    - SCREEN: To be provided by authors: The number of scans being selected.

# Results

- Results: Analysis of produced sample and Two Different Constructs Based on an RNA Hairpin

  - The results of several cleavage reactions of tandem transcripts are shown here [1]. A 20-nucleotide target RNA was generated in a successful reaction [2]. Unsuccessful reactions resulted in incomplete [3] and failed cleavage of the sample [4].

    - LAB MEDIA: Figure 3 A.

    - LAB MEDIA: Figure 3 A. Video Editor: Emphasize the spot for 20 nt, Lane 1.

    - LAB MEDIA: Figure 3 A. Video Editor: Emphasize the lane 2a.

    - LAB MEDIA: Figure 3 A. Video Editor: Emphasize the lane 4.

  - HPLC injection of the RNA sample [1] revealed a peak for pure target RNA at 38 minutes [2], while longer and shorter products were well-separated from the peak of interest [3].

    - LAB MEDIA: Figure 3 B.

    - LAB MEDIA: Figure 3 B. Video Editor: Emphasize the target RNA.

    - LAB MEDIA: Figure 3 B. Video Editor: Emphasize abortive products, uncleaved repeats.

  - Analysis of the RNA hairpin showed that the number of observed imino protons [1] matched the number of imino protons expected from a secondary structure simulation [2].

    - LAB MEDIA: Figure 4 A and C. Video Editor: Emphasize red and blue lines.

    - LAB MEDIA: Figure 4 A and C. Video Editor: Emphasize the grey panels and lines between letters in panel C.

  - The proton-carbon-HSQC spectrum of the aromatic resonances of the RNA showed 4 signals after folding [1]  and only 3 signals before folding [2].

    - LAB MEDIA: Figure 4 B and D. Video Editor: Emphasize red spots and blue A letters in 4D.

    - LAB MEDIA: Figure 4 B and D. Video Editor: Emphasize blue spots and blue A letters in 4D.

  - In the representative on-resonance curves obtained for two different H8 atoms [1] in two different synthetic RNA hairpins [2], the G6H8 experiences conformational exchange [3] while the A4H8 does not [4]. For G6H8 the colored spin lock powers were selected and off-resonance data was recorded [5].

    - LAB MEDIA: Figure 5 A. Video Editor: Emphasize highlighted G and A.

    - LAB MEDIA: Figure 5 B - G.

    - LAB MEDIA: Figure 5 A, C, G. Video Editor: Emphasize grey highlighted G in 5A.

    - LAB MEDIA: Figure 5 A, B, F. Video Editor: Emphasize grey highlighted A in 5A.

    - LAB MEDIA: Figure 5 C, D, G, H. Video Editor: Emphasize colored legends of spin lock strengths in 5C and 5G

  - In the C-G construct, where the exchange is slow, the R1ρ (r 1 rho’) values versus offset plot already displayed a slight asymmetry of the curve, indicating the sign of the chemical shift difference (‘delta omega’) [1-TXT]. This becomes even more apparent in the R2+REX (‘r 2 plus r e x’) plot where the R1 contribution is removed [2].

    - LAB MEDIA: Figure 5 D. TEXT: Chemical shift difference: 𝛥𝜔

    - LAB MEDIA: Figure 5 E.

  - On the other hand, the G-C construct experiences faster exchange and the R1ρ (r 1 rho’) values versus offset plot presented a broader curve where the extraction of delta omega becomes less obvious even in the R2+REX (r 2 plus r e x) plot [1].

    - LAB MEDIA: Figure 5 H and I.

# Conclusion

- Conclusion Interview Statements

  - Katja Petzold: Once the RNA dynamics have been elucidated, the states can be characterized and trapped. These trapped states can for example be tested in biological assays.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera

  - Katja Petzold: This method allows us to observe functionally relevant base pairing shifts in an active microRNA mRNA complex, loop rearrangement in ribosomal RNA and base-pair stability around single-nucleotide bulges.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-Roll: 3.4.2 for “to observe”