# Submission ID #:  62533

# Scriptwriter Name: Madhulika Pathak

# Supervisor Name: Anastasia Gomez

# Project Page Link: https://www.jove.com/account/file-uploader?src=19072613

# Title:   An Adjustable High-definition Imaging System for Behavioral Studies of Drosophila Adults

# Authors and Affiliations:

Tong Li*1,2, Yujia Weng*1,2, Daxiang Yang1

1Department of Zoology and Animal Physiology, College of Biological Sciences, China Agricultural University, Beijing, China

2College of life sciences, Capital Normal University, Beijing, China

# Corresponding Authors:

Daxiang Yang	(dxyoung@cau.edu.cn)

# Email Addresses for All Authors:

dxyoung@cau.edu.cn

Litong_CNU@163.com

yjiaweng@163.com

## Author Questionnaire

1. Microscopy: Does your protocol require the use of a dissecting or stereomicroscope for performing a complex dissection, microinjection technique, or something similar?  No

2. Software: Does the part of your protocol being filmed include step-by-step descriptions of software usage?  No

3. Interview statements: Considering the COVID-19-imposed mask-wearing and social distancing recommendations, which interview statement filming option is the most appropriate for your group?

☒ 	Interviewees self-record interview statements. JoVE can provide support for this option.

4. Filming location: Will the filming need to take place in multiple locations?   No

Current Protocol Length

Number of Steps:  17

Number of Shots:  40

# Introduction

- Introductory Interview Statements

REQUIRED:

  - Daxiang Yang: Due to its awkward size, Drosophila is difficult to image. This method demonstrates imaging of the behavior or morphology of flies through an eyepiece of a stereo microscope.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested b-roll: 3.5.2

OPTIONAL:

  - Daxiang Yang: This protocol can be used both for scientific research and teaching.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

# Introduction of Demonstrator on Camera

  - Daxiang Yang: Demonstrating the procedure will be Ms. Li Tong and Ms. Weng Yujia, undergraduates doing research training under my guidance.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

# Protocol

- Construction of the Observing/Documenting System

  - To begin, make a fly behavior observation chamber, or FBOC using a translucent plastic bottle cap of about 17 by 22 millimeters or by cutting a section of 17 millimeters from the thick end of a 5-milliliter pipette tip [1].

    - Talent shows translucent plastic bottle cap, and cut a section of 17 millimeters from the thick end of a 5 milliliter pipette tip.

  - Pour 1% agar into the FBOC to adjust its depth [1]. If food needs to be placed in the FBOC, pour the agar to obtain a depth of 12 millimeters. If food does not need to be placed in the FBOC, pour the agar to a depth of 5 millimeters to track the whereabouts of fruit flies more easily [2].

    - Talent marking the height of the agar on the FBOC wall

    - Talent pouring agar into FBOC, Agar up to the depth of 12 millimeters and agar up to the depth of 5 millimeters

  - If using a pipette tip to make an FBOC, place the cut-off pipette tip section in a 35- or a 60-millimeter Petri dish [1]. Pour the 1% agar gel into the Petri dish to a thickness of about 5 millimeter [2] and wait for the agar to solidify and seal the bottom of the FBOC. Then, pour the agar gel into the FBOC to the desired thickness [3].

    - Talent placing the tip section in Petri dish

    - Talent pouring the agar in the Petri dish

    - Talent pouring the agar gel into FBOC when agar in Petri dish solidified.

  - Make an FBOC base by boring a 10-millimeter-deep hole in the center of a piece of ethylamine foam sheet with the same diameter as the FBOC [1-TXT]. Insert the FBOC into the hole [2].

    - Talent boring a hole in foam sheet TEXT: Ethylamine foam sheet: 60 mm x 60 mm x 15 mm

    - Talent inserting FBOC into the hole

  - Make fly food, if necessary, with yeast medium, artificial diet, or pure sucrose or glucose, depending on the purpose of the observation [1]. To visually determine whether flies are feeding [1a], add food dyes to the food to a final concentration of 12.5 milligrams per 100 milliliters [2]. NOTE: Extra screen shot 2.5.1a is added

    - Talent preparing fly food

2.5.1a Added shot: LAB MEDIA: fly behavior.mp4.02:36-02:34

    - Talent adding food dye

  - Pour the prepared food into a Petri dish to a height of 8 millimeters [1]. After solidification, place food on a rectangle drawn paper and cut the food accordingly [2-TXT] and place it on a piece of plastic, and cut into a quadrangular pyramid or quadrangular frustum pyramid [3] to allow recording of fly behavior from different angles as the flies land on the food [3a]. NOTE: Extra screen shot 2.6.3a is added

    - Talent pouring prepared food into Petri dish

    - Talent cutting the solidified food TEXT: Food size: 8 mm x 4 mm x 8 mm NOTE: In the video 2-6-2, talent drawing an 8 mm×4 mm rectangle on the paper, place the paper under the food, and cut out the food according to this size. Hence, VO changed and exceeded the four-line limit

    - Talent placing it on a piece of plastic and cut it into a quadrangular pyramid

2.6.3a Added shot: LAB MEDIA: fly behavior.mp4.02:18-02:30

  - Cut a piece of plast, use tweezers to place the plastic and food in the center of the FBOC [2].

    - Talent cuting a piece of plastic and using tweezers to place the plastic and food in the center of the FBOC TEXT: plastic size: 9 mm x 5 mm

  - Pour 1% agar gel into a clean empty bottle to a thickness of 1 to 2 centimeters and place it at room temperature for 1 to 2 hours [1-TXT]. Transfer flies to the bottle and place it at 25 degrees Celsius for 36 hours or more [2].

    - Talent pouring agar gel into empty bottle TEXT: 1 g agar/100 mL water, 600 μL of propionic acid

    - Talent transferring flies to the bottle

  - Transfer one or more flies into the FBOC using an aspirator [1]. If using an aspirator is difficult, chill and inactivate the flies in crushed ice [2], sort them on an ice pack [3], and transfer them to the FBOC as previously described [4].

    - Talent inactivating the flies on crushed ice

    - Talent sorting the flies on ice pack

    - Talent transferring the sorted flies to FBOC

  - After transferring the flies to the FBOC, cover it with 30 to 40 millimeters of UV or clear filter for the camera to form an FBOC complex [1]. Place the FBOC complex under the stereomicroscope for observation [2].

    - Talent covering the FBOC with camera filter

    - Talent placing FBOC complex under stereomicroscope

  - Illuminate the FBOC. Mount mini-LED video lights to flash hot shoe mount stands [1] and place the lights on the left and right sides of the FBOC [2]. Turn on the lights, and set the brightness to 100% and the color temperature to 5000 to 5600 Kelvin [3].

    - Talent mounting mini-LED video lights to hot shoe mount stands;

    - Place mini-LED video lights on the left and right sides of the FBOC

    - Talent turn on the lights, setting the brightness and color temperature

- Observation and Videography of Fly Behavior

  - Turn on the LED video lights, and adjust the stereo zoom microscope until the edge of the FBOC can be clearly seen with the naked eye [1]. Move the FBOC to the center of the field of view [2].

    - Talent turning the LED lights on and adjusting the stereo zoom microscope

    - Talent moving the FBOC to the center of the field of view

  - Attach the clamp of the universal telescope digital camera adapter to an eyepiece of the stereo microscope [1], then attach a compact digital camera to the adapter securely by alternately turning the camera mounting screw and camera fixing screw [2].

    - Talent attaching the clamp of camera to the eyepiece of microscope

    - Talent attaching a compact digital camera to the adapter

  - Turn on the digital camera, and turn the horizontal/vertical fine-tuning knobs [1] until the FBOC edge clearly appears in the center of the bright circular field of view on the camera's LCD screen [2].

    - Talent turning on the digital camera and turning the fine-tuning knobs

    - Clear FBOC edges on camera’s LCD screen

  - Rotate the mode dial to Aperture-priority auto Mode, press focus mode on the multi selector, choose Macro close-up, and then press OK button [1]. Move the zoom switch from the wide-angle end to the telephoto end [2] and zoom into the circular image until its central portion fills the full LCD screen [3]. Press the Movie-record button [4] and start recording the fly behavior [4a]. NOTE: Authors have modified the VO and hence the four-line limit exceeded and one extra screen shot 3.4.4a is added

    - Talent rotating the mode dial to Aperture-priority auto mode, pressing focus mode on multi selector and choose Macro close-up, and pressing the OK button

    - Talent moving the zoom switch from Wide angle to telephoto end

    - Talent zooming into the circular image and central image filling the LCD screen

    - Movie record button pressed

3.4.4a Added shot: LAB MEDIA: fly behavior.mp4.01:44-01:57

  - Turn the focus knob of the microscope until the flies in the FBOC are clearly visible [1]. Choose the fly behavior of interest for observation or video recording. Turn the zoom knob to zoom in and out [2] to achieve the desired magnification for observation or video recording [2a]. NOTE: Extra screen shot 3.5.2a is added

    - Focusing the flies in FBOC

    - Talent zooming in and zooming out and finally selecting the desired magnification for recording

3.5.2a Added shot: LAB MEDIA: fly behavior.mp4.00:05-00:15

# Results

- Results: Imaging System for Behavioral Studies of Drosophila

  - The representative photograph taken through the UV filter is clear and sharp [1], very similar to the photograph taken when the culture vial is not covered [2]. The quality of the photo taken through the glass of the Petri dish is very poor and partly blurred [3].

    - LAB MEDIA: Figure 5 Video editor: Please highlight the second/middle image of the figure

    - LAB MEDIA: Figure 5 Video editor: Please highlight the first/leftmost image of the figure

    - LAB MEDIA: Figure 5 Video editor: Please highlight the third/rightmost image of the figure

  - A photograph taken from the video recording showing the details of each part of the fly's body is shown here. Because the camera is connected to a zoom stereo microscope, it is very easy to shoot from panoramic to close-up shots using the zoom system [1].

    - LAB MEDIA: Figure 6

  - Using this protocol, fly behavior can be observed and documented from multiple viewing angles [1]. For example, the female fly is constantly rubbing the ovipositor with her hind legs during the process of laying eggs. This detail of egg-laying behavior cannot be seen clearly from the side [2].

    - LAB MEDIA: Figure 7

    - LAB MEDIA: Figure 7 Video editor: Please highlight portion with white egg in between the legs of the fly in the figure

# Conclusion

- Conclusion Interview Statements

  - Daxiang Yang: Using this simple and economical method, we can take a panoramic or close-up of the behavior or morphology of flies in all directions and obtain a good-quality video or photo.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested b-roll: LAB MEDIA: Figure 6