4. Operating Protein Biomarker Seeker software

1. Double click on the Protein Biomarker Seeker

executable file. A graphical user interface (GUI) window

will appear (Figure 1, top panel).  00:00-00:17

2. Enter the mass of the protein biomarker (as measured

in MS-linear mode) into the Mature Protein Mass field.

Next, enter the mass measurement error into the Mass

Tolerance field. The standard mass measurement error

is ±10 Da for a 10,000 Da protein.  00:17-01:00

3. Optionally, click on the Complementary b/y ion Protein

Mass Calculator button in order to calculate the protein

mass from a putative complementary fragment ion pair

(CFIP or b/y). A pop-up window, Protein Mass Calculator

Tool, will appear (Figure 1, bottom panel).  01:00-01:36

1. Enter the m/z of the putative CFIP and click on the

Add Pair button. The calculated protein mass will

appear.  01:36-02:10

2. Copy and paste this number into the Mature Protein

Mass field and close the Protein Mass Calculator

Tool window.  02:10-02:40

4. Select an N-terminal Signal Peptide Length by clicking

on the Set Residue Restriction box. A pop-up with a

sliding scale and cursor will appear. Move the cursor

to the desired signal peptide length (maximum 50). If

no signal peptide length is selected, an unrestricted

sequence truncation will be performed by the software.  02:40-03:48

5. Under the Fragment Ion Condition in the GUI, select

residues for polypeptide backbone cleavage (PBC). Click

on the boxes of one or more residues: D, E, N, and/or P.  03:48-04:22

1. Click on the Enter Fragment Ions (+1) To Be

Searched button. A pop-up Fragment Page will

appear. Next, click on the Add Fragment Ion button,

which corresponds to the number of fragment ions

to be entered, i.e., one click for each fragment ion.

A dropdown field will appear for each fragment ion

to be entered.  04:22-05:06

2. Enter the m/z of the fragment ions and their

associated m/z tolerance. When completed, click on

the Save and Close button.  05:06-7:17

NOTE: A reasonable m/z tolerance is ±1.5.

3. Select the minimum number of fragment ions that

must be matched for an identification by scrolling to

the desired number in the box to the right of How

Many Fragment Ions Need to be Matched.  07:17-07:59

NOTE: Three matches should be adequate.

4. Select cysteine residues to be in their oxidized state

by clicking on the corresponding circle. If no protein

identifications are found after the search, repeat the

search with cysteines in their reduced state. If no

identifications are found after the search, widen the

fragment ion tolerance to ±3 and repeat the search.  07:59-09:12

6. Under the File Setup, click on the Select FASTA

File button to browse and select the FASTA (text) file

containing the in silico protein sequences of the bacterial

strain previously constructed in protocol steps 3.1 to 3.2.

Then select an output folder and create an output file

name.  09:12-10:04

7. Click on the Run Search on File Entries button. A

pop-up window will appear entitled Confirm Search

Parameters (Figure 1, bottom panel), displaying the

search parameters before the search is initiated.  10:04-10:34

8. If the search parameters are correct, click on the Begin

Search button. If the search parameters are not correct,

click on the Cancel button and re-enter the correct

parameters. Once the search is initiated, the parameter

window closes, and a new pop-up window with a

progress bar appears (Figure 1, bottom panel) showing

the progress of the search and a running tally of the

number of identifications found.  10:34-10:39

9. Upon completion of the search (a few seconds), the

progress bar automatically closes, and a summary of the

search is displayed in the Log field of the GUI (Figure

2, top panel). In addition, a new pop-up window will

also appear displaying the protein identification(s) if any

(Figure 2, bottom panel).

NOTE: In silico protein sequences having unrecognized

residues, e.g., U or X, are automatically skipped from the

analysis and these sequences are subsequently reported

with a separate pop-up window to alert the operator as to

which (if any) sequences were skipped upon completion

of the search.  10:39-17:01