# Submission ID #: 62621

# Scriptwriter Name: Nilesh Kolhe

# Supervisor Name: Anastasia Gomez

# Project Page Link: https://www.jove.com/account/file-uploader?src=19099453

Title: Density Gradient Ultracentrifugation for Investigating Endocytic Recycling in Mammalian Cells

Authors and Affiliations: Wai Wa Ray Chan1*, Yu Qi Zhai1*, Kwok-Fai Lau1

1School of Life Sciences, The Chinese University of Hong Kong, Hong Kong Special Administrative Region, China

# Corresponding Authors:

Kwok-Fai Lau  kflau@cuhk.edu.hk

# Email Addresses for All Authors:

raychan@link.cuhk.edu.hk

1155118995@link.cuhk.edu.hk

kflau@cuhk.edu.hk

## Author Questionnaire

1. Microscopy: Does your protocol require the use of a dissecting or stereomicroscope for performing a complex dissection, microinjection technique, or something similar? NO

2. Software: Does the part of your protocol being filmed include step-by-step descriptions of software usage? NO

3. Interview statements: Considering the COVID-19-imposed mask-wearing and social distancing recommendations, which interview statement filming option is the most appropriate for your group?

☒ 	Interview Statements are read by JoVE’s voiceover talent.

4. Filming location: Will the filming need to take place in multiple locations? NO

Current Protocol Length

Number of Steps: 12

Number of Shots: 31

# Introduction

- Introductory Interview Statements

  - This protocol helps in isolating the recycling endosomes-containing fraction from transfected mammalian cells to facilitate the investigation of protein trafficking via endocytic trafficking.

    - 4.1.2

  - This technique is easy to handle and applicable to both tissue and cell samples.

    - 3.1

# Introduction of Demonstrator on Camera

  - Demonstrating the procedure will be Miss Zhai Yu Qi, a PhD student from Dr. Lau’s laboratory.

    - The named demonstrator(s) looks up from workbench or desk or microscope and acknowledges the camera.

# Protocol

- Mammalian Cell Culture, Transfection and Cell Harvest

  - To begin, plate 2 x 106 cells in a 100-millimeter culture dish [1-TXT]. On the next day, transfect the cells with Lipofectamine according to the manufacturer's instructions [2].

    - WIDE: Establishing shot for talent plating cells in a culture dish TEXT: 4 dishes per transfection

    - Talent adding Lipofectamine to the cells

  - To harvest the cells, discard the culture medium 48 hours post-transfection [1] and wash the cells twice with ice-cold PBS [2-TXT].

    - Talent removing the culture medium from the dishes

    - Talent washing the cells with PBS TEXT: 10 mM Na+ phosphates, 2.68 mM KCl, 140 mM NaCl

  - Then, add 1 milliliter of ice-cold PBS supplemented with 0.5x protease inhibitor cocktail and 0.5x phosphatase inhibitor cocktail to each dish [1]. Next, using a cell scraper, collect the cells [2] and transfer the cell suspension to a 15-milliliter centrifuge tube [3].

    - Talent adding PBS+ to each dish

    - Talent collecting the cells

    - Talent transferring the cell suspension to a 15 mL tube

  - Pellet the cells by centrifugation using a swing bucket rotor [1-TXT]. Discard the supernatant [2] and resuspend the cell pellet gently in 5 milliliters of homogenization buffer [3].

    - Talent placing the tubes in centrifuge TEXT: 5 min, 400 x g

    - Talent removing the supernatant

    - Talent resuspending the pellet in homogenization buffer

  - Collect the cells again by centrifugation [1-TXT] and resuspend the cell pellet in 1 milliliter of homogenization buffer [2]. Homogenize the cells with a Dounce homogenizer using 15 to 20 strokes [3], then transfer the homogenate to a 2-milliliter centrifugation tube [4].

    - Talent placing the tubes in centrifuge TEXT: 10 min, 1,300 x g

    - Talent resuspending the cell pellet in 1mL HB

    - Shot of cell homogenization in Dounce homogenizer

    - Talent transferring the homogenate to a 2 mL tube

  - Add 700 microliters of homogenization buffer to the homogenate [1], then spin the diluted homogenate [2-TXT] and collect 1.5 milliliters of the supernatant [3].

    - Talent adding HB to the tube

    - Talent placing the tubes in centrifuge TEXT: 10 min, 2,000 x g at 4 °C

    - Talent collecting the 1.5 mL of the supernatant

  - Spin the collected supernatant again [1], then collect 1.4 milliliters of the supernatant and label it as post-nuclear supernatant [2].

    - Talent placing the tube in the centrifuge

    - Talent collecting the supernatant

- Density Gradient Column Preparation

  - To prepare the density gradient column, transfer 1.2 milliliters of the post-nuclear supernatant to an ultracentrifuge tube [1], then add 1 milliliter of 62% sucrose solution to the sample [2-TXT] and mix well by gentle pipetting [3]. Videographer: This step is important!

    - Talent adding PNS to an ultracentrifuge tube

    - Talent adding sucrose solution to the tube containing sample TEXT: 2.351 M sucrose, 3 mM imidazole at pH 7.4

    - Talent mixing the content

  - Next, carefully add 3.3 milliliters of 35% sucrose solution on top of the sample [1-TXT], followed by 2.2 milliliters of 25% sucrose solution on top of the 35% sucrose solution [2-TXT]. Finally, fill the ultracentrifuge tube to the top with homogenization buffer [3]. Videographer: This step is important!

    - Talent adding 35% sucrose solution on the top of the sample TEXT: 1.177 M sucrose, 3 mM imidazole at pH 7.4

    - Talent adding 25% sucrose solution on the top of the 35% solution TEXT: 0.806 M sucrose, 3 mM imidazole at pH 7.4

    - Talent filling the tube with HB

- Fractionation and Recovery of Recycling Endosome-Containing Fraction

  - Centrifuge the density gradient column [1-TXT] and carefully collect 12 fractions of 1 milliliter each, starting from the top of the gradient [2]. Videographer: This step is important!

    - Talent placing the column in the ultracentrifuge TEXT: 3h, 210,000 x g at 4 °C

    - ECU: Talent collecting the fractions

  - Next, dilute all the fractions with 1 milliliter of dilution buffer [1-TXT] and centrifuge the diluted samples [2-TXT]. Videographer: This step is important!

    - Talent adding dilution buffer to each fraction TEXT: Dilution buffer: 3 mM imidazole at pH 7.4, 1 mM EDTA

    - Talent placing the tubes in centrifuge TEXT: 1 h, 100,000 x g at 4 °C

  - After centrifugation, aspirate the supernatant [1] and add 50 microliters of 1x sample buffer to harvest the fractions [2].

    - Talent removing the supernatant

    - Talent adding sample buffer to the tubes

# Results

- Results: Density Gradient Ultracentrifugation for Investigating Endocytic Recycling in Mammalian Cells

  - The recycling endosome marker Rab11 (rab-eleven) was detected in fraction 7 (seven) [1]. Other subcellular markers, including β-COP (beta-cop) [2], COX IV (cox-four) [3], GAPDH (gap-D-H) [4], EEA1 (E-E-A-One) [5], Rab7 (rab-seven) [6], and Lamp1 (lamp-one) were also probed [7]. A positive EEA1 signal was also detected in fraction 7 [8].

    - LAB MEDIA: Figure 1 Video editor: please emphasize the band in fraction 7 for Rab11 (recycling endosome; first row) from the image

    - LAB MEDIA: Figure 1 Video editor: please emphasize the β-COP (Golgi) marker lane from the image

    - LAB MEDIA: Figure 1 Video editor: please emphasize the COX IV (mito) marker lane from the image

    - LAB MEDIA: Figure 1 Video editor: please emphasize the GAPDH (cytosol) marker lane from the image

    - LAB MEDIA: Figure 1 Video editor: please emphasize the EEA1 (early endosome) marker lane from the image

    - LAB MEDIA: Figure 1 Video editor: please emphasize the Rab7 (late endosome) marker lane from the image

    - LAB MEDIA: Figure 1 Video editor: please emphasize the Lamp1 (lysosome) marker lane from the image

    - LAB MEDIA: Figure 1 Video editor: please emphasize the band in fraction 7 for EEA1 (early endosome; fifth row) from the image

  - The measured band intensities of GAPDH [1], COX IV [2], and Rab11 in both fraction 7 and the post-nuclear supernatant are shown here [3].

    - LAB MEDIA: Figure 1 Video Editor: please emphasize the GAPDH bar from the graph

    - LAB MEDIA: Figure 1 Video Editor: please emphasize the COX IV bar from the graph

    - LAB MEDIA: Figure 1 Video Editor: please emphasize the Rab 11 bar from the graph

  - After transfecting HEK293 (hek-two-nine-three) cells with either ELMO1 (elmo-one) [1], ELMO1 + ARF6 Q67L (elmo-one plus A-R-F-six-Q-sixty-seven-L) [2], or ELMO1 + ARF6 Q67L + FE65 (elmo-one plus A-R-F-six-Q-sixty-seven-L plus F-E-sixty-five) [3], no changes were found in ELMO1 distribution with ARF6 Q67L [4] and FE65 overexpression [5].

    - LAB MEDIA: Figure 2A Video Editor: please emphasize on lanes (top 3 rows) marked with ELMO1

    - LAB MEDIA: Figure 2A Video Editor: please emphasize on lanes (middle 3) marked with ELMO1 + ARF6 Q67L

    - LAB MEDIA: Figure 2A Video Editor: please emphasize on lanes (bottom 3) marked with ELMO1 + ARF6 Q67L + FE65

    - LAB MEDIA: Figure 2A Video Editor: please emphasize the bands for ELMO1 in fraction 7, 11 and 12 from ELMO1 + ARF6 Q67L

    - LAB MEDIA: Figure 2A Video Editor: please emphasize the bands for ELMO1 in fraction 7, 11 and 12 from ELMO1 + ARF6 Q67L + FE65

  - The ELMO1 level in fraction 7 was compared between different transfections [1] and found to be elevated after co-transfection of ARF6 Q67L [2]. A further increase was observed when both ARF6 Q67L and FE65 were co-expressed [3].

    - LAB MEDIA: Figure 2B Video editor: Only show the graph on right, please emphasize the ELMO1 black bar

    - LAB MEDIA: Figure 2B Video editor: Only show the graph on right, please emphasize the black bar for ELMO1 from ELMO1 + ARF6 Q67L

    - LAB MEDIA: Figure 2B Video editor: Only show the graph on right, please emphasize the black bar for ELMO1 from ELMO1 + ARF6 Q67L+ FE65

  - Conversely, knocking out FE65 diminished [1] the ARF6-mediated ELMO1 enrichment in fraction 7 [2].

    - LAB MEDIA:  Figure 2C Video Editor: Only show the graph on right, please emphasize the white bar for FE65 KO from ELMO1 + ARF6 Q67L

    - LAB MEDIA: Figure 2C Video Editor: Only show the graph on right, please emphasize the black bar for WT from ELMO1 + ARF6 Q67L

# Conclusion

- Conclusion Interview Statements

  - The fraction obtained could be used for subsequent analysis, for example Western blotting, to visualize protein level changes in the fraction.

    - LAB MEDIA: Figure 2B and 2C: Video Editor only show the Western blotting images on the left