# Submission ID #:  62680

# Scriptwriter Name: Nilesh Kolhe

# Supervisor Name: Anastasia Gomez

# Project Page Link: https://www.jove.com/account/file-uploader?src=19117448

Title:   Polysome Profiling Without Gradient Makers or Fractionation Systems

Authors and Affiliations: Mack Sobhany1, Robin E. Stanley1

1Signal Transduction Laboratory, National Institute of Environmental Health Sciences, Department of Health and Human Services, National Institutes of Health

# Corresponding Authors:

robin.stanley@nih.gov

sobhany@niehs.nih.gov

# Email Addresses for All Authors:

robin.stanley@nih.gov

sobhany@niehs.nih.gov

## Author Questionnaire

- Microscopy: Does your protocol require the use of a dissecting or stereomicroscope for performing a complex dissection, microinjection technique, or something similar?  No

2. Software: Does the part of your protocol being filmed include step-by-step descriptions of software usage?  No

3. Interview statements: Considering the COVID-19-imposed mask-wearing and social distancing recommendations, which interview statement filming option is the most appropriate for your group?

☒ 	Interviewees self-record interview statements. JoVE can provide support for this option.

4. Filming location: Will the filming need to take place in multiple locations?   No

Current Protocol Length

Number of Steps: 17

Number of Shots: 36

# Introduction

- Introductory Interview Statements

  - Robin Stanley: This protocol describes how to generate a polysome profile, which provides molecular details about the activity of ribosomes inside the cell [1].

    - LAB MEDIA: To be provided by Authors: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B role: 5.4.2.

  - Mack Sobhany: This technique allows users to obtain the valuable information provided by a polysome profile who do not have access to automated gradient fractionation systems [1].

    - LAB MEDIA: To be provided by Authors: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

  - Mack Sobhany: Take your time while preparing the gradients and store the gradients in a location where they will not be disturbed by a compressor or other mechanical operations as they settle into a linear gradient [1].

1.3.1. LAB MEDIA: To be provided by Authors: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B role: 2.7.1.

# Protocol

- Preparation of 7 to 47% Sucrose Gradients

  - To begin, prepare stock solutions of 7 and 47% sucrose in sucrose gradient buffer [1-TXT]. Filter-sterilize the sucrose stock solutions through a 0.22-micron filter [2].

    - WIDE: Talent at workbench preparing sucrose stock solution TEXT: Sucrose gradient buffer: 20 mM Tris-HCl pH 7.4, 60 mM KCl, 10 mM MgCl2, and 1 mM DTT

    - Talent filter sterilizing the solution

  - Prepare 14 milliliters of 17, 27, and 37% sucrose solutions by dispensing and mixing the 7 and 47% stock solutions [1-TXT].

    - Talent preparing desired concentrations of sucrose solutions TEXT: Refer to Table 1 for details

  - Place six polypropylene centrifuge tubes into a full-view test tube rack. Ensure that there is enough space between the tubes so that actions with one tube do not disturb the others [1].

    - Talent placing centrifuge tubes into a full view test tube rack

  - Attach a 9-inch, 22-Gauge needle with a blunt tip to a 3-milliliter syringe [1] and perform a test fill and dispense to ensure that the syringe can hold the sucrose solution without any dripping before setting up the gradients [2].

    - Talent attaching a needle to the syringe

    - Talent performing a test fill and dispensation from the syringe

  - Add 2 milliliters of the 7% sucrose to the bottom of each centrifuge tube [1]. Then add 2 milliliters of the 17% sucrose beneath the 7% solution by positioning the needle tip within the immediate vicinity of the tube bottom [2]. Carefully and slowly dispense the solution [3]. Videographer: This step is important!

    - Talent adding 7% sucrose to the bottom of the tube

    - Talent positioning the needle tip within the immediate vicinity of the tube bottom

    - Talent dispensing the solution

  - Repeat the procedure with 2 milliliters each of the 27, 37, and 47% sucrose solution [1], ensuring that each layer is distinguishable from the other by a line marking the separation of densities [2]. Videographer: This step is important!

    - Talent adding higher percentage sucrose solution

    - Talent checking for line marking for the separation of densities/LAB MEDIA: Figure 2

  - Store the gradients at 4 degrees Celsius overnight to allow them to settle into a continuous, increasing percentage of sucrose [1-TXT].

    - Talent leaving the gradient for overnight incubation TEXT: Linear sucrose gradients are stable for up to 48 h Videographer: This step is important!

- Preparation of Yeast Cell Extracts

  - After the growth and harvesting of yeast Saccharomyces cerevisiae cells, resuspend the cells in chilled 700 microliters of polysome extraction buffer [1-TXT]. Add 100 units of RNase (RN-ase) inhibitor [2] and transfer it to a 1.5-milliliter centrifuge tube [3].

    - Talent resuspending the cells in extraction buffer TEXT: Refer to the text for details of polysome extraction buffer

    - Talent adding RNase inhibitor to the tube

    - Talent transferring the mixture to the centrifuge tube

  - Add 400 microliters of pre-chilled glass beads with an approximate size of 425 to 600 microns to a centrifuge tube [1] and disrupt the yeast cells by vigorous agitation in a bead-beater for 5 minutes [2].

    - Talent adding glass beads to a centrifuge tube

    - Talent disrupting the cells in bead-beater

  - After disrupting the cells, clarify the lysate by centrifugation [1-TXT]. Determine the concentration of the RNA in the clarified lysate by measuring the absorbance at 260 nanometers using a fluorescence-based RNA detection system [2].

    - Talent placing the tubes in centrifuge TEXT: 5 min, 8,000 x g at 4 C

    - Talent measuring the concentration of RNA

  - Ensure that RNA concentration is 0.5 to 1 microgram per microliter. If the RNA concentration is too low, reduce the volume of extraction buffer used to resuspend the cells [1].

    - Talent checking the appropriate concentration of RNA

- Centrifugation of Gradients

  - Carefully load the lysate onto the top of the gradients [1]. Place the pipette tip against the inner wall, at the top of the polypropylene tube [2]. Angle the tube and slowly dispense the lysate onto the top of the gradient, dribbling against the wall [3-TXT]. Videographer: This step is important!

    - Talent loading the lysate onto the top of the gradient

    - Talent placing the pipette tip at the top of the tube

    - Talent dispensing the lysate onto the top of the gradient TEXT: Do not disturb the gradient

  - Gently place the tubes into the pre-chilled buckets of a swinging bucket rotor [1] and centrifuge the gradients [2-TXT]. Videographer: This step is important!

    - Talent placing the tubes into swinging bucket rotor

    - Talent turning the centrifuge on TEXT: 150 min, 260,110 x g at 4 C

- Fraction and Data Collection

  - After centrifugation, carefully remove the centrifuge tubes from the swinging bucket rotor [1] and place them in a tube holder [2]. Label the 96-well plates to store the fractions [3] and pre-chill on ice [4].

    - Talent removing the tubes from the swinging bucket rotor

    - Talent placing the tubes in a tube holder

    - Talent labeling the 96-well plates

    - Talent placing the plates on ice

  - Collect 100 or 200 microliter fractions starting from the top of the gradient by carefully inserting a pipette tip into the top of the gradient [1] and collecting the fractions until the entire gradient is aliquoted [2]. Videographer: This step is important!

    - Talent inserting a pipette tip into the top of the gradient

    - Talent collecting the fractions from the gradient

  - Transfer each fraction to the 96-well plate to reach the bottom of the centrifuge tube [1] and keep the collected fractions on ice [2].

    - Talent transferring the fraction to the 96-well plate

    - Talent placing the collected fraction/tube on the ice

  - Measure the absorbance of each fraction at 254 nanometers with a spectrophotometer against the 7 and 47% sucrose solutions as blanks [1]. Create the polysome profile by plotting the fraction number versus absorbance [2].

    - Talent measuring the absorbance on the spectrophotometer

    - Talent plotting a polysome profile with monitor visible in the screen

# Results

- Results: Polysome Profiling Without Gradient Makers or Fractionation Systems

  - Three representative polysome profiles from yeast, S. cerevisiae, are shown [1]. The crest of each ribosomal subunit and polysome peak is apparent on each profile [2].

    - LAB MEDIA: Figure 3

    - LAB MEDIA: Figure 3 Video Editor: Please emphasize on the 40S, 60S, 80S, and polysomes

  - A representative profile from an automated density fractionation system is shown here [1]. This profile was produced from a continuous absorbance profile as the sucrose gradient was displaced from the bottom up by a chase solution through a detector flow cell and collected in fractions [2].

    - LAB MEDIA: Figure 3A

    - LAB MEDIA: Figure 3A

  - The polysome profile generated by hand-fractionating 200 [1] and 100 microliter samples is shown here [2].

    - LAB MEDIA: Figure 3B

    - LAB MEDIA: Figure 3C

# Conclusion

- Conclusion Interview Statements

  - Mack Sobhany: The most important thing to remember with this procedure is to not disrupt the gradients. Take care not to introduce air bubbles or disrupt gradient by dispensing solution too rapidly [1].

    - LAB MEDIA: To be provided by Authors: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B role: 2.5, 2.6 and 2.7

  - Robin Stanley: Following this procedure RNA can be extracted from the gradient fractions for further analysis to determine mRNAs that are associating with translating ribosomes [1].

    - LAB MEDIA: To be provided by Authors: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B role: 3.1-3.4