TITLE:

A visual protocol for the quantitative 31P NMR analysis of lignins and Ttannins

AUTHORS AND AFFILIATIONS:

Nicolò Pajer1, Claudia Crestini*1, Dimitris S. Argyropoulos*2

1Department of Molecular Sciences and Nanosystems, Ca’ Foscari University of Venezia, Venezia, Italy

2Departments of Chemistry and Forest Biomaterials, North Carolina State University, Raleigh, North Carolina, United States of America

nicolo.pajer@unive.it

claudia.crestini@unive.it

dsargyro@ncsu.edu

SUMMARY:

# 31P NMR is a powerful tool for the structural elucidation of polyphenols. This fast, easy, precise, quantitative, and highly reproducible analytical procedure, that allows for the quantification and differentiation of the different types of hydroxy, phenolic and carboxylic groups in lignins and tannins has now become a routine analytical tool.

ABSTRACT:

The development of sustainable biorefinery products is confronted, amongst others, with the challenge of lignin and tannin valorization. These abundant, renewable aromatic biopolymers have not been widely exploited due to their inherent structural complexity and high degrees of variability and species diversity. The lack of a defined primary structure for these polyphenols is further compounded with complex chemical alterations induced during processing, eventually imparting to them a large variety of structural features of extreme significance for any further utilization efforts.

Consequently, a protocol for the rapid, simple, and unequivocal identification and quantification of the various functional groups, present in natural polyphenols, is a fundamental prerequisite for understanding and accordingly tailor their reactivity and eventual utility.

Quantitative 31P NMR offers the opportunity to rapidly and reliably identify unsubstituted, o-mono substituted, and o-disubstituted phenols, aliphatic OH’s, and carboxylic acid moieties in lignins and tannins with broad application potential.

The methodology consists of an in situ quantitative lignin or tannin labeling procedure using a suitable 31P containing probe, followed by the acquisition of a quantitative 31P NMR spectrum in the presence of an internal standard. The high natural abundance of the 31P nucleus allows for small amounts of sample (~30 mg) and short NMR acquisition times (~30-120 min) with well-resolved 31P signals that are highly dependent on the surrounding chemical environment of the labeled OH group.

INTRODUCTION:

This procedure, which was recently published in Nature Protocols1 has been cited over 3000 times in the archival literature and has now become a routine measurement for lignin and tannin characterization since it provides essential, rapid, and reproducible structural information for them.

Lignin and tannins

When Green Chemistry was introduced by Paul T. Anastas and John C. Werner2, 3, it drastically changed the general conception of Chemistry. In particular, the importance of employing sustainable materials instead of fossil feedstocks, like oil and coal as a starting point is highlighted as a crucial point2, 3. Among the different types of biomass, lignin is the most abundant aromatic biopolymer and, can be seen as a potential source for industrial commodities and high-value products4.

Lignin is the second most abundant wood-constituent (with cellulose being first and hemicellulose third). Its content in plants varies depending on plant-type hardwoods are characterized by a lower amount of lignin if compared to softwoods (20% ± 4% vs. 28% ± 4%). In addition, lignin distribution within vegetable tissue is not homogeneous: the higher content of lignin can be found in the cell wall5, 6.  Lignin is a polyphenolic material industrially obtained as a by-product of the paper/cellulose industry7. It is recovered from the wood pulping process, in which wood chips are mostly processed in the presence of OH- and/or OH- + HS- ions conditions to separate cellulose from hemicellulose and lignin (Soda and/or Kraft processes)8, 9.

The first attempts to study lignin were made by Payen and Schultze, respectively in 1838 and 186510. In 1977 Adler summarized all the relevant available knowledge of that time11. It is currently recognized that the lignin building blocks are the three phenyl-propanoidic units: p-coumaryl, coniferyl and sinapyl alcohols. These monomers, thanks to a combinatorial free radical polymerization process, give rise to p-hydroxyphenyl, guaiacyl and sinapyl units that eventually broadly constitute lignin (Figure 1)12. The lack of a primary structure in lignins implies an inherent difficulty for its structural characterization. Accordingly, the evaluation of the distribution of molecular weight has always been rather controversial. Milled wood lignin, the lignin isolated under mild conditions that approximate mostly protolignin10, composed by oligomers13 which highly interact via supramolecular aggregation processes14, 15.

[Place Figure 1 here]

Lignins are commonly classified depending on: (a) the type of wood from which they derive (e.g., hardwood and softwood), (b) the process used to isolate it. The most important industrial lignin types are Kraft, Lignosulfonates, and Organosolv.

The structure of lignin is highly dependent upon its origin and processing chemistry. More specifically, when the rather complex and irregular structure of lignin is compounded with its natural diversity and the complex processing chemistries, a material of extreme variability, diversity  and heterogeneity emerges, which limits its use to low-value applications16. While softwood lignins contain mainly guaiacyl units (G) with negligible amounts of p-hydroxyphenyl groups (G lignin), hardwood lignins are composed by guaiacyl and syringyl subunits (GS lignin) in varying ratios and grass lignins are constituted by guaiacyl, syringyl and p-hydroxyphenyl (GSH lignin) subunits. If the extractive approach used to isolate lignin is considered, it dramatically affects the structure of the emerging lignin17. Figure 2 depicts three lignin structures, differing by the isolation approach employed. Some considerations regarding the effect of the extraction method could be highlighted. Firstly, Kraft lignin is a dealkylated, highly fragmented, and condensed lignin, while Organosolv lignin has a structure similar to  milled wood lignin (isolated using the Bjorkman approach)18–20. Finally, lignosulfonates are characterized by a high degree of sulfonation, depending on the intensity and the conditions of the extractive sulfonation process.

[Place Figure 2 here]

Like lignins, tannins are polyphenolic compounds that are found in plants. A recent and updated review on tannins extractive approaches and applications was recently released by Das et al.21. The importance of tannins in everyday life can be highlighted considering two examples. They impart taste and color to wines22, and their poly-phenolic structure offers antioxidant characteristics and makes them ideal for application in the tanning industry23. Tannins are divided into two classes, hydrolyzable and non-hydrolyzable. Hydrolyzable tannins can be considered as a polymer of gallic, di-gallic, and ellagic acid esters (Figure 3). These esters result from the esterification of the phenolic acids with sugar molecules (e.g. glucose, rhamnose, and arabinose).

[Place Figure 3 here]

Non-hydrolyzable tannins, also known as condensed tannins, are polymers and oligomers deriving from flavan-3-ols. . Among flavan-3-ols, catechins and gallocatechin are the most frequent, they are, colorless crystalline compounds (Figure 4). The polymerization creates a polymer that is characterized by a helicoidal structure in which the aromatic hydroxy groups are directed on the exterior of the helix, while the pyran oxygens are in the interior.

[Place Figure 4 here]

Characterization of lignin and tannins using NMR

Two types of information are crucial in lignin or tannin characterization, (a) chemical structure (e.g., hydroxy group content, nature, and frequency of interunit linkages) and (b) molecular weight and polydispersity. Since the early studies on lignin, different techniques have been employed to achieve these goals and two classes of methods have emerged, chemical and physical methods.

In lignin chemistry, chemical methods, such as alkaline nitrobenzene oxidation, derivatization followed by reductive cleavage, permanganate oxidation, and thioacidolysis, have been historically widely used24–29. However, even if the analytical protocols have been implemented and optimized, they are time-demanding, laborious, and require extensive experimental skills30. Alternatively, from the beginning of the instrumental analysis, physical methods have been used to perform lignin and tannins characterisation31. These techniques allow overcoming the problems of classical methods making it easy to characterize lignin structure.

Among the instrumental techniques, Nuclear Magnetic Resonance (NMR) allows obtaining different information about lignin structure and chemical composition. In particular, information from quantitative monodimensional 1H NMR spectra and quantitative 13C NMR spectra can give information of different types of lignin interunit bondings 32–35. Unfortunately, monodimensional spectra suffer from signal overlap which can seriously undermine signal integration efforts. Quantitative versions of HSQC (Heteronuclear Single Quantum Coherence), Q-HSQC (Quantitative – Heteronuclear Single Quantum Coherence), have been used to better understand lignin structure, providing useful information about internal linkages, however, they still cannot be fully used for the quantitative determination of the various buildings units13, 36, 37.

To overcome the issues associated with mono- and two-dimensional NMR, substrate derivatization has been considered. Amongst the advantages of this approach is the fact that specific labels can be introduced within the complex macromolecule and no-spectral interference results from the solvent in which the labeled substrates are dissolved1. Verkade was the pioneer in this field, performing 31P NMR analysis of phosphorous derivatives of coal derivatives and related compounds38. In its publication, a screening of different phosphorus-containing reagents (phospholanes) was performed, and the chemical shift of different labeled compounds was recorded. The application of derivatization for the quantitative and qualitative analysis of hydroxy groups in lignin was firstly introduced by Argyropoulos’ team in 1991. After studying the derivatization of lignin model compounds using phosphorus-containing reagents, his group paved the way for one of the most daily-used techniques in lignin chemistry, 31P NMR analysis39–43. Among the different phospholanes examined, Argyropoulos arrived at the use of 2-chloro-4,4,5,5-tetramethyl-1,3-2-dioxaphospholane (TMDP) as being the most suitable one to perform lignin analysis44. TMDP selectively reacts with hydroxy groups causing the quantitative formation of phosphorus-containing derivatives which are characterized by specific 31P NMR chemical shifts (Figure 5).

[Place Figure 5 here]

Sample derivatization is performed in a pyridine/chloroform (1.6:1) mixture; this choice is the result of an accurate evaluation. Pyridine has two advantages. Firstly, the selection of a solvent characterized by a Hildebrand parameter of about 22.1 MPa1/2 simplifies and amplifies lignin solubilization 45. Consequently, the addition of pyridine as a solvent, whose Hildebrand parameter equals 21.7, is thus optimal. Secondly, the reaction of TMDP with hydroxy groups is accompanied by the formation of hydrochloric acid (HCl) as a by-product with concomitant negative implications toward the facile formation of lignin-phospholane derivatives. For this reason, the resulting HCl needs to be neutralized. The basicity of the pyridine when present in large excess, relative to TMDP, allows for the neutralization of the HCl (via the formation of pyridine hydrochloride).

The use of the recommended pyridine/deuterated chloroform binary solvent system is based on three reasons. Firstly, it favors sample dissolution. Secondly, as pyridine hydrochloride is soluble in chloroform, it can be useful to prevent precipitation and deterioration of the final spectrum. Thirdly, deuterated chloroform is chosen for its unique singlet signal, which allows locking of the NMR spectrometer during the acquisition process. Sample derivatization is performed in the presence of an internal standard. In this way, when the sample is derivatized as well as the standard, the comparison of the integrals of the peaks of the sample and the standard allows the quantification of the amount for each type of hydroxy group present. Various compounds have been considered as internal standards. These compounds are characterized by a single hydroxy group per molecule, offering a single sharp signal in the 31P NMR spectrum after derivatization. The selection of the standard must be made carefully, its signal should not overlap with those of the derivatized sample. Cholesterol was widely used during the early days. However, a partial overlap with signals arising from aliphatic hydroxy group limits its use. For routine analysis, internal standard solutions of N-hydroxy-5-norbornene-2,3-dicarboximide (NHND) are preferred. However, owing to NHND instability, its standard solutions can be stored only for a few days46.

PROTOCOL:

The following flow chart (Figure 6) outlines the whole experimental protocol to perform a 31P NMR analysis of lignin and tannins.

[Place Figure 6 here]

1. Sample pretreatment

1.1 	Dry an aliquot (around 100 mg) of the analyte (lignin or tannin sample) overnight in a vacuum oven set at 40 °C.

NOTE: Particular attention is needed on the temperature selection since temperatures higher than 40 °C may chemically alter the sensitive structure of the examined polyphenols.

1.2 	After drying, rapidly transfer the sample to an anhydrous calcium sulfate desiccator until it reaches room temperature. This step is compulsory to avoid the sample absorbing humidity from the environment.

2. Solvent solution preparation

  - 2.1 Prepare a  pyridine/deuterated chloroform solvent mixture in a 20 mL sample vial      (CAUTION) by mixing anhydrous pyridine and deuterated chloroform in a 1.6/1 (v/v) ratio.

CAUTION: Pay attention, while manipulating pyridine and deuterated chloroform, these compounds are flammable, harmful, and toxic. Prepare and use the solution in a well-ventilated fume-hood using appropriate gloves.

  - 2.2 Add 5-8g of  well-washed and dried activated 5A molecular sieves in 3.2 mm pellets to remove water traces. In addition, the use of a septum cap is highly recommended to prevent air contact and moisture contamination of the solvent system. Store the prepared solution in the dark.

3. Internal standard solution (IS) preparation

3.1 In a 2 mL Erlenmeyer flask, prepare a 0.1 M solution of chromium (III) acetylacetonate (CAUTION) (about 10 mg) and internal standard (around 35.8 mg of NHND or 77.3 mg of cholesterol) in the solvent solution previously prepared. Chromium (III) acetylacetonate is harmful, during its manipulation, wear appropriate gloves.

3.2 Record the exact weight of the IS added in the IS solution.

3.3 Transfer the IS solution in a vial equipped with a sealed cap containing activated molecular sieves (see point 2.2) and store in the dark at °C.

4. 	NMR sample solution preparation

4.1 	Accurately weigh ~ 30 mg of sample in a 2 mL vial equipped with a stirring bar. Seal the vial with a septum cap.

4.2 Add 0.5 mL of the solvent system solution to the sample vial.

4.3 Transfer 100 µL of the IS solution in the sample vial via a micropipette. Magnetically stir the resulting dispersion (500 rpm) until all the lignin or the tannin is dissolved, resulting in a clear solution.

NOTE: Since complete sample solubilization is imperative, this step could take up to twelve hours.

4.4 Transfer 0.1 mL of TMDP (CAUTION) to the sample solution. Place the sample under vigorous magnetic stirring. Keep the sample solution sealed. Keep the sample solution sealed. TMDP and its vapours are corrosive, harmful and rapidly interact with water. Use TMDP in a well-ventilated fume-hood while wearing appropriate gloves.

NOTE: The formation of a yellow precipitate is due to the presence of water traces either in the sample or in the pyridine/chloroform solution. In such a case the procedure needs to be repeated by ensuring all possible moisture contamination is avoided.

4.5	 Transfer the sample solution into an NMR tube using a Pasteur pipette.

5. 	NMR analysis

5.1 	 Load the tube into the NMR instrument. The spectrometer used to perform this analysis needs broadband to observe the probe.

5.2 	Set the experimental parameters according to the setting shown in  Table 11

[Place Table 1 here]

5.3 	Set the spectrometer frequency using the resonance frequency of deuterated chloroform,  shim the sample and tune the spectrometer. Then, start the acquisition.

6. 	Spectrum processing and analysis

Process 31P NMR raw data by a standard appropriate  software according to the following steps..

6.1 Perform Fourier transformation.

6.2 Adjust Phase by manual phase correction (Processing > Phase Correction > Manual Correction).

6.3

Correct baseline manually carefully setting zero points. (Processing > Baseline > Multipoint Baseline Correction)

6.4 Signal calibration.

Set the signal for the phosphitylated water at the chemical shift value of 132.2 ppm . (Analysis > Reference > Reference).

NOTE: The presence of a sharp 31P signal at 175 ppm is due to the excess of TMDP. Its presence ensures the complete derivatization of the sample. If this peak is absent then one needs to revisit the whole procedure by ensuring thorough sample and solvent drying and by adding more TMDP. Once this is ensured, the spectrum is zoomed in the spectral range 132 to around 150 ppm. (Figure 7).

[Place Figure 7 here]

6.5 Integration

Normalize integration by setting the internal standard are to 1.0 (click on the peak > Edit Integral > Normalized: 1.00). Perform spectrum integration according to the chemical shifts reported in the following tables  Use Table 2 for lignins and Table 3 for tannins.

[Place Table 2 here]

[Place Table 3 here]

NOTE: Using standard spectral-processing software, it is possible to set pre-defined regions of the chemical shift to be integrated. This opportunity is particularly useful when several spectra have to be processed.

7.  Functional group quantification

7.1 Calculate the concentration of the IS solution.

IS concentration M=mass of IS(mg)MWIS(g/mol)∙ISpurity(%)100∙1volume of IS solution (mL)

IS concentration M=mass of IS(mg)MWIS(g/mol)∙ISpurity(%)100∙1volume of IS solution (mL)

7.2 Calculate the equivalent amount of the specific signal:

Functional group amount mmol per gram of lignin=normalized peak area ∙IS concentration∙volume of IS added to the sample mmolsample lignin or tannin weight g

Functional group amount (mmol per gram of lignin)=normalized peak areasample weight (g)∙IS concentration∙0,1

Functional group amount mmol per gram of lignin=normalized peak area ∙IS concentration∙volume of IS added to the sample mmolsample lignin or tannin weight g

Functional group amount (mmol per gram of lignin)=normalized peak areasample weight (g)∙IS concentration∙0,1

REPRESENTATIVE RESULTS:

The described protocol can be applied both for the analysis of lignins and/or tannins. In lignin chemistry, this method is fundamental because it allows the detection and quantification of the different types of hydroxy groups. Figure 8a and Figure 8ddepict a good for the spectrum quantification of the various hydroxy groups in a softwood Kraft lignin derivatized with TMDP. The spectrum depicted in Figure 8a was recorde using a 300 MHz NMR spectrometer, while Figure 8d was recorded with a 700 MHz NMR instrument.

[Place Figure 8 here]

These spectra were carefully recorded and processed manually. The typical signals for aliphatic (150-145 ppm), aromatic (145-137 ppm), and carboxylic (136-134 ppm) hydroxy groups are very well resolved and as such readily integrated. If the spectral window is opened (from 95 to 190 ppm, Figure 8), three sharp, strong peaks (175, 144, and 132 ppm) are apparent. Those are due to the excess of TMDP, the internal standard (cholesterol or NHND), and the hydroxylated-TMDP (caused by water traces) respectively.

In contrast to kraft and organosolv lignin, lignosulfonates are insoluble in the pyridine/chloroform mixture. To obtain a reliable 31P NMR spectrum, solubility is mandatory. To overcome this issue, lignosulfonates can be converted to the corresponding lignosulfonic acids before derivatization. Treating lignosulfonates solutions with strong acids (i.e. sulfuric acid), or acid exchange resins (e.g., Dowex 1H, a strong acidic cation exchanger) drives the conversion of all the sulfonate groups in their acidic forms. The resulting products, which can be removed from the acidic solution using selective adsorptive resins (XAD-7, a polar adsorbent, used for the isolation of compounds characterized by molecular weights up to 60,000 u.m.a.)) can be then analyzed using this protocol. Figure 8b shows the quantitative 31P NMR spectrum of a TMDP derivatized lignosulfonic acid. Even in this case, the different signals of the hydroxy groups are clear. Figure 8c shows a typical quantitative 31P NMR spectrum of a tannin sample derivatized using TMDP. A characteristic signal from the different aliphatic OH (Ring C), pyrogallol, and catechol units in ring B and units in ring A are well visible.

FIGURE AND TABLE LEGENDS:

Figure 1: A representative model of softwood lignin in which the different types of bonds are highlighted.

Figure 2: Representative structures for technical lignins.  In this figure the differences among the different types of lignin can be seen.(A) softwood Kraft lignin is highly condensed,(B) lignosulfonates are characterized by the presence of sulfonic groups on saturated carbons and (C) organosolv lignin has a structure more similar to the one of milled wood lignin.

Figure 3: Typical hydrolysable tannins: tannic acid, vescalgin.

Figure 4: Proantocyanidin structures: R can be hydrogen, hydroxy group or methoxy group

Figure 5: Lignin and tannin phosphytilation chemistry. Labelling lignin and tannin labile H hroups is accomplished by in situ reaction. The labeled polyphenols are characterized by specific 31P NMR bands, corresponding to the different type of hydroxy groups.

Figure 6: Procedure for the 31P NMR analysis of lignin and tannins.

Figure 7: Check the presence of an excess ofTMDP: if it can be seen, the derivatization of the sample is complete. The spectra can be then processed. To do that, focuses the spectral range between 155 and 132 ppm.

Figure 8: Quantitative 31P NMR spectrum of TMDP derivatized (A) softwood kraft lignin,

Figure 9: Quantitative 31P NMR spectra of the same Organosolv lignin derivatized using TMDP (A) Prior and (B) post its oxidation. The spectra were recorded using a 300 MHz NMR spectrometer.

Table 1: Experimental parameters to record 31P NMR spectra of derivatized lignin or tannins

Table 2: 31P NMR chemical shifts for lignin phosphitylated OH-groups.

Table 3: 31P NMR chemical shift for tannins phosphitylated OH-groups.

DISCUSSION:

The described method represents the implementation and optimization of the analytical protocol aimed at the qualitative and quantitative characterization of lignins as developed by Argyropoulos37-42. In comparison to many other techniques available for lignin structural elucidation, the method has been widely accepted as being amongst the most facile, rapid, and reproducible.

The validity of the wet chemical methods (e.g., nitrobenzene, permanganate oxidations, etc.), relies on the good experimental skills of the operator effectively confining the method to limited operators. Furthermore, it is not uncommon to encounter the use of correction factors in the literature for wet chemical methods aimed to account for several drawbacks. The described 31P NMR protocol does not require advanced experimental skills making this readily applicable, user friendly, and thus widely available. Compared to other instrumental analytical methods, 31P NMR is the only technique capable of precisely detect and quantify the different hydroxy groups in lignin. For instance, FTIR can be used to identify various hydroxy groups, in a manner like 1H NMR. Both techniques, however, suffer since they cannot offer reliable quantitative data due to extensive signal overlap issues. Another widely used technique is UV-Vis spectroscopy, firstly reported by Goldschmid. The approach, however, is limited to a general overall determination of hydroxy groups since it cannot effectively differentiate among aliphatic, aromatic, and carboxylic protons47.

From an economic point of view, the only limitation of the 31P NMR technique is the price of TMDP, which is a relatively expensive reagent. It costs about 190 USD per gram; consequently, if the cost of analysis would be approximated only to the cost of TMDP, excluding those deriving from the pyridine/chloroform mixture and those of the operator time, it would amount to about 24 USD per analysis. To solve this issue, many laboratories resort to synthesizing TMDP, thus reducing the costs of the reagent. To do this, pinacol and phosphorous trichloride are reacted in the presence of triethylamine44. Technically, this reaction is relatively easy, however, care in the use of phosphorous trichloride and its work-up, including a well -controlled-vacuum distillation, is required. More details as to the synthesis of the TMDP can be supplied upon request.

Even though this protocol is amongst the best in terms of ease, reproducibility, and precision, some critical points need to be highlighted. Firstly, the sample needs to be fully soluble in the identified pyridine/chloroform mixture. This consideration is fundamental because the quantitative phosphitylation reaction of the hydroxyl groups needs to take place under completely homogeneous conditions. If only part of the sample is solubilized, the resulting analysis would be inaccurate. Secondly, the sample to be examined needs to be moisture and solvent-free since these variables will detrimentally affect the precision and overall success of the analysis. Traces of humidity will react with TMDP giving 2-hydroxy-4,4’-5,5’-tetramethyl-1,3,2-dioxaphospholane. This compound is a pale-yellow flocculating salt, insoluble in the pyridine/chloroform solvent mixture, causing inadequate NMR signal acquisition. Since only a small weight (~ 30 mg) of a sample is required, it needs to be free of volatiles for its precise weight to be accurately known before the analysis.

Sometimes, sample solvation issues can be promoted (especially for highly oxidized samples) by the addition of small amounts of a co-solvent, (i.e., dimethylformamide), aiding sample dissolution. In principle, every solvent which does not interact with TMDP can be used to help sample dissolution. The election of a co-solvent cannot include co-solvents containing labile hydroxy or amino groups since they react with the reagent, causing misleading final spectra. Notably, dimethylsulfoxide also reacts with TMDP precluding its use as a co-solvent. Pyridine-based-ionic liquids, such as 1-allyl-3-butylpyridinium chloride, can be used when solubility issues arise, however, the ionic liquid should be once again dry48. To dissolve lignosulfonates (a lignin type characterized by a high sulfonation degree), a pre-treatment involving the conversion of neutralized groups into their acidic form was demonstrated to be useful. Lignosulfonates can be conveniently converted into their acidic forms using acidic exchange resins in aqueous media. The resulting lignosulfonic acids are isolated from the solution by their adsorption on specific resins (e.g. XAD-7) and desorption in ethanol. Evaporation of the ethanolic solutions over reduced pressure at 40 °C allows the isolation of lignosulfonic acids. These lignins can be then characterized by 31P NMR because they are soluble in the pyridine/chloroform mixture proposed by the protocol.

Prolonged vacuum drying at mild temperatures effectively reduces the amount of moisture and other volatiles in each sample. Notably, small quantities of water do not affect the final spectrum, because the amount of TMDP used for the analysis is in excess. In addition, in some cases, a small amount of 2-hydroxy-4,4’-5,5’-tetramethyl-1,3,2-dioxaphospholane may results from the humidity present in the NMR tube or the sample vial. In these cases, stirring is sufficient to completely dissolve the amount of the formed precipitate. If a high amount of 2-hydroxy-4,4’-5,5’-tetramethyl-1,3,2-dioxaphospholane is formed, it is suggested to repeat the sample preparation, improving the drying treatment. For instance, before use, all glassware can be briefly heated with a heat gun.

The spectral range used to record the spectrum is wide if compared to the region of interest for the signal regarding the different hydroxyl groups. However, this amplitude is compulsory to understand if the sample derivatization occurred successfully. The confirmation of complete sample derivatization is given by the presence of a strong signal around 174 ppm. This sharp peak is due to the unreacted TMDP and its presence ensures that the reagent was present in excess, and therefore, all hydroxyl groups have been derivatized. If this peak is absent, two are the most probable causes. (1) The amount of TMDP used is not sufficient to perform the complete derivatization of the sample, or (2) a high amount of water is present in the sample. In the first case, the use of a higher amount of TMDP would likely ensure the sample’s complete derivatization and the signal at 174 ppm will appear. In the second case, the sample should be dried more extensively. Once an excess of TMDP is ensured, peak integration can be performed. Before this operation, the chemical shift range must be reduced to a narrower window, (150 to 132 ppm), that confines the signals of interest.

The amount of sample (~ 30 mg) to be analyzed, reported in the above experimental protocol, has been selected to collect good quality spectra for a 300 MHz NMR spectrometer or more. Nevertheless, we have observed that it is possible to reduce the amount of sample if an NMR spectrometer equal to or more than 500 MHz is used. For instance, in Figure 8d, the NMR spectrum (resulting from a 700 MHz instrument) of a sample prepared with 7.2 mg of lignin is shown. Signal integration of this spectrum offers the same results as those obtained when using higher amounts of lignin. This fact amplifies the applicability of this protocol for all those research in which small amounts of products are available.

Overall, this experimental protocol can be applied to a multitude of research and development applications, when an understanding of the origin and fate of the various hydroxy groups, present in lignins and tannins, is required. In particular, when coupled with GPC and HSQC data, the resulting data offers the opportunity to further elaborate and speculate over the structure of lignin or a tannin. In many instances where chemical modifications are applied to the  hydroxy groups of lignin or a tannin, quantitative 31P NMR analyses can be extremely valuable to detect if these modifications occurred and to which degree. For instance, Figure 9 shows two NMR spectra of the same lignin prior and post its oxidation. A simple visual inspection (without signal integration) shows a reduction of both aliphatic and aromatic hydroxy groups in the oxidized samples, providing valuable qualitative information and guidance.

[Place Figure 9 here]

In conclusion, this technique has all attributes of being amongst the most important and powerful tools when inquiries dealing with polyphenolic, OH bearing lignin and tannins (and even synthetic polymers)49–51 need to be made in a variety of fields, ranging from chemistry to engineering, from biology to polymer and pharmaceutical applications.

DISCLOSURES:

Claudia Crestini and Dimitris S Argyropoulos ensure that all authors (C.C., N.P., and D.S.A.) have no conflicts of interest.

ACKNOWLEDGMENTS:

This work over the years has been supported by various financial awards that included organizations such as the Pulp and Paper Research Institute of Canada, McGill University Montreal, the Natural Sciences and Engineering Research Councilof Canada, National Science Foundation USA, United States Department of agriculture, Solvay company.

REFERENCES:

1.	Meng, X. et al. Determination of hydroxyl groups in biorefinery resources via quantitative 31 P NMR spectroscopy. Nature Protocols. 14 (9), 2627–2647 (2019).

2.	Anastas, P.T., Williamson, T.C. Green Chemistry: An Overview. Green Chemistry. 626, 1–17,      (1996).

3.	Anastas, P., Eghbali, N. Green Chemistry: Principles and Practice. Chemical Society Reviews. 39 (1), 301–312 (2010).

4.	Collins, M.N. et al. Valorization of lignin in polymer and composite systems for advanced engineering applications - A review. International Journal of Biological Macromolecules. 131, 828–849, (2019).

5.	Biorefinery: From Biomass to Chemicals and Fuels. Biorefinery. De Gruyter. (2012).

6.	Sannigrahi, P., Pu, Y., Ragauskas, A. Cellulosic biorefineries—unleashing lignin opportunities. Current Opinion in Environmental Sustainability. 2 (5), 383–393, (2010).

7.	Lange, H., Decina, S., Crestini, C. Oxidative upgrade of lignin – Recent routes reviewed. European Polymer Journal. 49 (6), 1151–1173, (2013).

8.	Glasser, W.G. Classification of Lignin According to Chemical and Molecular Structure. Lignin: Historical, Biological, and Materials Perspectives. 742, 216–238, (1999).

9.	Kirk-Othmer Concise Encyclopedia of Chemical Technology, 2 Volume Set, 5th Edition | Wiley. Wiley.com.

10.	Lewis, N.G., Sarkanen, S. Preface. Lignin and Lignan Biosynthesis. 697, 9-11 (1998).

11.	Adler, E. Lignin chemistry—past, present and future. Wood Science and Technology. 11 (3), 169–218 (1977).

12.	Ragauskas, A.J. et al. Lignin Valorization: Improving Lignin Processing in the Biorefinery. Science. 344 (6185) (2014).

13.	Crestini, C., Melone, F., Sette, M., Saladino, R. Milled Wood Lignin: A Linear Oligomer. Biomacromolecules. 12 (11), 3928–3935 (2011).

14.	Guerra, A., Gaspar, A.R., Contreras, S., Lucia, L.A., Crestini, C., Argyropoulos, D.S. On the propensity of lignin to associate: A size exclusion chromatography study with lignin derivatives isolated from different plant species. Phytochemistry. 68 (20), 2570–2583      (2007).

15.	Contreras, S., Gaspar, A.R., Guerra, A., Lucia, L.A., Argyropoulos, D.S. Propensity of Lignin to Associate: Light Scattering Photometry Study with Native Lignins. Biomacromolecules. 9 (12), 3362–3369 (2008).

16.	Gigli, M., Crestini, C. Fractionation of industrial lignins: opportunities and challenges. Green Chemistry. 22 (15), 4722–4746 (2020).

17.	Adler, Erich. Structural Elements of Lignin. Industrial & Engineering Chemistry. 49 (9), 1377–138 (1957).

18.	Bjorkman, A. Studies on finely divided wood. Part 1. Extraction of lignin with neutral solvents. Svensk Pappersit. 477–485 (1956).

19.	Bjorkman, A. Studies on finely divided wood. Part 2. Extraction of lignin-carbohydrate compelexes with neutral solvents. Svensk Pappersit. 243–251 (1957).

20.	Bjorkman, A. Studied on finely divided wood. Part 5. The effect of milling. Svensk Pappersit. 329–335 (1957).

21.	Das, A.K., Islam, Md.N., Faruk, Md.O., Ashaduzzaman, Md., Dungani, R. Review on tannins: Extraction processes, applications and possibilities. South African Journal of Botany. 135, 58–70, (2020).

22.	Laitila, J.E. Composition and evolution of oligomeric proanthocyanidin–malvidin glycoside adducts in commercial red wines. Food Chemistry. 340, 127905 (2021).

23.	Tanning Chemistry (RSC Publishing) Anthony D Covington, William R Wise.     .

24.	Tarabanko, V.E., Tarabanko, N. Catalytic Oxidation of Lignins into the Aromatic Aldehydes: General Process Trends and Development Prospects. International Journal of Molecular Sciences. 18 (11), 2421 (2017).

25.	Guerra, A., Mendonça, R., Ferraz, A., Lu, F., Ralph, J. Structural Characterization of Lignin during Pinus taeda Wood Treatment with Ceriporiopsis subvermispora. Applied and Environmental Microbiology. 70 (7), 4073–4078 (2004).

26.	Faix, O., Andersons, B., Zakis, G. Determination of Carbonyl Groups of Six Round Robin Lignins by Modified Oximation and FTIR Spectroscopy. Holzforschung. 52 (3), 268–274 (1998).

27.	Santos, R.B., Capanema, E.A., Balakshin, M.Yu., Chang, H., Jameel, H. Lignin Structural Variation in Hardwood Species. Journal of Agricultural and Food Chemistry. 60 (19), 4923–4930 (2012).

28.	Bose, S.K., Wilson, K.L., Hausch, D.L., Francis, R.C. Lignin Analysis by Permanganate Oxidation. II. Lignins in Acidic Organosolv Pulps. Holzforschung. 53 (6), 603–610 (1999).

29.	Harman‐Ware, A.E. et al. A thioacidolysis method tailored for higher-throughput quantitative analysis of lignin monomers. Biotechnology Journal. 11 (10), 1268–1273 (2016).

30.	Lupoi, J.S., Singh, S., Parthasarathi, R., Simmons, B.A., Henry, R.J. Recent innovations in analytical methods for the qualitative and quantitative assessment of lignin. Renewable and Sustainable Energy Reviews. 49, 871–906 (2015).

31.	S. Y. Lin, Carlton, W.D. Methods in Lignin Chemistry. Springer, Berlin, Heidelberg (1992).

32.	Lundquist, K. Proton (1H) NMR Spectroscopy. Methods in Lignin Chemistry. 242–249,       (1992).

33.	Robert, D. Carbon-13 Nuclear Magnetic Resonance Spectrometry. Methods in Lignin Chemistry. 250–273 (1992).

34.	Li, S., Lundquist, K. A new method for the analysis of phenolic groups in lignins by 1H NMR spectrometry. Nordic Pulp & Paper Research Journal. 9 (3), 191–195 (1994).

35.	Hallac, B.B., Pu, Y., Ragauskas, A.J. Chemical Transformations of Buddleja davidii Lignin during Ethanol Organosolv Pretreatment. Energy & Fuels. 24 (4), 2723–2732 (2010).

36.	Sette, M., Wechselberger, R., Crestini, C. Elucidation of Lignin Structure by Quantitative 2D NMR. Chemistry – A European Journal. 17 (34), 9529–9535 (2011).

37.	Sette, M., Lange, H., Crestini, C. Quantitative HSQC Analyses of Lignin: A Practcal ComparisonP. Computational and Structural Biotechnology Journal. 6 (7), e201303016       (2013).

38.	Wroblewski, A.E., Lensink, C., Markuszewski, R., Verkade, J.G. Phosphorus-31 NMR spectroscopic analysis of coal pyrolysis condensates and extracts for heteroatom functionalities possessing labile hydrogen. Energy & Fuels. 2 (6), 765–774 (1988).

39.	Archipov, Y., Argyropoulos, D.S., Bolker, H.I., Heitner, C. 31P NMR Spectroscopy in Wood Chemistry. Part I. Model Compounds. Journal of Wood Chemistry and Technology. 11 (2), 137–157 (1991).

40.	Argyropoulos, D.S., Heitner, C., Morin, F.G. P NMR Spectroscopy in Wood Chemistry - Part III. Solid State 31P NMR of Trimethyl Phosphite Derivatives of Chromophores in Mechanical Pulp. Holzforschung - International Journal of the Biology, Chemistry, Physics and Technology of Wood. 46 (3), 211–218 (2009).

41.	Argyropoulos, D.S., Heitner, C. 31P NMR Spectroscopy in Wood Chemistry. Part VI. Solid State 31P NMR of Trimethyl Phosphite Derivatives of Chromophores and Carboxylic Acids Present in Mechanical Pulps; a Method for the Quantitative Determination of ortho-Quinones. Holzforschung. 48 (s1), 112–116 (1994).

42.	Argyropoulos, D.S., Bolker, H.I., Heitner, C., Archipov, Y. 31 P NMR Spectroscopy in Wood Chemistry Part V. Qualitative Analysis of Lignin Functional Groups. Journal of Wood Chemistry and Technology. 13 (2), 187–212 (1993).

43.	Argyropoulos, D.S., Bolker, H.I., Heitner, C., Archipov, Y. 31P NMR Spectroscopy in Wood Chemistry. Part IV. Lignin Models: Spin Lattice Relaxation Times and Solvent Effects in 31P NMR. Holzforschung. 47 (1), 50–56 (1993).

44.	Granata, A., Argyropoulos, D.S. 2-Chloro-4,4,5,5-tetramethyl-1,3,2-dioxaphospholane, a Reagent for the Accurate Determination of the Uncondensed and Condensed Phenolic Moieties in Lignins. Journal of Agricultural and Food Chemistry. 43 (6), 1538–1544 (1995).

45.	Duval, A., Vilaplana, F., Crestini, C., Lawoko, M. Solvent screening for the fractionation of industrial kraft lignin. Holzforschung. 70 (1), 11–20 (2016).

46.	Ben, H., Iii, J.R.F. In-depth investigation on quantitative characterization of pyrolysis oil by 31P NMR. RSC Advances. 6 (21), 17567–17573 (2016).

47.	Goldschmid, Otto. Determination of Phenolic Hydroxyl Content of Lignin Preparations by Ultraviolet Spectrophotometry. Analytical Chemistry. 26 (9), 1421–1423 (1954).

48.	Ben, H. et al. Characterization of Whole Biomasses in Pyridine Based Ionic Liquid at Low Temperature by 31P NMR: An Approach to Quantitatively Measure Hydroxyl Groups in Biomass As Their Original Structures. Frontiers in Energy Research. 6, (2018).

49.	Debuissy, T., Pollet, E., Avérous, L. Synthesis of potentially biobased copolyesters based on adipic acid and butanediols: Kinetic study between 1,4- and 2,3-butanediol and their influence on crystallization and thermal properties. Polymer. 99, 204–213 (2016).

50.	Debuissy, T., Pollet, E., Avérous, L. Synthesis and characterization of biobased poly(butylene succinate-ran-butylene adipate). Analysis of the composition-dependent physicochemical properties. European Polymer Journal. 87, 84–98 (2017).

51.	Chan, K.P., Argyropoulos, D.S., White, D.M., Yeager, G.W., Hay, A.S. Facile Quantitative Analysis of Hydroxyl End Groups of Poly(2,6-dimethyl-1,4-phenylene oxide)s by 31P NMR Spectroscopy. Macromolecules. 27 (22), 6371–6375 (1994).