Screenshot Summary

62880_screenshot_1

    - Run the tube with Non-Stained (NS) cells and optimize the forward (FSC) and side scatter (SSC) voltage settings 00:02-00:13

    - Draw a gate on cells 00:14-00:31

    - Draw a gate on singlets 00:32-00:40

    - Adjust the voltages for each fluorophore and draw relative gates 00:41-01:44

    - Run mono-stained sample for DAPI 02:18-02:47

    - Run mono-stained sample for fluorogenic probe 03:05-03:27

    - Run mono-stained sample for GFP 03:42-04:03

62880_screenshot_2

    - On the positive control (TBHP) sample, turn on the laser and correspondent filter in order to visualize the fluorogenic probe 00:00-00:11

    - Define the exposure time 00:12-00:16

    - Proceed on live and adjust the laser intensity to define the maximum signal in all the z-stack 00:17-00:46

    - On the negative control (NT) sample, check that the signal intensity is lower 00:50-01:17

62880_screenshot_3

    - Open the Atlas overview to generate a stitched image 00:03-00:07

    - Define the step size to 5 µm 00:08-00:10

    - Define the Top and Bottom limits of the stack in order to image 5 slices including the middle region of the organoid 00:20-00:27

    - Go to the middle position and add locations in order to include the whole organoid area 00:28-00:46

    - Generate the montage and acquire the entire volume 00:47-00:55