# Submission ID #: 62947

# Scriptwriter Name: Domnic Colvin

# Supervisor Name: Shehnaz Lokhandwala

# Project Page Link: http://www.jove.com/files_upload.php?src=19207118

# Title: Single-Molecule Imaging of EWS-FLI1 Condensates Assembling on DNA

# Authors and Affiliations:

Linyu Zuo1,*, Jiawei Ding1,*, Zhi Qi1

1Center for Quantitative Biology, Peking-Tsinghua Center for Life Sciences, Academy for Advanced Interdisciplinary Studies, Peking University

# * These authors contributed equally

# Corresponding Authors:

Zhi Qi					(zhiqi7@pku.edu.cn)

# Email Addresses for All Authors:

zhiqi7@pku.edu.cn

zuolinyu1994@pku.edu.cn

djw20@stu.pku.edu.cn

Author Questionnaire

1. Microscopy: Does your protocol require the use of a dissecting or stereomicroscope for performing a complex dissection, microinjection technique, or something similar? No

2. Software: Does the part of your protocol being filmed include step-by-step descriptions of software usage? Yes

3. Filming location: Will the filming need to take place in multiple locations? No

Current Protocol Length

Number of Steps:  18

Number of Shots:  39

Introduction

- Introductory Interview Statements

REQUIRED:

  - Linyu Zuo: We provide a single-molecule assay to observe the phase separation of transcription factors on DNA.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

  - Linyu Zuo: This technique allows real-time visualization of the dynamic process of transcription factors assembling as a liquid droplet on DNA [1].

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: 4.2.

OPTIONAL:

  - Linyu Zuo: EWS-FLI1 is one of the most frequently involved fusion genes in Ewing sarcoma tumorigenesis; our method could identify the relationship between DNA motif number and EWS-FLI1 condensate formation, which is associated with aberrant gene transcription in tumor cells and may be helpful for the prognosis [1].

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: 3.3.1., 4.3.

  - Linyu Zuo: This method could be applied to study any transcription condensates or complexes in vitro, like p53 and MED1 [1]. Videographer: This statement is optional. If you don’t have time, skip it.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

Protocol

- Preparation of a DNA Curtain Flowcell

  - To prepare a flowcell with zig-zag nanofabricated barriers for a DNA Curtain experiment, connect the input and output tubes in the correct direction [1]. Then, wash the flowcell with 2.5 milliliters of lipid buffer using two 3-milliliter syringes, ensuring there is no bubble in the flowcell [2]. Videographer: This step is important!

    - WIDE: Establishing shot of talent connecting the input and output tubes.

    - Talent washing the flowcell with lipid buffer.

  - Remove the syringe from the output [1] and inject 1 milliliter of the liposome solution three times with a 5-minute incubation between each injection [2-TXT]. Videographer: This step is important!

    - Talent removing the syringe from the output tube.

    - Talent injecting the liposome solution. TXT: 40 μL liposome stock + 960 μL of lipid buffer.

  - For healing, rewash the flowcell with 2.5 milliliters of lipid buffer slowly [1] and incubate at room temperature for 30 minutes [2].

    - Talent washing the flowcell with lipid buffer.

    - Shot of flowcell incubating.

  - During the incubation, prepare the BSA buffer as mentioned in the text manuscript [1-TXT]. Then, after 30 minutes of healing, wash the flowcell with 2.5 milliliters of BSA buffer from the output side [2].

    - Shot of prepared BSA solution. TXT: BSA: Bovine serum albumin

    - Talent washing the flowcell with BSA buffer.

  - Next, inject 800 microliters of streptavidin buffer into the flowcell from the input side in two steps with a 10-minute incubation after each step [1-TXT]. Then, wash the flowcell with 2.5 milliliters of BSA buffer to remove all free streptavidin molecules [2]

    - Talent injecting the streptavidin buffer. TXT: 10 μL of 1 mg/mL streptavidin stock; 790 μL of BSA buffer

    - Talent washing flowcell with BSA buffer.

  - Next, dilute the Biotin-Lambda DNA with cloned motifs using BSA buffer [1-TXT], then inject it 4 times slowly at 5-minute intervals [2].

    - Talent diluting DNA with motifs. TXT: 2.5 μL of DNA + 998 μL of BSA buffer

    - Talent injecting the diluted DNA.

  - During the injection, turn on the microscope and the scientific Complementary Metal Oxide Semiconductor system [1]. Then, wash the tubing with 10 milliliters of double distilled water [2] and rinse the prism and the tubing connector with water, 2% liquid cuvette cleaner, and 99% ethanol [3].

    - Talent turning on the microscope and the sCMOS system.

    - Talent washing the tubing with water.

    - Talent rinsing the prism and tubing connector.

  - Next, draw up at least 20 milliliters of the imaging buffer into a 30-milliliter syringe [1-TXT]. Set up the flowcell on the microscope stage [2] and connect it to the microfluidic system [3]. Videographer: This step is important!

    - Talent drawing up the buffer in the syringe. TXT: See text for imaging buffer preparation

    - Talent setting up the flowcell on the microscope stage.

    - Talent connecting the flowcell to the microfluidic system.

  - Using a flow rate of 0.03 milliliters per minute, flush the DNA molecules to the barrier for 10 minutes [1]. Then, switch off the microfluidic system [2] and incubate for 30 minutes with the flow stopped, allowing the DNA to diffuse laterally [3].

    - Talent setting the flow rate to 0.03 mL/min.

    - Talent switching off the microfluidic system.

    - Shot of the flowcell incubating.

- Imaging of EWS-FLI1 Condensation Formation on DNA Curtains

  - To image the EWS-FLI1 (E-W-S-F-L-I-one) condensation formation on DNA curtains, open the imaging software [1] and find and mark the positions of the 3 zig-zag patterns under bright-field [2].

    - WIDE: Talent at the computer, opening software, monitor visible in frame.

    - SCREEN: 3-1-2.mp4. 00:07-00:15, then LAB MEDIA: 3-1-2.png

  - Then, turn on the flow at 0.2 milliliters per minute to stain the DNA with the double-stranded DNA dye for 10 minutes [1].

    - Talent setting the flow rate to 0.2 mL/min.

  - Next, dilute the mCherry-EWS-FLI1 protein with the imaging buffer at a concentration of 100 nanomoles in 100 microliters [1], then load the protein sample through the valve with a 100-microliter glass syringe [2] and change the flow rate to 0.4 milliliters per minute [3].

    - Talent diluting mCherry-EWS-FLl1 with imaging buffer.

    - Talent loading the protein sample with a glass syringe.

    - Talent setting the flow rate to 0.4 mL/min.

  - After turning on the 488-nanometer laser [1], pre-scan each region to check the DNA distribution state [2-TXT] and select the region in which the DNA molecules distribute evenly [3].

    - Talent turning on the 488-nm laser.

    - SCREEN: 3-4-2&3.mp4: 00:15-00:30. TXT: Laser power 10% for 488 nm and 20% for 561 nm laser.

    - SCREEN: 3-4-2&3.mp4: 01:25-01:33.

  - Then, set the laser power to 10% for the 488 nm laser and 20% for the 561 nm laser [1], and using the power meter, measure the real laser power near the prism [2-TXT].

    - LAB MEDIA: 3-5-1.png.

    - Talent measuring the real laser power. TXT: 4.5 mW for the 488 nm and 16.0 mW for the 561 nm laser

  - Start acquiring images at 2-second intervals with both 488 and 561-nanometer lasers simultaneously [1].

    - LAB MEDIA: 3-6-1.png, then 3-6-1.mp4 03:15-03:30

  - Then, change the valve from the manual mode to injection mode to let the imaging buffer flush the protein sample into the flowcell after 60 seconds [1].

    - Talent changing the valve mode.

  - To remove the free EWS-FLI1, keep washing the flowcell with the imaging buffer for 5 minutes with only the 561-nanometer laser switched on [1], then stop the flow [2] and incubate at 37 degrees Celsius for 10 minutes [3].

    - Talent washing of flowcell with imaging buffer.

    - Talent stopping the flow.

    - Talent incubating the flowcell at 37 °C.

  - After 10 minutes, turn on the flow at 0.4 milliliters per minute to let the DNA extend [1] and acquire images at 2-second intervals between different frames to obtain high-throughput data of EWS-FLI1 condensate formation [2]. Videographer: This step is important!

    - Talent turning on the flow rate, setting at 0.4 mL/min.

    - SCREEN: 3-9-2.mp4. 00:25-00:40

Results

- EWS-FLI1 Condensate Formation on Lambda DNA and Binding Events of the Detached Domain of mCherry-EWS-FLI1 on 25× GGAA Domain Repeats

  - EWS-FLI1 molecules were visualized by detecting the mCherry-labeled EWS-FLI1 signals obtained with a 561-nanometer laser [1].

    - LAB MEDIA: Figure 1C and 1E.

  - The in vitro formation of EWS-FLI1 condensates at the site of the 25 GGAA (G-G-A-A) repeats in the DNA substrate could be directly visualized [1].

    - LAB MEDIA: Figure 1B-1E.

  - The specificity of the mCherry-EWS-FLI1 used in DNA Curtains was confirmed by an electrophoretic mobility shift assay using a DNA template with and without the 25 GGAA repeats separately [1].

    - LAB MEDIA: Figure 2A.

  - When the EWS-FLI1 concentration was titrated from 20 to 500 nanomoles [1], the EWS-FLI1 intensity increased dramatically [2], whereas the change in the FLI1DBD (F-L-I-one-D-B-D) intensity was negligible when the proteins were saturated to cover the GGAA repeats, suggesting that EWS-FLI1 formed condensates on DNA [3].

    - LAB MEDIA: Figure 2B-2E. Video Editor: Please emphasize the x-axis in Figure 2E

    - LAB MEDIA: Figure 2B-2E. Video Editor: Please emphasize the cyan dotted curve in Figure 2E

    - LAB MEDIA: Figure 2B-2E. Video Editor: Please emphasize the black dotted curve in Figure 2E

Conclusion

- Conclusion Interview Statements

  - Linyu Zuo: The injection should be very slow and soft so that the liposome and DNA distribute evenly on the flowcell [1].

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: 2.2.2, 2.6.2

  - Linyu Zuo: It is important to perform EMSA to confirm that the purified transcription factor specifically binds to its target sequence in vitro [1].

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

  - Linyu Zuo: This technique allows people to explore whether the condensate will facilitate the target search of transcription factors and its effect on transcription [1].

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: 3.9.2.