In-gel fluorescence staining of FlAsH-tagged recombinant proteins from TX-TL

Prepare the following:

4x SDS-PAGE loading buffer

200 mM Tris-HCl (pH 6.8)

8% (w/v) sodium dodecyl sulfate

0.4% (w/v) bromophenol blue

40% (v/v) glycerol

FlAsH-EDT2 dilution

30 mM FlAsH-EDT2 in 50 L of DMSO

Dilute to 1 mM (10x) in ddH2O – slightly insoluble, vortex before pipetting in next steps

0.5 M TCEP—5 mL

Sample preparation

- For a 33 L cell-free sample, add 1 mL of ice-cold acetone.

- Store in freezer for 1 h, or freeze on dry ice.

- Spin down at 16,000 × g, 10 min, RT.

- Remove supernatant.

- Wash pellet with 1 mL of ice-cold 70% acetone

- Spin down at 16,000 × g, 10 min, RT.

- Remove supernatant and dry pellet.

- Add the following:

- 22 L ddH2O

- 10 L of 4x SDS-PAGE loading buffer

- 4 L of 0.5 M TCEP

- Boil for 5 min.

- Add 4 L of 1 mM FlAsH-EDT.

- Incubate for 15–30 min at RT.

- Centrifuge for 10 min, 16,000 × g.

- Add 5 L of supernatant onto SDS-PAGE following the manufacturer’s guidelines. Avoid overloading, as this leads to smearing of the bands. Optional: run a fluorescent protein ladder (Biorad - Precision Plus Protein™ WesternC™ Blotting Standards) or purified FlAsH-tagged standard protein in parallel.

- Visualize unstained gels on an ultraviolet light-box or by using a camera and Xe-light source with appropriate filters for FlAsH fluorescence (excitation at 480 nm, with emission at 535 nm).

- Optional: Gels can be stained with routine Coomasie Blue staining or used for western blotting analysis.