| Module | Parameters |
| --- | --- |
| Raw data methods >Mass detection | MS level: 2 Mass detector: Wavelet transform >Noise level: 5.0E1 >Scale level: 1 >Wavelet window size: 100 % |
| Raw data methods >Feature detection >ADAP Chromatogram builder | MS level: 2 Min group size in # of scans: 5 Group intensity threshold: 5.0E1 Min highest intensity: 5.0E1 m/z tolerance: 0.2 m/z |
| Feature list methods >Feature detection >Chromatogram deconvolution | Algorithm: Wavelets (ADAP) >S/N threshold: 1 >S/N estimator: Intensity window SN >min feature height: 10 >coefficient/area threshold: 0.5 >Peak duration range: 0.00 – 100.00 >RT wavelet range: 0.00 – 0.50 m/z center calculation: MEDIAN |
| Feature list methods >Isotopes >Isotopic peaks grouper | m/z tolerance: 0.015 m/z Retention time tolerance: 0.5 absolute (min) Maximum charge: 5 Representative isotope: Most intense |
| Feature list methods >Export/Import >Export to CSV file | Field separator: “,” Export common elements: >Export row m/z >Export row retention time Export data file elements: >Peak height >Peak charge |