- SCREENSHOT SUMMARY

63495R2_Screenshot 1_

    - (Press the Run Experiment module on the automated software to choose the desired protocol for the experiment) 00.00-00.30sec

63495R2_Screenshot 2_

    - (Choose your desired protocol [10 Points with Regeneration] for the experiment and enter/Fill the Experimental details to proceed) 00.00-00.1min

63495R2_Screenshot 3_

    - (Press the Begin the Experiment button displayed on the software and follow the instructions shown by the automated software) 00.00-00.30sec

63495R2_Screenshot 4_

    - (A pop-up window showing the first calibration step of the experiment. Apply 50 µL of 1XPBS. Then Press the Continue button and wait for 5 minutes until the calibration step is finished) 00.00-00.5min

63495R2_Screenshot 5_

    - (A pop-up window showing the second Association step of the experiment. Apply 50 µL of lowest analyte concentration. Then Press the Continue button and wait for 5 minutes until the association step is finished) 00.00-00.5min

63495R2_Screenshot 6_

    - (A pop-up window showing the third Dissociation step of the experiment. Apply 50 µL of 1XPBS. Then Press the Continue button and wait for 5 minutes until the dissociation step is finished) 00.00-00.5min

63495R2_Screenshot 7_

    - (A pop-up window showing the fourth Regeneration step of the experiment. Apply 50 µL of 1XPBS. Then Press the Continue button and wait for 30 seconds until the dissociation step is finished) 00.00-00.30sec

63495R2_Screenshot 8_

    - (A pop-up window showing the Fifth Wash step of the experiment. Apply 50 µL of 1XPBS, aspirate the solution from the chip, and repeat this 5X. Then Press the Continue button and wait for 30 seconds until the wash step is finished) 00.00-00.30sec

63495R2_Screenshot 9_

    - (Press the Analysis button in the automated analysis software) 00.00-00.30sec

63495R2_Screenshot 10_

    - (The display window showing all the experimental points to check with the analyte concentrations used for the experiment) 00.00-00.1min

63495R2_Screenshot 11_

    - (Press the Run Analysis button to generate the KD value automatically) 00.00-00.30sec

63495R2_Screenshot 12_

    - (The software generates a Hill fit plot by plotting the analyte concentrations against the corresponding I-Responses from which KD value, is generated) 00.00-00.30sec