# Submission ID #:  63529

# Scriptwriter Name: Nuzhat Ahsan

# Project Page Link: https://www.jove.com/account/file-uploader?src=19376393

# Title:  Tension Gauge Tether Probes for Quantifying Growth Factor Mediated Integrin Mechanics and Adhesion

# Authors and Affiliations:

# Tejeshwar C. Rao1, Alexa L. Mattheyses1*

# 1Department of Cell, Developmental, and Integrative Biology, University of Alabama at Birmingham

# Corresponding Author:

Alexa L. Mattheyses	mattheyses@uab.edu

# Email Addresses for All Authors:

tjrao@uab.edu

mattheyses@uab.edu

## Author Questionnaire

1. Microscopy: Does your protocol require the use of a dissecting or stereomicroscope for performing a complex dissection, microinjection technique, or something similar? Yes

If Yes, can you record movies/images using your own microscope camera?

Yes , Nikon Eclipse Ti2 inverted microscope

2. Software: Does the part of your protocol being filmed include step-by-step descriptions of software usage? No

3. Filming location: Will the filming need to take place in multiple locations?   Yes

If Yes, how far apart are the locations? All rooms are located on the same floor (level), less than a minute apart.

Current Protocol Length

Number of Steps:  22

Number of Shots:  55

# Introduction

- Introductory Interview Statements

Videographer: Obtain headshots for all authors.

REQUIRED: Why is your protocol significant? OR What key questions can this method help answer?

  - Alexa Mattheyses: This protocol is significant due to the simplicity of TGT sensor platform employed to study EGFR-integrin crosstalk in isolation from other membrane receptors and its influence on cell mechanical forces.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

  - Tejeshwar Rao: Though outlined to investigate EGF-dependent regulation of cell mechanics, the protocol is adaptable for different ligands, cell types, stimulation paradigms and can be coupled with TGTs of varying tension thresholds.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

OPTIONAL:

  - Tejeshwar Rao: Surface synthesis may seem intimidating at first. The key is to be prepared and organized to precisely execute the intricate steps by employing a checklist for reagent preparation and calculations.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: 2.11.3. or 2.11.4.

# Protocol

- Surface Preparation

  - On Day 1, place up to 8, 25-millimeter glass coverslips into a polytetrafluoroethylene rack [1]. Place the rack in a 50-milliliter borosilicate beaker containing 40 milliliters of 200 proof ethanol [2-TXT].

    - Talent placing glass coverslips on rack

    - Talent placing the rack in the beaker. TXT: Cover the beaker with paraffin film

  - Sonicate at an operating frequency of 35 kiloHertz for 10 to 15 minutes at room temperature [1]. Fill a 50-milliliter beaker with 40 milliliters of Piranha solution freshly prepared by mixing sulfuric acid and hydrogen peroxide in a 3 to 1 ratio in a pyrex beaker and stirring with a glass pipette [2].

    - Talent sonicating at RT

    - Talent filling a beaker with Piranha solution

  - Transfer the coverslip rack into the beaker [1] and incubate for 30 minutes at room temperature in the fume hood to etch the coverslip surface [2]. After etching, transfer the coverslip rack to a beaker with ultrapure water with tweezers [3-TXT].

    - Talent transferring the coverslip rack into the beaker

    - Talent placing the beaker in the fume hood

    - Talent transferring the coverslip rack to a beaker with water with tweezers. TXT: Repeat this step 6 X at 15 s intervals

  - Visually inspect the coverslips to ensure the surfaces look clean with no patterns or dust particles on the glass surface [1-TXT]. Transfer the coverslip rack to a beaker with 200 proof ethanol and wash twice for 15 seconds to equilibrate surfaces to organic solvent [2].

    - Talent inspecting the coverslips. TXT: Repeat cleaning if patterns or dust are detected

    - Talent washing the coverslip rack in a beaker with 200 proof ethanol

  - Transfer the coverslip rack into 200 proof ethanol solution with 3 percent APTES for 1 hour at room temperature to silanize the coverslips [1-TXT]. Immerse the rack in a clean beaker with 200 proof ethanol solution [2-TXT].

    - Talent transferring the coverslip rack into 200 proof ethanol with 3 percent APTES. TXT: Cover the beaker with paraffin film

    - Talent immersing the rack in a clean beaker with 200 proof ethanol solution. TXT: Repeat this wash 3X for 15 s each

  - Dry the coverslips using nitrogen gas with low exit pressure [1]. Place the coverslips into a 10-centimeter polystyrene dish with a piece of paraffin film laid flat inside it [2-TXT].

    - Talent drying the coverslips using nitrogen gas

    - Talent placing the coverslips into a 10 cm dish with parafilm laid flat inside it. TXT: Ensure the coverslips are dry and separated

  - Add 100 microliters of 2 milligrams per milliliter NHS-biotin solution in DMSO to four coverslips placed on paraffin film [1]. Set a “sandwich” with the other four coverslips on top with two coverslips facing toward each other with the functionalization solution in between [2-TXT].

    - Talent adding NHS-biotin solution to coverslips

    - Talent setting a sandwich with coverslips. TXT: Incubate the dish overnight at 4 °C

  - On Day 2, remove the dish from 4 degrees Celsius and separate the sandwiched coverslips. Avoid scratching or damaging the functionalized surface while separating the sandwich. [1]. Then, orient the slips in the rack with the coated surface facing each other [2-TXT]. Dry with nitrogen gas [3].

    - Talent separating the sandwiched coverslips.

    - Talent orienting the slips in the rack with the coated surface facing each other. TXT: Wash with 200 proof ethanol solution 3X for 15 s each

    - Talent drying the coverslips with nitrogen gas

  - Place the coverslips in a new dish with a paraffin film inside it [1-TXT]. Add 800 microliters of 0.1 percent bovine serum albumin in 1x PBS to each coverslip [2].

    - Talent placing the coverslips in a new dish. TXT: Wash the coverslips 3X with 1 mL, 1x PBS

    - Talent adding BSA to coverslips

  - Incubate the coverslips at room temperature for 30 minutes to passivate the surface and block non-specific binding of subsequent functionalization reagents [1-TXT]. To functionalize the coverslips, add 800 microliters of 1 microgram per milliliter of streptavidin in 1x PBS at room temperature for 45 to 60 minutes [2].

    - Shot of the coverslips at room temperature. TXT: Wash the coverslips 3 X with 1 mL of 1x PBS

    - Talent adding streptavidin at room temperature

  - Assemble the TGT probes in a PCR tube using a thermocycler [1-TXT]. After incubation, wash the coverslips three times with 1x PBS [2]. Add 100 microliters of the pre-assembled TGT probes to four coverslips [3] and make sandwiches using the remaining 4 coverslips with the functionalized side facing the probes [4-TXT]. Videographer: This step is important!

    - Talent assembling the TGT probes in a thermocycler TXT: Refer to the manuscript for program details.

    - Talent washing the coverslips

    - Talent adding pre-assembled TGT probes to four of the coverslips

    - Talent making sandwiches with the remaining 4 coverslips. TXT: Eight surfaces require four tubes of hybridized TGT probes

  - Cover with aluminum foil [1-TXT]. After incubation, separate the sandwiches [2] and wash the coverslips with 1x PBS three times [3]. Carefully assemble the coverslips into precleaned imaging chambers [4-TXT]. Videographer: This step is important!

    - Talent covering the coverslips with aluminum foil. TXT: Incubate for 1 h at RT

    - Talent separating the sandwiches

    - Talent washing the coverslips

    - Talent assembling the coverslips into precleaned imaging chambers. TXT: Add 1x PBS to keep the surfaces hydrated

- Cell Preparation and Staining

  - To investigate the effect of the epidermal growth factor stimulation on Cos-7 mechanics, adhesion, and cell spreading, trypsinize Cos-7 cells with 0.05 percent Trypsin-EDTA for 2 minutes [1]. Neutralize the trypsin by washing with HBSS [2] and centrifuging at 800 x g for 5 minutes [3-TXT].

    - Talent trypsinizing Cos-7 cells with 0.05% Trypsin EDTA

    - Talent washing the cells with HBSS

    - Talent centrifuging the cells. TXT: Repeat the neutralization step once more

  - Plate cells at a density of forty thousand cells on the assembled TGT surfaces in DMEM supplemented with 50 nanograms per milliliter EGF or DMEM without EGF [1-TXT]. Following incubation, wash the cells three times with 1x PBS [2]. Videographer: This step is important!

    - Talent plating the cells. TXT: Incubate for 60 min at 37◦C with 5 % CO2

    - Talent washing the cells with PBS

  - Fix with 2 milliliters of 4 percent paraformaldehyde for 12 minutes at room temperature [1-TXT]. Wash the coverslips five times with 1x PBS at 5-minute intervals at room temperature. [2]. Optionally, incubate the coverslips with 50 millimolar ammonium chloride in 1x PBS for 30 minutes at 37 degrees Celsius [3-TXT]

    - Talent fixing the cells with PFA TXT: Aspirate the fixative

    - Talent washing the coverslips

    - Talent incubating the coverslips in ammonium chloride. TXT: wash 3X with 1X PBS, 5 min interval

  - Add Buffer A [1-TXT] and incubate for 30 minutes at 37 degrees Celsius to block and permeabilize the cells [2-TXT]. Place the imaging chambers with coverslips in a humidity container [3].

    - Talent adding Buffer A. TXT: Refer to the manuscript for buffer recipe

    - Talent placing the coverslips in the incubator. TXT: Wash 3X with 1x PBS at 5-min intervals

    - Talent placing the imaging chambers in the humidity container

  - Dilute the primary anti-paxillin antibody at 1 to 250 dilutions in blocking buffer [1]. Incubate with 200 microliters of primary antibody solution per coverslip for 2 hours at 37 degrees Celsius [2-TXT]. Videographer: This step is important!

    - Talent diluting the primary antibody

    - Talent incubating the coverslips in primary antibody TXT: Wash the coverslips 5X with 1x PBS at 5-min intervals, return them to the humidity container

  - Label cells simultaneously with a mixture of dye conjugated goat anti-rabbit secondary antibody at 1 to 800 dilution and dye conjugated phalloidin at 1 to 400 dilutions in 200 of blocking buffer per coverslip [1-TXT]. Wash the surfaces five times with 1x PBS at 5-minute intervals [2-TXT].

    - Talent labeling the coverslips TXT: Incubate at 37◦C for 60 min

    - Talent washing the coverslips. TXT: Store at 4◦C until ready for imaging

- Image Acquisition

  - To begin, add oil to the objective [1-TXT], clean the coverslip bottom of the sample chamber, and place the sample on the stage [2]. Focus on a cell and engage the perfect focus [3-TXT].

    - Talent adding oil to the objective TXT: Refer to the manuscript for specifications and settings

    - Talent cleaning the coverslip bottom of the sample chamber

    - Talent focusing on the cell. TXT: RICM imaging mode, epifluorescence excitation, GFP filter cube with the emission filter removed

  - Align the RICM by closing and centering the epi-illumination aperture diaphragm [1-TXT]. Focus the 488-nanometer laser to a small spot on the ceiling of the room and increase the angle of incidence until past the critical angle while monitoring fluorescence on the camera in live mode [2].

    - Talent aligning the RICM. TXT: TIRF mode, laser excitation, and a quad-pass TIRF filter cube

    - WIDE: Talent focusing the laser to a small spot on the ceiling of the room

  - Observe a sharp reduction in background fluorescence and a single in-focus plane when the critical angle is surpassed [1]. Identify cells for imaging using the “live” mode of the camera using RICM [2].

    - SCOPE/SCREEN: to be uploaded by the authors: Reduction in the background and single in-focus plane

    - SCOPE/SCREEN: to be uploaded by the authors: cells on the LIVE mode of the camera

  - Acquire the RICM and TIRF images of actin at 488-nanometer extinction, integrin tension at 561-nanometer extinction, and paxillin at 647-nanometer extinction [1]. Obtain images sequentially using an exposure time of 200 milliseconds [2-TXT]. Change coverslips, focus, and repeat [3]. Videographer: This step is important!

    - SCOPE/SCREEN: to be uploaded by the authors: RICM and TIRF images of actin, integrin, and paxillin

    - SCOPE/SCREEN: to be uploaded by the authors: Sequential imaging with an exposure time of 200 ms TXT: Repeat for at least 30 cells

    - SCOPE/SCREEN: to be uploaded by the authors: Talent changing coverslips

# Results

- Results: Effect of Ligand-activated EGFR on Integrin-mediated Cell Mechanics and Adhesion Formation in Cos-7 Cells

  - Cos-7 cells were plated on this TGT surface with [1] or without EGF stimulation [2] to study the impact of EGFR activation with ligand stimulation on integrin mechanics [3]. If an integrin binds the ligand and applies a force larger than the T total of the probe, the DNA duplex will separate, leading to fluorescence [4].

    - LAB MEDIA: Figure 5 Video Editor emphasize +EGF in Figure 5A (inset) and 5B

    - LAB MEDIA: Figure 5 Video Editor emphasize -EGF in Figure 5A (inset) and 5B

    - LAB MEDIA: Figure 5

    - LAB MEDIA: Figure 5 Video Editor emphasize Figure 5A (inset) yellow circular spot

  - Any TGT probe that a mechanical force has not ruptured will remain non-fluorescent [1]. The cells were incubated with or without EGF on the TGT surfaces for 60 minutes, fixed, and immuno-stained to display the focal adhesion distribution [2] and the organization of the cytoskeleton [3].

    - LAB MEDIA: Figure 5 Video Editor emphasize Figure 5A (inset) yellow circular spot

    - LAB MEDIA: Figure 5 Video Editor emphasize Figure 5B, paxillin

    - LAB MEDIA: Figure 5 Video Editor emphasize Figure 5B, F-actin

  - In the RICM image, Cos-7 cell spreading on the 56 piconewton TGT surface was significantly enhanced with EGF stimulation [1] compared to without stimulation [2]. Stimulation with EGF resulted in a more circular morphology, representing Cos-7 cells spreading and growing [3].

    - LAB MEDIA: Figure 5 Video Editor emphasize Figure 5C, Green spots

    - LAB MEDIA: Figure 5 Video Editor emphasize Figure 5C, Blue spots

    - LAB MEDIA: Figure 5 Video Editor emphasize Figure 5D

  - The fluorescence from open probes is also higher with EGF stimulation, as observed in the tension fluorescence image [1]. The integrated intensity of open probes, proportional to the number of open probes, was much higher with EGF stimulation than without stimulation [2].

    - LAB MEDIA: Figure 5 Video Editor emphasize Figure 5B, Tension

    - LAB MEDIA: Figure 5 Video Editor emphasize Figure 5E

# Conclusion

- Conclusion Interview Statements

  - Alexa Mattheyses: Always remember the orientation of the coverslips keeping the functionalized surface facing upwards. When separating the sandwich be gentle so the functionalized surface is not scratched or damaged.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: 2.8.1. or 2.8.2.

  - Dr. Tejeshwar Rao: Following surface synthesis, one can probe for cytoplasmic proteins regulating cell adhesion or mechanotransduction. This allows identification of downstream signaling molecules involved in orchestrating cell mechanics at the plasma membrane.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: 3.1.1., 3.2.1. or 3.2.2.