63579 Screenshot summary

- 63759_6.1.2

  - Set the camera acquisition parameters. Set the binning to 2 by 2, exposure to 20 milliseconds, rolling shutter ON, readout rate to 540-Megahertz, dynamic range to 12-bit & gain 1, and sensor mode to overlap.

- 63759_6.3.1

  - Select ROI containing the innervated skeletal tissues tissue.

- 63579_6.4.2

  - Click Live view, center and focus on the desired ROI.

- 63579_6.5.1

  - Download and upload custom code to Arduino board. TXT: https://github.com/ofvila/NMJ-function-analysis

- 63579_6.5.2

  - Download and upload custom microscope macro code.

- 63579_6.5.3

  - Set output movie name. Include tissue group and day of culture in naming.

- 63579_6.6.1

  - Save the macro with X and Y coordinates and number of frames (1700).

- 63579_6.6.2

  - Running macro to acquire fast time-lapse.

- 63579_7.1.2-7.3.3

  - 7.1.2 (Download the file. TXT: GitHub folder) 00:00-00:10

  - 7.2.1 (Open recursive-OS-analysis script | Have all videos in one folder) 00:10-00:30

  - 7.2.2 (Copy address of folder containing videos into the code) 00:30-00:44

  - 7.2.3 (Set pre-analysis parameters | numSteps = 30 |startFrequency = 0.5 | endFrequency = 3 | baselineTime = 0.1 | peakDistance = 20 | minPeakProminence = 5 | minMinProminence = 1 | minMinWidth = 1) 00:44-01:11

  - 7.2.4 (Run recursive-OS-analysis) 01:11-04:37 (Video section with progress bar can be sped up)

  - 7.3.1 (Adjusting baseline time | Click No when prompted if analysis is correct | Click 1 to change baseline | Select a time point when the curve is flat) 04:38-06:44

  - 7.3.2 (Adjusting peak threshold | Click No when prompted if analysis is correct | Click 2 to change peak threshold | Select an amplitude above which there are only peaks) 06:45-07:47

  - 7.3.3 (Adjusting prominence and width | Click No when prompted if analysis is correct | Click 3 to change prominence and width | Decrease each value individually until the desired peak is detected) 07:48-09:04

- 63579_7.3.4

  - Show final scores for each example tissue

- 63579_7.4.1

  - Run recursive OS movie script.

- 63579_7.4.2

  - Open generated video file of example tissue