- 63846_411-431

  - 4.1.1 (Open Fiji, open a couple of z-stack images [wt/atg8d/ear1d/hfl1d]. Drag the z-position in each stack to the mid-section) 00:11-00:48

(Click Image | Adjust | Brightness/Contrast | Reset) 00:49-01:05

  - 4.2.1 (Manual adjusting the scroll bars to change two parameters, Minimum and Maximum values, to discern the structure under investigation clearly) 01:06-01:17

  - 4.3.1 (Click Set | Propagate to all other Open Images) 01:18-01:26

- 63846_ 441-452

- 4.4.1 (Open a multichannel image [YSL102]. Click Image | Color | Channels Tool | select the Greyscale) 00:12-00:31

- 4.5.1 (Click Image | Adjust | Brightness/Contrast | pick the proper Brightness/Contrast for that channel) 00:32-01:13

- 4.5.2 (Go back to the Channels window and change to Composite mode) 01:14-01:25

- 63846_ 461

- 4.6.1 (Open a couple of z-stack images of different colonies [Colony1/Colony2/Colony3/Colony4]) 00:12-00:31

(Drag the z-position in each stack to the mid-section) 00:32-00:49

(The same Brightness/Contrast settings are applied across all opened images) 00:50-01:35

(Multi-copy integrations (See Colony 3 & 4) look substantially brighter than single-copy ones (colony 1 & 2)) 01:36-01:47

- 63846_ 481-482

- 4.8.1 (Select the area of interest. Click Image | Duplicate | make a copy of the image of interest, and give it a name of choice) 00:11-00:30

- 4.8.2 (Click Image | Type | 8-bit | convert the duplicate copy from 16-bit to 8-bit and save it as needed) 00:31-00:45