# Submission ID #:  64148

# Scriptwriter Name: Sweety Arora

# Project Page Link: https://www.jove.com/account/file-uploader?src=19565188

# Title: Preparation and Structural Evaluation of Epithelial Cell Monolayers in a Physiologically Sized Microfluidic Culture Device

# Authors and Affiliations:

Eshan B. Damle, Eiichiro Yamaguchi, Joshua E. Yao, Donald P. Gaver III

Department of Biomedical Engineering, School of Science and Engineering, Tulane University

# Corresponding Authors:

Eshan B. Damle		(edamle@tulane.edu)

# Email Addresses for All Authors:

Eiichiro Yamaguchi		(guchi@tulane.edu)

Joshua E. Yao			(jyao2@tulane.edu)

Donald P. Gaver III		(dpg@tulane.edu)

Eshan B. Damle		(edamle@tulane.edu)

## Author Questionnaire

1. Microscopy: Does your protocol require the use of a dissecting or stereomicroscope for performing a complex dissection, microinjection technique, or something similar?  Yes

If Yes, can you record movies/images using your own microscope camera? No

If your microscope does not have a camera port, the scope kit will be attached to one of the eyepieces and you will have to perform the procedure using one eye.

NOTE: This does not necessarily involve microscopy; scope shots are added to improve the video.

Videographer: Please use the SCOPE kit to film 2 scope shots, 3.2.2 and 5.4.3.

2. Software: Does the part of your protocol being filmed include step-by-step descriptions of software usage?  Yes

If Yes, we will need you to record using screen recording software.

We recommend using the screen capture program OBS. JoVE’s tutorial for using OBS Studio is provided at this link: https://www.jove.com/v/5848/screen-capture-instructions-for-authors?status=a7854k

As these files are necessary for finalizing your script, please upload all screen captured video files to your project page as soon as possible.

Author’s NOTE: Will upload screen captures after filming occurs.

3. Filming location: Will the filming need to take place in multiple locations?   Yes

If Yes, how far apart are the locations? Within the same research complex, in separate buildings – approximately a 5 minute walk from one location to the other (from the lab to the confocal microscope room).

Current Protocol Length

Number of Steps:  23

Number of Shots:  52

# Introduction

- Introductory Interview Statements

Videographer: Obtain headshots for all authors.

REQUIRED:

  - Eshan Damle: This protocol serves to lower a barrier to entry for cell culture experimentation and enables researchers to evaluate and characterize adherent cell monolayers cultured in dynamic microfluidic environments.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: LAB MEDIA: Figure 6

  - Eshan Damle: The primary advantage of this technique is that it enables both qualitative and quantitative evaluation of cell monolayer characteristics to serve as a basis for further monolayer-dependent experimentation.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: LAB MEDIA: Figure 9

OPTIONAL:

  - Donald Gaver: This method enables the recapitulation of an alveolar epithelium in-vitro, permitting exploration of the dynamic responses during Acute Respiratory Distress Syndrome (ARDS) as well as Ventilator-Induced Lung Injury (VILI).

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: 3.2.1 and 3.2.2

Videographer: Obtain headshots for all authors.

# Protocol

- Microfluidic Channel Fabrication and Pre-Treatment

  - To begin, obtain a single-channel flow array [1-TXT].

    - WIDE: Talent bringing/placing single channel flow array on the workbench. TXT: Note: this video protocol shows one channel for demonstration purposes, but more may be prepared in tandem

  - Clean the cover glass surface in an ultrasonic bath [1]. Immerse the cover glass in Poly-D-Lysine at room temperature for 5 minutes [2]. Then, dry it at 60 degrees Celsius for 30 minutes [3].

    - Talent removing a cover glass from an ultrasonic bath.

    - Talent immersing the coverglass in a container with Poly-D-Lysine.

    - Talent placing the treated cover glass at 60 °C.

  - Next, affix double-sided adhesives and mylar spacers, laser cut as described in the manuscript, to the cover glass [1] and ensure that the channel cut-outs are precisely aligned [2].

    - Talent affixing double-sided adhesives and mylar spacers (sandwiched) to the cover glass.

    - Talent showing/pointing at the aligned channel cut-outs.

  - Affix a rectangular cover glass to the bottom-most adhesive strip [1]. Use the peeled adhesive papers to cover the portions of adhesive still exposed [2]. After assembly is complete, apply firm and equal pressure to the construction [3]. Videographer: Important step!

    - Talent affixing cover glass to the bottom-most adhesive strip.

    - Talent applying adhesive papers and cutting to length.

    - Talent applying pressure to the construction.

  - Using a syringe filled with deionized water, rinse the channel [1-TXT]. Sterilize the channel enclosure in an ultraviolet sterilizer for 30 minutes [2].

    - Talent placing the syringe filled with deionized water over the channel and pushing the plunger TXT: Check for leaks simultaneously

    - Talent placing the channel enclosure in UV sterilizer.

  - Using a sterile technique, treat the channel with human fibronectin solution prepared in PBS [1] and incubate for at least 30 minutes at 37 degrees Celsius [2].

    - Talent adding human fibronectin solution to the inside of the channel enclosure using a micropipette.

    - Talent placing the channel in the incubator.

- Cell Culture in the Microfluidic Channel

  - In the sterile laminar flow hood, prepare NCI-H441 (N-C-I H-four-four-one) cell suspension using RPMI 1640 (R-P-M-I sixteen-forty) medium with 10 percent FBS [1-TXT]

    - Talent adding RPMI 1640 to a vial containing NCI-H441 cells and pipetting up and down. TXT: Cell concentration: 2.5 x 106 cells/mL

  - Use 0.25 milliliters of this cell suspension to fill the channel and a portion of the ports [1-TXT]. Using brightfield microscopy, verify that cells have been distributed evenly within the channels [2]. Add the media reservoir and stopcock to the channel [3]. Videographer: Important step!

    - Talent filling the channel and portion of ports using a micropipette. TXT: Generated surface density: 150,000 cells/cm2

    - Talent examining cells in the microscope.

    - Talent adding the media reservoir and a stopcock to the channel.

  - Begin culturing the channel at 37 degrees Celsius with 5 percent carbon dioxide [1]. Use a programmable syringe pump to draw spent media out from the channel [2] and fresh media into the channel from a sterile media reservoir attached to the channel inlet [3-TXT]. Videographer: Important step!

    - Talent placing the channel in the incubator.

    - Shot of programmable syringe pump set up (Including channel and reservoir).

    - CU: Shot of fresh media entering the channel. TXT: Flow program: wait 10 min, start at 0.2 μL/min, ramp up to 10 μL/min across 4 h, and maintain thereafter

- Cell Fixation

  - In a chemical fume hood, dilute a 4% formaldehyde solution into DPBS to create a 1% solution (D-P-B-S) [1] and store in an appropriately labeled tube. Prepare the 2% formaldehyde solution in a similar fashion [2].

    - Talent adding 1 ml formaldehyde in 3 mL of DPBS.

    - Shot of tubes labeled as 1% formaldehyde and 2% formaldehyde placed in a rack on workbench

  - Transfer the formaldehyde solutions to appropriately labeled 5-milliliter syringes [1]. Draw up 20 milliliters of DPBS into a separate 20-milliliter syringe [2]. This DPBS will be used for the washing steps.

    - Talent transferring formaldehyde solution to a labeled syringe and placing it next to an already filled labeled syringe.

    - Show DPBS-containing syringe already in the hood.

  - Next, remove the microfluidic channel from the culture apparatus [1] and secure it in the chemical fume hood [2].

    - Talent removing the microfluidic channel from the culture apparatus.

    - Talent placing the microfluidic channel into the chemical fume hood.

  - To assemble the fixation and staining apparatus, attach a segment of transfer tubing to the side port of a three-way stopcock via a male Luer lock to hose barb adapter [1], then connect the stopcock to the inlet port of the flow array [2].

    - Talent attaching segment of transfer tubing to the side port of a three-way stopcock using adapter.

    - Talent connecting stopcock to the inlet port of the flow array.

  - Attach another segment of transfer tubing to the outlet port of the existing stopcock using the same type of hose barb adapter [1]. Finally, secure the free ends of both transfer tubes, now designated as waste lines, into a chemical and biohazard-appropriate waste container [2].

    - Talent attaching transfer tubing to the outlet port of the flow array.

    - Talent placing the free ends of the transfer tubes into 50 mL conical tube.

  - Ensure the stopcock is blocking off the flow array inlet port [1] and flush the waste line with DPBS [2]. Then, turn the stopcock to block off the waste line [3] and slowly wash the cells with 2 milliliters of DPBS [4-TXT]. Videographer: Important step!

    - Talent ensuring that the stopcock is blocking off the flow array inlet port.

    - CU: Shot of waste lines flushed with DPBS.

    - Talent turning the stopcock to block off the waste line.

    - ECU/CU: Shot of plunger of a labeled syringe containing DPBS is pushed and DPBS entering the channel TXT: Flush when introducing new solutions to avoid bubble formation Videographer: If possible, please obtain multiple reusable takes as it is going to be used in next step.

  - Slowly push 2 milliliters of 1 percent fixative through the channel, let it sit for 5 minutes, and repeat with 2 percent fixative [1]. Then wash cells three times with fresh DPBS as demonstrated previously [2].

    - ECU/CU: Shot of plunger of a labeled syringe containing 1% fixative is pushed and fixative entering the channel

    - Talent placing DPBS labeled syringe over the channel and pushing the plunger or reuse 4.6.4; Video editor, please note: this sequence was filmed in one take but accidentally out of order. The filmed order was 1%, DPBS, 2%; whereas it should have been 1%, 2%, DPBS. There is adequate interval in between to cut and rearrange these discrete actions.

- Cell Staining and Mounting

  - Prepare the 0.1% saponin solution as described in the manuscript [1]. Draw up additional DPBS into a 20-milliliter syringe for use in the washing steps [2].

    - Talent adding 4 mg saponin into labeled 15 mL tube.

    - Talent drawing DPBS into a 20 mL syringe.

  - Add a filamentous-actin-staining phalloidin reagent and a nucleus-staining Hoechst (hookst (with the “oo” sounding close to “uh” or “oe”)) reagent to the 0.1 percent saponin solution [1]. Cover the prepared staining and permeabilizing solution with aluminum foil to protect it from light [2]. Transfer the solution to a foil-covered syringe [3]. Videographer: Important step!

    - Talent covering the tube with aluminum foil.

    - Talent dropping filamentous-actin-staining phalloidin reagent into saponin solution-containing vial.

    - Talent drawing up saponin solution into syringe covered with aluminum foil.

  - Flush the waste line with a small amount of permeabilizing and staining solution [1]. Then introduce 2 milliliters of the solution to the microfluidic channel [2]. Cover the channel with aluminum foil to block light [3]. After 30 minutes, flush out the permeabilizing solution with 2 milliliters of DPBS twice for 5 minutes each [4].

    - Talent flushing the line with a permeabilizing solution.

    - Talent adding a permeabilizing solution to the microfluidic channel.

    - Talent covering the channel with aluminum foil and placing it aside.

    - Talent adding DPBS and the permeabilizing solution is being flushed out.

  - Using a micropipette, introduce a minimal amount of a soft-set antifade mountant to the microfluidic channel, ensuring to cover the bottom surface completely [1-TXT]. Then seal the ends of the channel [2]. Using a brightfield microscope, verify the integrity of the cell layer [3].

    - Talent introducing antifade mountant to port of microfluidic channel using micropipette. TXT: Avoid bubbles in the imaging area

    - Talent sealing the ends of the channel.

    - SCOPE: Shot of properly integrated cell layer.

- Cell Imaging

  - Test imaging locations by taking reference scans and Z-stacks until desired image parameters and conditions have been met [1].

    - Talent creating reference scans and Z stacks are being obtained. Video editor: several on-camera recordings were taken, sequentially dialing in higher objectives and showing more detail of the cell layers on-screen. Please feel free to select the best one from among these – ideally showing a zoomed out view including the talent before then going into the screen captures in the following steps.

Authors: Please create screen capture videos of the shots labeled as SCREEN, create a screenshot summary, and upload the files to your project page as soon as possible: https://www.jove.com/account/file-uploader?src=19313258

  - Using the flow array base plate as a reference, construct Z-stacks at various locations along the length of the channel [1].

    - SCREEN: Z-stacks being constructed for the last location.

  - Finally, analyze cross-sectional data, depth map data, and any other relevant characteristics to evaluate cell layer properties [1].

    - SCREEN: Scrolling through the obtained data.

# Results

- Results: Culture Duration Effects on Monolayer Production

  - The successful use of the technique is demonstrated in the microfluidic dynamic-culture environment through image acquisition at least 1 centimeter away from the inlet and the outlet [1].

    - LAB MEDIA: Figure 4

  - In a representative culture-duration experiment, successful monolayer production was observed when the cells were cultured for 24 hours [1]. However, when cultured for 48 hours, undesirable multilayer formation was seen [2].

    - LAB MEDIA: Figure 4 Video editor: Please emphasize figures A and B

    - LAB MEDIA: Figure 4 Video editor: Please emphasize figures C and D

  - The data collected from each of the five microfluidic channel imaging locations [1] also reveals the relationship between increased culture duration and increased cross-sectional area, suggesting uneven layer formation or overgrowth occurs with 48 hours of culture [2].

    - LAB MEDIA: Figure 2

    - LAB MEDIA: Figure 5 Video editor: Please emphasize green bars of ‘48 h’

  - The depth map of a central location within the microfluidic channel cultured for 24 hours was obtained, which is useful for the qualitative evaluation of layer characteristics [1].

    - LAB MEDIA: Figure 7

# Conclusion

- Conclusion Interview Statements

  - Eshan Damle: This protocol's three most crucial aspects are proper channel construction, accurate culture and media flow conditions, and careful use of the fixation and staining apparatus.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: 2.6 (for VO “proper channel construction”), 3.3 (For VO “accurate culture and media flow conditions”), and 5.3 (For VO “careful use of the fixation and staining apparatus”)

  - Donald Gaver: Following this procedure, other methods may be used in parallel to validate the experimental viability of the produced cell monolayers, such as Electric Cell-substrate Impedance Sensing (ECIS) or Trans-Epithelial Electrical Resistance (TEER).

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.