64919_Screenshot_Step3_4.mp4

- 3.4.2. Workspace is created by clicking the New Workspace menu under the File tab. 00:00-00:07

- 3.4.3 Scan for Amplifier button is clicked, and parameters such as sampling rates and low and high cutoff frequencies for recording are set. 00:08-00:19

- 3.4.4. Setting the scanner parameters for the fMRI. 00:20-00:36

64919_Screenshot_Step4.mp4

- 4.1.2. Transformations, then Special Signal Processing and MR Correction tab is clicked. Then, Use Markers is selected, a trigger signal is chosen, and Next is clicked. 00:00-00:05

- 4.2.1. Interleaved and Based on Time options are selected. Then the time information of the trigger signal is entered, and Next is clicked. 00:06-00:09

- 4.2.2. Enable Baseline Correction for Average is ticked/checked, and Compute Baseline Over the Whole Artifact is unticked/unchecked. Inserting Start and End values and the Use Sliding Average Calculation option is selected. Inserting Total Number of Intervals for Sliding Average, checking/ticking Common Use of All Channels for bad Intervals and Correction, and Next being clicked. 00:10-00:16

- 4.3.1. Use All Channels for Correction being selected and Next is clicked. 00:17-00:18

- 4.4.1. Do Downsampling is checked, and 500 Hz for New Frequency is selected. 00:19-00:20

- 4.4.2. Apply Lowpass Filter is checked, Use FIR Filter is selected, 70 Hz for Cutoff Frequency is entered, and Next is clicked. 00:21-00:24

- 4.4.3. Store Corrected Data option is selected, and Finish is clicked. 00:25-00:29

- 4.5.1. Transformation, then Data Filtering and IIR Filters tab being clicked. 00:30-00:32

- 4.5.2. Enabled under Low Cutoff is checked, cutoff Frequency of 0.5 Hz is entered, filter Order is chosen to 2, and OK is clicked. 00:33-00:36

- 4.6.1. Transformations, then Special Signal Processing and CB Correction tab is clicked. Then, Use Peak Detection is selected, and Semiautomatic Mode is checked. 00:37-00:42

- 4.6.2. Under the Search Pulse Template section, Start[s] and Length[s] values are entered. Then, Mark Found Template is checked/ticked. 00:43-00:45

- 4.6.3. Parameters for Pulse Rate and Correlation and Amplitude are entered. Then R for Mark Pulses with R-Peak Markers is entered, and Next is clicked.  00:46-00:49

- 4.7.1. Use Whole Data to Compute the Time Delay is checked/ticked, and Total Number of Pulse Intervals Used for Average is entered. 00:50-00:51

- 4.7.2. Under Correct Following Channels, EEG channels are being moved to the right column except for the ECG channel and Next being clicked. 00:52-00:55

- 4.7.3. Selecting Store Corrected Data option and clicking Finish. 00:56-00:57

- 4.8.1. Sliding over the time axis from the Navigation Bar and shot of base template ECG highlighted with a blue shaded box. 00:58-01:02

- 4.9.1. In the CB Correction – Interactive Mode window, marked ECG being checked. Then, detected peak locations being adjusted by moving the red vertical lines. Finally, Finish is clicked. 01:03-01:08

- 4.10.1. Transformation, then Data Filtering and IIR Filters tab is clicked. Then Notch Enabled is checked, Frequency is selected, and OK is clicked. 01:09-01:14