65122_Screenshot_1

3.4.3.1 (Open Pump Control software) 00:00:00-00:00:14

3.4.3.2 (Click OK to initialize the Fluidic Unit) 00:00:15-00:00:30

3.4.3.3 (Set the perfusion set according to the one you are using and select without any slide) 00:00:31-00:00:40

3.4.3.4 (Click Tutorial > Load demo setups > Remove air Bubbles > OK) 00:00:41-00:00:51

3.4.3.5 (at the window Air bubble remove stings, click OK) 00:00:52-00:00:55

3.4.3.6 (Wait until comes out the message: New recalibration factor has been found and set!) 00:00:55-00:01:07

3.4.3.7 (Click OK) 00:01:08-00:01:13

65122_Screenshot_2

3.6.1.1. (In the Pump control software, adjust the parameters of Fluidic Unit Setup. Select the type of slide, set the viscosity of the medium) 00:00:00-00:00:12

3.6.1.2. (Click Apply new settings) 00:00:13-00:00:16

3.6.1.3. (Adjust the Flow Parameters. Set the value of shear stress and the shear rate will be completed automatically. Check the box infinite (to maintain the cycle infinite – you will turn the device off manually) and check the box to select the type of flow (unidirectional)) 00:00:17-00:00:30

3.6.1.4. (Click in the Play button to start the experiment) 00:00:30-00:00:32

3.6.1.5 (Wait until the Equilibration dialog window pops out) 00:00:32-00:00:34

3.6.1.3 (Click Start experiment) 00:00:35-00:00:49

65122_Screenshot_3

4.5.2.1 (Open Flexcell Control software – FX-5000) 00:00:00-00:00:02

4.5.2.2 (To create a regimen, click Regimen > Create New. The Regimens window will pop up) 00:00:03-00:00:06

4.5.2.3 (Select an Existing User add write the Regimen Name. To add the first step, put the min and max values of stretch, frequency and time. Click Add Step) 00:00:07-00:00:37

4.5.2.4 (The step 2 of the regimen will Back To step 1 repeating 3 times. Click Add Step, Save Regimen and Close) 00:00:38-00:00:55

4.5.2.5 (Select On to turn the vacuum system on) 00:00:56-00:01:00

4.5.2.6 (Click Regimen > Assign > Choose Platform (BFlx Loading Station 25mm). Choose the User and the Regimen) 00:01:01-00:01:10

4.5.2.7 (A window to confirm the regimen of stretch will pop up. Press Yes. Click on Start to initiate the stretch regimen. Certify if the minimum and maximum values of elongation are been reached) 00:01:11-00:01:30