5.5.1 65172_screenshot_1

- 5.5.1.1 Create a Dot-plot graph, click on the polygonal symbol to select the subpopulation of interest and excluding the debris areas using FSC-A vs SSC-A gatering. 00:00-00:08

- 5.5.1.3 Create a new dot-plot, click on polygonal symbol and excluding the duplets and multiplets cells using SSC-H vs SSC-A followed FSC-A vs FSC-H gating. 00:08-00:38

5.5.2 65172_screenshot_2

- 5.5.2.1 Create a dot-plot, right click and select Y610m-CHERRY-A detector. Create another dot-plot, right click and select R660-APC-A detector. 00:00-00:12

- 5.5.2.3 Click on the quadrant symbol and set the threshold for auto-fluorescence cells for each detector. 00:00:12-00:19

- 5.5.2.4 Select samples labelled with CD-105 for positive signals and CD-31 for negative signals. 00:19-00:25