Submission ID #: 65471

Scriptwriter Name: Sritama Bose

Project Page Link: https://review.jove.com/account/file-uploader?src=19968703

# Title: Mapping the Cellular Distribution of an Optogenetic Protein Using a Light-Stimulation Grid

Landing Page Title (not for video use): A Light-Stimulation Grid Approach to Study Cellular Optogenetic Protein

Authors and Affiliations:

Alejandro Pizzoni1, Nyla Naim1,2, Xuefeng Zhang1, Daniel L Altschuler1

1Department of Pharmacology and Chemical Biology, University of Pittsburgh School of Medicine

2Addgene

# Corresponding Authors:

Daniel L. Altschuler 		(altschul@pitt.edu)

# Email Addresses for All Authors:

Alejandro Pizzoni		(acp78@pitt.edu)

Nyla Naim			(nyla.naim@gmail.com)

Xuefeng Zhang		(xuefeng_zhng@yahoo.com)

Daniel L. Altschuler 		(altschul@pitt.edu)

## Author Questionnaire

1. Microscopy: Does your protocol require the use of a dissecting or stereomicroscope for performing a complex dissection, microinjection technique, or something similar?  NO

2. Software: Does the part of your protocol being filmed include step-by-step descriptions of software usage?  YES

If Yes, we will need you to record using screen recording software.

We recommend using the screen capture program OBS. JoVE’s tutorial for using OBS Studio is provided at this link: https://www.jove.com/v/5848/screen-capture-instructions-for-authors?status=a7854k

Authors: Please upload all screen captured video files to your project page as soon as possible. https://review.jove.com/account/file-uploader?src=19968703

3. Proposed filming date: To help JoVE process and publish your video in a timely manner, please indicate the proposed date that your group will film here: 12/10/23

When you are ready to submit your video files, please contact our Content Assistant, Utkarsh Khare

Current Protocol Length

Number of Steps:  16

Number of Shots:  42

# Interviews

- Video 1: Author Spotlight: Studying Spatial and Temporal Regulation of Cyclic AMP Using bPAC-NanoLuciferase as an Optogenetic Tool

What is the scope of your research? What questions are you trying to answer?

  - Nyla Naim: Our lab investigates spatial and temporal regulation of cyclic AMP to understand what conditions trigger specific downstream events. To effectively mimic subcellular activation and inhibition of cAMP signaling, we characterized the distribution of an optogenetic tool called bPAC-nanoLuciferase.

INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

What research gap are you addressing with your protocol?

  - Nyla Naim: To test the validity of current cAMP signaling models, we’ve developed tools to evaluate, mimic, and block cAMP signaling from sub-cellular compartments in living cells. This protocol can be used to stimulate and evaluate the distribution and function of optogenetic tools in a systematic manner.

INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: 2.17.1

What advantage does your protocol offer compared to other techniques?

Alejandro Pizzoni: Our goal was to test if focused light pulses can precisely activate optogenetic proteins in specific cell zones, avoiding impact on surrounding proteins. This method could be vital for studying proteins with widespread distributions or if the intended effect is to generate localized increases in cAMP or other signaling molecules.

INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera.

How will your findings advance research in your field?

  - Alejandro Pizzoni: In optogenetic experiments, the typical approach involves stimulating either the entire field of cells or, at times, smaller designated areas of interest. However, the selection of the area and intensity is often arbitrary. Our systematic approach enables investigators to conduct more accurate experiments, helping to mimic patterns of signaling pathways.

INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: 2.17.2, 2.16.5

# Protocol Videos

Video 2: Light Stimulation and Live Cell Imaging to Study an Optogenetic Protein

Demonstrator: Alejandro Pizzoni

Protocol

For this study, use cells co-transfected with a nuclear-targeted cylic AMP or cAMP (cylic-A-M-P) sensor and an optogenetic nuclear-targeted protein NLS-bPAC-nLuc (N-L-S-B-Pack-nanoluciferase) [1-TXT]. Culture and manipulate the cells expressing the nuclear bPAC (B-pack) in a dark environment [2]. Use a red safelight lamp to avoid exposure to wavelengths less than 500 nanometers [3-TXT].

WIDE: An establishing shot of talent preparing materials for the study in the biosafety cabinet. TXT: cAMP sensor used: H208; bPAC-nLuc: Bacterial photoactivatable adenylyl cyclase-nanoluciferase

Talent turning off the ceiling lights.

Talent switching on the red lamp to begin work. TXT: Use 13 W amber compact fluorescence bulb with the appropriate filter.

Perform the imaging experiments in a motorized two-deck microscope equipped with a multi-LED (L-E-D) light engine [1], an emission filter wheel [2], XY stage [3], an ORCA (Orca)-Fusion Digital CMOS (Cee-MOS) camera [4], and a 100x (hundred ex) magnification oil objective with 1.4-numerical aperture [5].

Talent placing the sample under the microscope.

Talent pointing at the emission filter wheel.

Talent pointing at the XY stage.

Talent pointing at the ORCA fusion Digital CMOS camera.

Talent pointing at the oil objective.

To capture images, use digital microscopy software [1]. This setup is capable of stimulating cells and performing FRET (Fret) measurements simultaneously [2-TXT] by using an independent lightpath equipped with a ZT458rdc (Z-T-4-5-8-r-d-c) dichroic filter [3].

Talent sitting in front of a computer and opening the digital microscopy software.

Shot of the entire system showing the microscope, the computer, and any other connected equipment. TXT: TXT: FRET: Förster resonance energy transfer; Imaging: Olympus IX83 two-deck Microscope + Spectra X; Optogenetics: LDI-7 + UGA‑42 Geo

Shot of the two lightpaths that converge into the microscope.

After turning on the UGA‑42 (U-G-A-forty-two) Geosystem and the lasers [1], switch on the microscope to set it up appropriately for acquiring data with the H208 (H-2-0-8) FRET sensor [2]. Then, sequentially open the SysCon (sys-con) software and the imaging software that controls the microscope [3].

Talent turning on the UGA-42 Geosystem.

- Authors: The shots labeled as SCREEN are those that would be visualized on a computer screen. Because filming the screen directly does not produce good-quality images, we recommend recording the shots using screen recording software.

Talent switching on the microscope.

SCREEN: The SysCon software and the imaging software are opened.

To calibrate the system before each use, navigate to the Camera window and select the Calibration Tab [1]. Set the laser wavelength compatible with the optogenetic protein - choose 445 nanometers to stimulate bPAC-nanoluciferase [2]. Select the Camera tab and click Start Acquisition to begin the acquisition from the imagining software [3].

SCREEN: The Camera window is clicked, and then the Calibration Tab option is selected.

SCREEN: The laser wavelength is set to 445 nm in the Calibration Tab.

SCREEN: The Camera tab is clicked, followed by the Start Acquisition option being clicked.

Next, in the Image Viewer window, visualize either a live or a previously acquired fluorescence image of the cells from the imaging software [1] and use it as a guide to position the stimulation objects [2].

SCREEN: The Image Viewer window is clicked, and an appropriate desired fluorescence image is selected for visualization.

SCREEN: The selected image being used to guide the position of the stimulation object.

Prior to starting the actual experiment, use the Click and Fire mode to quickly evaluate the responsiveness of an individual cell [1]. After that, in the Sequence-Manager window, select add to create a new sequence [2].

SCREEN: The Click and Fire is clicked, and the responsiveness value of an individual cell appears.

SCREEN: The Sequence-Manager window is clicked, and then the option <add> is clicked.

By properly calibrating the system, in the Image Viewer window, draw objects of various sizes and shapes corresponding precisely to the stimulation patterns projected onto the cells [1]. This ensures that the patterns accurately match the intended stimulation objects drawn [2].

SCREEN: The Image Viewer window is selected, and objects with various shapes and sizes are drawn that match the stimulation patterns projected onto the cells.

SCREEN: The screen showing one of the stimulation objects drawn accurately matches the pattern on the cell.

Then, in the Image Viewer window, select the Round icon in the toolbar on the left to draw circular stimulation objects [1]. Create multiple small identical circles and distribute them evenly, ensuring homogeneous coverage of the entire cytoplasm of the cells to be stimulated [2].

SCREEN: The Image Viewer window is open, the Round icon in the toolbar on the left is clicked, and a circle is drawn.

SCREEN: Several small identical circles are drawn, and they are distributed evenly to cover the entire cytoplasm of the cells.

Right-click on each circle to verify the homogeneity of the circular stimulation objects. Use the window that appears to examine their radius, position, etc. [1]. Set the radius between 1 and 5, depending on the objective used and the size of the cell [2].

SCREEN: One of the drawn circles is right-clicked, and a window appears.

SCREEN: In the window that appears, the radius is set to a value between 1-5.

In the View subwindow, click the Snap to Grid button to maintain even spacing and alignment of the circles [1]. Additionally, set the properties of the grid with the Set Grid button [2].

SCREEN: The View window is clicked, and then the Snap to Grid option is clicked.

SCREEN: The Set Grid option is clicked, and the window/tab to set properties appears.

Ensure that the grid or matrix of evenly spaced circular stimulation objects completely covers the cell, and its size is determined by the size of the cell [1]. Include some circles positioned outside the cell as a control for null stimulation that should not trigger any cellular events [2]

SCREEN: A cell is examined, and the cursor is moved over it to check if the grid of evenly spaced circular stimulation objects completely covers it.

SCREEN: A few circular stimulation objects are drawn outside the cell.

Each object created in the Image Viewer window will appear in the Timeline window [1]. In the Object Timing subwindow, configure the temporal properties of each stimulation object, including start time, duration, delay, etc. [2].

SCREEN: The Timeline window is opened, and the objects created in the Image Viewer window appear in the Timeline window.

SCREEN: The Object Timing subwindow is clicked, and the properties like start time, duration, and delay are set.

In the Lightsource subwindow, configure the object wavelengths and intensities - in this case, ensure identical wavelength, duration, and intensity for each object [1]. To prevent superimposing the effect of various stimulations, set different start times for each object [2] and vary the delay between them depending on the expected outcome [3].

SCREEN: The Lightsource subwindow is clicked, and the object wavelengths and intensities are set for an object.

SCREEN: The start time of the object is set.

SCREEN: The delay time is set.

Next, in the Timeline window, alter the sequence in which each stimulation object will be activated if required [1]. Configure stimulations in a regular or random pattern, depending on the experiment [2].

SCREEN: The Timeline window is opened, and the sequence in which the objects will be activated is altered.

SCREEN: The stimulations are being configured appropriately.

After uploading the sequence to the system [1], configure the number of sequence cycles or runs the system will perform [2]. Place the regions of interest in the imaging software at the desired positions [3]. Finally, press Play at the UGA-42 window [4] to start acquiring fluorescence images with the microscope [5].

SCREEN: The sequence is uploaded to the system.

SCREEN: The number of sequence cycles or runs is set.

SCREEN: The position of the regions of interest is set properly in the imaging software.

SCREEN: The UGA-42 window is open, the option Play is clicked, and image acquisition begins.

Shot of the microscope stage with a close-up of the chamber where the cells are being imaged, showing light emission due to imaging (e.g., red) and optogenetic stimulation (e.g., blue).

Observe the intensity change of the cAMP sensor due to stimulation [1] [2].

SCREEN: The intensity change of a cAMP sensor due to stimulation, preferably including the actual image recorded by the microscope and the measured trace as it rises in the imaging software – SLIDEBOOK.

SCREEN: The actual stimulation beam recorded by the microscope to illustrate the exact stimulation points, especially their discrete locations and transient nature.

- Authors: Please record all shots labeled as SCREEN using screen recording software and upload the video files to your project page as soon as possible. https://review.jove.com/account/file-uploader?src=19968703

Representative Results

The detected yellow fluorescence from H208 revealed that only stimulations directed to the cell nucleus were able to generate measurable cAMP elevations [1].

LAB MEDIA: Figure 1

This confirmed that bPAC-nanoluciferase was expressed exclusively in the nuclear compartment of HC-1 (H-cee-one) cells [1]. Transient cAMP increase triggered by bPAC nanoluciferase stimulation was observed at each stimulation spot [2].

LAB MEDIA: Figure 1. Video Editor: Emphasize the yellow region in the center of the image at the left.

LAB MEDIA: Figure 1. Video Editor: Emphasize the black curves under labels ‘B - F’ in the right portion of the image. The other parts of the image should remain visible.