# Submission ID #: 66257

# Scriptwriter Name: Poornima G

# Project Page Link: https://review.jove.com/account/file-uploader?src=20208433

# Title: Multi-modal Pulmonary Imaging: Using Complementary Information from CT and Hyperpolarized 129Xe MRI to Evaluate Lung Structure-Function

Landing Page Title (not for video use): Hyperpolarized 129Xe MRI and CT Registration to Evaluate Lung Structure-Function

☐   The Landing Page Title is correct. (Character limit with spaces: 80)

# Authors and Affiliations:

# Rachel L Eddy1,2*, George H Xu3, Jonathon A Leipsic1,4, Janice M Leung1,2, Don D Sin1,2, Chase S Hall5, Roger C Tam3,4

# 1UBC Centre for Heart Lung Innovation, St. Paul’s Hospital

# 2Division of Respiratory Medicine, Department of Medicine, University of British Columbia

# 3School of Biomedical Engineering, University of British Columbia

# 4Department of Radiology, University of British Columbia

# 5Division of Pulmonary and Critical Care Medicine, University of Kansas Medical Center

☒   All author names and affiliations are correct (city/state/country information not included in video title page).

# Corresponding Authors:

# Rachel L Eddy			Rachel.Eddy@hli.ubc.ca

# Email Addresses for All Authors:

George H Xu			george.hy.xu@gmail.com

Jonathon A Leipsic		jonathon.leipsic@ubc.ca

Janice M Leung		Janice.Leung@hli.ubc.ca

Don D Sin			Don.Sin@hli.ubc.ca

Chase S Hall			chall2@kumc.edu

Roger C Tam			roger.tam@ubc.ca

# Rachel L Eddy			Rachel.Eddy@hli.ubc.ca

## Author Questionnaire

1. We have marked your project as author-provided footage, meaning you film the video yourself and provide JoVE with the footage to edit. JoVE will not send the videographer. Please confirm that this is correct.

√ Correct

☐ Incorrect

2. Microscopy: Does your protocol require the use of a dissecting or stereomicroscope for performing a complex dissection, microinjection technique, or something similar?  No

3. Software: Does the part of your protocol being filmed include step-by-step descriptions of software usage?  Yes

If Yes, we will need you to record using screen recording software.

We recommend using the screen capture program OBS. JoVE’s tutorial for using OBS Studio is provided at this link: https://www.jove.com/v/5848/screen-capture-instructions-for-authors?status=a7854k

Please upload all screen captured video files to your project page as soon as possible: https://review.jove.com/account/file-uploader?src=20208433

4. Proposed filming date: To help JoVE process and publish your video in a timely manner, please indicate the proposed date that your group will film here: MM/DD/YYYY

When you are ready to submit your video files, please contact our Content Manager, Utkarsh Khare.

Current Protocol Length

Number of Steps:  6

Number of Shots:  12

# Protocol Videos

- Video 1: 129Xe MRI to CT Image Registration with Open-Source Tools

Demonstrator: Rachel Eddy

Ethics Title Card

This research has been approved by the University of British Columbia Providence Health Care Research Ethics Board and participants provided written informed consent prior to imaging.

Protocol

  - To begin, open images, followed by masks in the desired image visualization software to verify the image and mask the orientation matches for all CT (C-T), H (pro-ton), and Xenon-129 (zee-non) files [1].

    - SCREEN: Open being clicked to open images, followed by masks and orientation of images and masks being reviewed with scrolling. In this case, mask files are already .nii file format by default. Showing CT only for demonstration.

  - Then, Save the image DICOMs (Dye-coms) and single-label masks as NIfTI (nif-tee) files in the same folder as the reg.py (reg-dot-pie) file [1-TXT].

    - SCREEN: Save being selected and images and masks being saved as NIfTI, *.nii in folder also containing reg.py file TXT: NIfTI: Neuroimaging Informatics Technology Initiative files (.nii)

Authors: Please create screen capture videos of the shots labeled as SCREEN, create a screenshot summary, and upload the files to your project page as soon as possible: https://review.jove.com/account/file-uploader?src=20208433

  - For CT-XeMRI (C-T-Xenon-M-R-I) registration, open the reg.py file in the desired Python computing environment setup [1]. If using a virtual environment, set the number of central processing units, number threads, and RAM (Ram) as desired or as available in the computing environment [2].

    - SCREEN: reg.py file being opened in Python computing environment setup (in this case, Microsoft Visual Studio Code for display).

    - SCREEN: For this case using a virtual computing environment, number of central processing units, number threads, and RAM will be set later in the virtual computing environment (cursor hovering over CPU count in script).

  - Next, set the desired transformation and interpolation, followed by the fixed and moving image [1]. Run reg.py in the Python computing environment [2].

    - SCREEN: Default transformation and interpolation being shown followed by fixed and the moving image.

    - SCREEN: reg.py being run in the Python computing environment.

  - To evaluate registration results, open the CT.nii (C-T-dot-N-I-I) image as the base image in the desired visualization software [1]. Then, open Ventilation_warped.nii.gz (Venti-lation-warped-dot-N-I-I-dot-G-Z) as another image and overlay it on the CT image with the desired color map [2-TXT].

    - SCREEN: CT.nii being opened in the visualization software

    - SCREEN: Ventilation_warped.nii.gz being opened as another image and being overlaid on CT image with the desired color map TXT: Alternatively, use Ventilation_label_warped.nii.gz

  - Review the overlap of the Xenon MR image with the CT image in all image planes [1-TXT], to evaluate the visual alignment of landmarks such as the carina and lung boundaries [2].

    - SCREEN: Scrolling through CT image with overlaid ventilation mask in coronal, followed by sagittal, followed by axial planes.

    - SCREEN: Pause in scrolling occasionally to show cursor being hovered over carina and lung boundaries.

Representative Results

  - Registration results showed good alignment of all lung boundaries for the healthy participant [1] and the participant with COPD (C-O-P-D) with diffuse ventilation abnormalities [2].

    - LAB MEDIA: Figure 3A

    - LAB MEDIA: Figure 3B