# Submission ID #: 66761

# Scriptwriter Name: Nilesh Kolhe

# Project Page Link: https://review.jove.com/account/file-uploader?src=20362153

# Title: Postmortem Diagnosis of Rabies in Animals by the Updated, Multiplexed LN34 Real-Time Reverse Transcription-Polymerase Chain Reaction Assay

Authors and Affiliations:

Crystal M. Gigante1, Vaughn Wicker1, Rene Edgar Condori1,2, Kimberly Wilkins1, Yu Li1

1Division of High-Consequence Pathogens and Pathology, Centers for Disease Control and Prevention

2Goldbelt Professional Services

# Corresponding Authors:

Crystal Gigante 		lzu1@cdc.gov

# Email Addresses for All Authors:

# lzu1@cdc.gov

tpk5@cdc.gov

hws5@cdc.gov

ibx4@cdc.gov

yuli@yahoo.com

## Author Questionnaire

1. Microscopy: Does your protocol require the use of a dissecting or stereomicroscope for performing a complex dissection, microinjection technique, or something similar?  No

2. Software: Does the part of your protocol being filmed include step-by-step descriptions of software usage?  No

3. Filming location: Will the filming need to take place in multiple locations?  No

Current Protocol Length

Number of Steps: 22

Number of Shots: 55

# Introduction

Videographer: Obtain headshots for all authors available at the filming location.

  - Crystal Gigante: Rabies is 100% fatal, requiring rapid, accurate diagnosis for life-saving treatment. Reliable laboratory testing is crucial for rabies control. The CDC developed a step-by-step protocol for rabies testing by quantitative reverse transcriptase PCR, or RT-PCR, using the validated pan-lyssavirus LN34 assay, enabling laboratories to improve diagnostic accuracy and support rabies prevention efforts [1].

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera.

What are the most recent developments in your field of research?

  - Crystal Gigante: For decades, rabies testing relied on the Direct Fluorescence Antibody Test (DFA/FAT). However, many international laboratories now also recommend RT-PCR for rabies diagnosis due to its superior accuracy and sensitivity [1].

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera.

What are the current experimental challenges?

  - Crystal Gigante: The DFA test for rabies requires specialized microscopes, cold-chain storage, and trained staff, but PCR and RT-PCR are now more widely used for pathogen testing. Many labs already have PCR and RT-PCR equipment and expertise, and its reagents are readily available from multiple vendors, unlike DFA reagents, which often face shortages [1].

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera.

What research gap are you addressing with your protocol?

  - Crystal Gigante: Cost is a major barrier to rabies testing in regions of the world with the most human rabies cases. CDC has developed a multiplexed LN34 assay that improves quality controls and decreases the cost per sample tested [1].

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera.

What advantage does your protocol offer compared to other techniques?

  - Crystal Gigante: RT-PCR is increasingly being used for rabies diagnostics because it is widely used for testing other pathogens, and the results are easy to interpret. But, using a validated test is extremely important. Our protocol provides step-by-step instructions, including details about safety for those performing the test, best practices, and recommended quality controls [1].

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera.

Videographer: Obtain headshots for all authors available at the filming location.

# Protocol

General NOTE: Please cut any footage showing someone’s hand outside of the biological safety cabinet while working under the biological safety cabinet.

- Sheep Brain Tissue Collection and Homogenization

Demonstrator: Lillian Orciari

NOTE: In this section, please cut any footage showing moving samples with a scalpel. Tissue should be transferred using plastic forceps.

  - To begin, wear personal protective equipment, including safety glasses, a closed front gown, and two pairs of gloves [1]. Clean and disinfect the work surface with quaternary ammonium compound or QAC (Q-A-C) disinfectant for 2 minutes [2]. Lay out a plastic-lined absorbent pad on the work surface [3].

    - WIDE: Establishing shot of talent with personal protective equipment on and walking into the room.

    - Talent spraying QAC disinfectant onto the work surface.

    - Talent placing a plastic-lined absorbent pad on the work surface.

  - Using a clean single-use scalpel, collect tissue representing a full cross-section of the brain stem and cerebellum (/ˌsɛr.əˈbɛl.əm/) [1-TXT].

    - Talent using a scalpel to collect brain stem and cerebellum tissue. TXT: Postmortem sheep brain purchased from a local butcher is used NOTE: After the video, add a still shot showing the scalpel cutting cross-section of the brainstem.

  - For homogenization and RNA (R-N-A) extraction, finely mince the required tissues using a single-use scalpel [1]. Smear the minced tissue with a swab [2] and transfer the swab to a tube prefilled with homogenization buffer and beads [3-TXT].

    - Talent mincing tissue using a scalpel.

    - Talent smearing the tissue with a swab.

    - Talent transferring the swab with tissue to the homogenization buffer tube. TXT: Place remaining tissues in a conical tube for long-term storage

  - Remove and discard the outer glove using QAC disinfectant diluted 1:256 (one to two fifty-six), clean and disinfect the workstation, equipment, and outside of the sample tubes [1]. NOTE: VO is modified to compensate for the removed shot.

    - Talent removing and discarding the outer glove. NOTE: Shot removed.

    - Talent spraying and wiping down the workstation, equipment, and sample tubes.

  - Homogenize samples with a mini bead beater for 60 seconds [1] and visually inspect the tubes [2-TXT]. Allow the samples to sit for 5 minutes at room temperature [3].

    - Talent placing sample tubes in the bead beater and starting the device.

    - Talent inspecting sample tubes for large tissue pieces. TXT: Repeat homogenization if large tissue pieces remain

    - Shot of labeled samples containing tubes placed on a working platform. NOTE: After the video, add a still shot showing two tubes, homogenized tissue (left) and non-homogenized (right) with TXT underneath as “Homogenized tissue” and “Non-homogenized tissue”

  - Finally, clean and disinfect the workstation, equipment, and outside of sample tubes with QAC disinfectant before proceeding to RNA extraction [1-TXT].

    - Talent disinfecting the workstation and tubes. TXT: Alternatively, store samples at -16 °C for long-term

- RNA Extraction from Sheep Brain Tissue

Demonstrator: Rene Edgar Condori

  - To begin, perform surface decontamination of the biological safety cabinet using QAC (Q-A-C) diluted to 1:256 (one to two fifty-six) [1]. Perform additional cleaning to remove dust or other environmental contaminants [2]. NOTE: VO is swapped for the swapped shots.

3.1.2. Talent spraying QAC disinfectant and wiping the BSC work surface. NOTE: Shots 3.1.1 and 3.1.2 are swapped.

3.1.1. WIDE: Talent cleaning the BSC work surface with a cloth to remove dust.

  - Lay out a plastic-lined absorbent work pad on the working platform [1]. Then, arrange the reagents and the samples in the biosafety cabinet [2].

    - Talent placing a plastic-lined absorbent pad inside the BSC.

    - Talent arranging reagents and samples in the BSC.

  - Lay out all collection tubes in a clean rack for microcentrifuge tubes [1]. Prefill one 1.5 milliliter microcentrifuge tube with 300 microliters of 100% ethanol for each brain sample [2-TXT].

    - Talent laying out microcentrifuge tubes in a rack.

    - Talent prefilling 1.5 milliliter tubes with 300 microliters of ethanol. TXT: For small tissue or bat samples, use 600 µL of ethanol

  - Centrifuge the homogenized sheep brain samples at 10,000 to 16,000 g for 2 minutes in a tabletop microcentrifuge [1]. Transfer the clear pink supernatant into a new sterile tube containing 100% ethanol [2].

    - Talent loading sample tubes into a tabletop microcentrifuge.

    - Talent transferring the clear pink supernatant to a new tube with ethanol.

  - Transfer 600 microliters of the ethanol-supernatant mixture to a spin column in a collection tube [1]. Centrifuge until the liquid passes through the column [2]. Discard the flow-through [3] and transfer each column to a new collection tube [4].

    - Talent transferring the ethanol-supernatant mixture to a spin column.

    - Talent placing the tube in a centrifuge.

    - Talent discarding the flow-through.

    - Talent placing the spin column in a new collection tube.

  - Add 400 microliters of RNA (R-N-A) prewash buffer to each column [1]. Centrifuge the mixture at 10,000 to 16,000 g for 30 seconds [2]. Discard the flow-through and return each column to the same collection tube [3].

    - Talent adding RNA prewash buffer to the spin columns.

    - Talent centrifuging the tubes.

    - Talent discarding the flow-through and placing the columns in the same tube.

  - Next, add 700 microliters of RNA wash buffer to each column [1]. Centrifuge at 10,000 to 16,000 g for 2 minutes [2] and carefully transfer the column into an RNase (R-N-ace)-free tube [3]. Discard the flow-through and the collection tube [4].  Then, remove and discard outer gloves [5].

    - Talent adding RNA wash buffer to the spin columns.

    - Talent placing the tubes in a centrifuge.

    - Talent transferring spin columns to RNase-free tubes.

    - Talent discarding the collection tubes.

    - Talent removing and discarding the outer gloves.

  - Now, add 50 microliters of DNase (D-N-ace) and RNase-free water directly to the column matrix and incubate for 30 seconds [1]. Then, centrifuge at 10,000 to 16,000 g for 1 minute [2]. Carefully transfer RNA to a new screw-top flat-bottom accession-labeled microcentrifuge tube for RT-PCR (R-T-P-C-R) assay [3-TXT].

    - Talent adding water to the spin columns to elute RNA.

    - Talent centrifuging the spin columns.

    - Talent transferring RNA to a labeled microcentrifuge tube. TXT: Alternatively, store at -70 °C

- Pan-Lyssavirus LN34 Real-Time RT-PCR Assay

Demonstrators: Vaughn Wicker and Kimberly Wilkins

  - To begin, thaw one-step RT-PCR (R-T-P-C-R) buffer, no template control, nuclease-free water, primers, and probes on ice in the master mix preparation space. Place one-step RT-PCR enzyme on ice [1]. NOTE: VO is modified for the removed shots.

    - WIDE: Establishing shot of talent placing the labeled container reagents on ice.

    - Talent vortexing the tube containing primers, buffers, and probes. NOTE: Shot removed

    - Talent placing enzyme tube and RNA sample tubes on ice. NOTE: Shot removed.

  - Label one microcentrifuge tube per assay for LN34 (L-N-thirty-four) and βA (beta-Actin) [1]. Calculate the volume of each reagent for the LN34 and βA mastermixes [2]. Designate wells for each sample in triplicate for the LN34 assay and singlicate for the βA assay using a 96-well plate map [3].

    - Talent labeling tubes for LN34 and βA assays.

    - LAB MEDIA: Table 2

    - Talent marking the wells of a 96-well plate for the assay.

  - After vortexing and spinning, dispense 23 microliters of LN34 assay master mix into each LN34-assigned well [1]. Similarly, add 23 microliters of βA assay master mix into each βA-labeled well [2].

    - Talent dispensing LN34 master mix from the labeled tube into labeled wells.

    - Talent adding βA master mix from the labeled tube into labeled wells

  - To set up negative template control reactions, add 2 microliters of PCR-grade water into each negative template control well [1]. Place the 96 well plate on ice [2]. Clean the workstation with 70% ethanol [3].

    - Talent transferring PCR-grade water into NTC wells.

    - Talent placing the plate on ice.

    - Talent wiping the workstation with 70% ethanol.

  - Now, at the template addition workspace, thaw RNA samples and positive control single use aliquot of artificial RNA on ice [1]. After vortexing, briefly centrifuge tubes containing RNA samples [2]. Pipette 2 microliters of sample and positive control RNA into the corresponding well [3].

    - Talent placing RNA samples and positive controls on ice. TXT: RNA samples extracted from sheep brains and previously extracted bat brain tissues were used NOTE: Text overlay removed per author’s request.

    - Talent placing RNA sample tube in a centrifuge.

    - Talent adding RNA into designated wells.

  - After adding all samples, place the optical adhesive cover over the wells to seal them completely [1]. Centrifuge the plate at 500 g for 1 minute at room temperature [2].

    - Talent sealing the 96-well plate with an optical adhesive cover.

    - Talent placing the sealed plate in a centrifuge.

  - Place the sealed plate into a real-time or quantitative PCR instrument calibrated for FAM (fam as in ‘FAM’ ily) and VIC/HEX (Vic-Hex) reporter dyes [1]. Set the instrument to the appropriate cycling parameters [2-TXT].

    - Talent loading the sealed plate into the PCR machine.

    - Display of the PCR cycling parameters on the instrument. TXT: Store RNA at ≤ -70 °C for long-term storage Video Editor: Split screen to show Table 3

  - After the run is completed, set the threshold values to 0.2 for LN34 and 0.05 for beta actin. Check the control sample curves for any quality issues [1].

    - Talent performing analyzation steps on computer and reviewing different views of data.

# Results

- Results

  - The LN34 assay showed successful amplification curves, with positive results crossing the threshold at distinct cycle threshold values when viewed on a logarithmic scale [1] and sigmoidal amplification curves on a linear scale for both LN34 and β (beta)-actin [2]. Negative results exhibited flat lines without amplification [3].

    - LAB MEDIA: Figure 2A and 2C

    - LAB MEDIA: Figure 2B and 2D

    - LAB MEDIA: Figure 2E and 2F

  - Abnormal amplification curves were identified, with some displaying linear rather than sigmoidal increases in fluorescence, indicating atypical results [1]. The corresponding multicomponent plots showed irregular wavy fluorescence signals rather than expected smooth curves, highlighting the need to analyze amplification plots instead of relying solely on threshold cycle values [2].

    - LAB MEDIA: Figure 3A

    - LAB MEDIA: Figure 3B

Pronunciation Guide:

Quaternary

- Pronunciation link (for "quaternary"): https://www.merriam-webster.com/dictionary/quaternary

- IPA: /ˈkwɑːrtəˌneri/

- Phonetic Spelling: KWAR-tuh-nair-ee

Scalpel

- Pronunciation link: https://www.merriam-webster.com/dictionary/scalpel

- IPA: /ˈskæl.pəl/

- Phonetic Spelling: SKAL-puhl

Homogenization

- Pronunciation link: https://www.merriam-webster.com/dictionary/homogenization

- IPA: /həˌmɑːdʒənəˈzeɪʃən/

- Phonetic Spelling: huh-MAH-juh-nuh-ZAY-shun

Microcentrifuge

- Pronunciation link: https://www.howtopronounce.com/microcentrifuge

- IPA: /ˌmaɪ.kroʊˈsɛn.trəˌfjuːdʒ/

- Phonetic Spelling: MY-kroh-SEN-truh-fyooj

Supernatant

- Pronunciation link: https://www.merriam-webster.com/dictionary/supernatant

- IPA: /ˌsuː.pɚˈnæt.ənt/

- Phonetic Spelling: SOO-per-NA-tuhnt

Lyssavirus

- Pronunciation link: https://www.howtopronounce.com/lyssavirus

- IPA: /ˈlɪsəˌvaɪrəs/

- Phonetic Spelling: LIH-suh-vy-ruhs

Primers

- Pronunciation link: https://www.merriam-webster.com/dictionary/primer (biotech meaning)

- IPA: /ˈpraɪmɚ/

- Phonetic Spelling: PRY-mer