# Submission ID #: 67134

# Scriptwriter Name: Debopriya Sadhukhan

# Project Page Link: https://review.jove.com/account/file-uploader?src=20475918

# Title: Assessment of DNase Activity by Ratiometric Fluorescence Resonance Energy Transfer

# Authors and Affiliations:

Candace L. Minchew1,2, Vladimir V. Didenko1,2

1 Baylor College of Medicine

# 2 Michael E. DeBakey Veterans Affairs Medical Center

# Corresponding Authors:

# Vladimir V. Didenko	(vdidenko@bcm.edu)

# Email Addresses for All Authors:

# Candace L. Minchew	(cminchew@bcm.edu)

# Vladimir V. Didenko	(vdidenko@bcm.edu)

## Author Questionnaire

1. Microscopy: Does your protocol require the use of a dissecting or stereomicroscope for performing a complex dissection, microinjection technique, or something similar?  No

2. Software: Does the part of your protocol being filmed include step-by-step descriptions of software usage? Yes

3. Filming location: Will the filming need to take place in multiple locations?   No

Current Protocol Length

Number of Steps:  11

Number of Shots:  16

# Introduction

Videographer: Obtain headshots for all authors available at the filming location.

REQUIRED:

  - Vladimir V. Didenko: We are addressing the need for sensitive detection of weak nuclease activity that requires prolonged observation and various pH conditions.

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: 2.1.1, 2.4.1, 2.4.2.

What research gap are you addressing with your protocol?

  - Vladimir V. Didenko: We developed this ratiometric FRET assay to detect and measure DNase activity of different biomolecules with high sensitivity and for extended time periods.

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: LAB MEDIA: Figure 2D, 2E.

How will your findings advance research in your field?

  - Candace L. Minchew: This assay enables the systematic evaluation of how experimental parameters like pH, temperature, and buffer composition affect DNase activity in real-time.

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: 3.1.2.

Videographer: Obtain headshots for all authors available at the filming location.

# Testimonial Questions:

# Videographer: Please ensure that all testimonial shots are captured in a wide-angle format, while also maintaining sufficient headspace, given that the final videos will be rendered in a 1:1 aspect ratio.

How do you think publishing with JoVE will enhance the visibility and impact of your research?

  - Vladimir V. Didenko, Associate Professor, Michael E. DeBakey Veterans Affairs Medical Center: Publishing with JoVE offers unique benefits. Its video format makes my research easier to understand, helping more people apply it in their own work.

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera.

# Protocol

- Preparation of Incubation Mix and Induction of DNase Reaction Using a Ratiometric FRET Probe

Demonstrator: Candace L. Minchew

  - To begin, dilute the protein sample to an appropriate concentration in each buffer solution ranging from pH 4.8 to pH 6.5 [1].

    - WIDE: Talent pipetting the protein sample into separate tubes containing buffers of pH 4.8 to pH 6.5. Videographer: Please make sure the labels on the tubes are visible in the frame.

  - Prepare the ratiometric FRET (Fret) probe solution by dissolving the FRET probe in nuclease-free water at a concentration close to 1 micromolar, or 1 picomole per microliter [1-TXT].

    - Talent pipetting nuclease-free water into a tube containing the FRET probe and gently mixing. TXT: Rapidly heat and cool the probe solution to dissociate unwanted base pairings

  - Now, prepare three wells in a 96-well plate for each pH condition to be tested by adding 70 microliters of nuclease-free water and 10 microliters of 10x (ten-ex) buffer to each well [1]. In three separate wells, prepare the positive controls by combining the specified reagents. For the negative controls, substitute the protein sample with nuclease-free water [2-TXT].

    - Talent pipetting 70 microliters of nuclease-free water and 10 microliters of buffer into the wells of pH series.

    - A shot of all specified reagents displayed on the bench as the talent pipetting them into the positive control wells. TXT: 79 μL nuclease-free water, 10 μL 10x DNase II buffer, 0.33 μL DNase II (1 U/μL stock), 10 μL ratiometric probe solution

  - Induce the DNase (D-N-ase) reaction by adding the probe solutions to the mixtures containing DNase protein at different pH values [1]. Mix all samples thoroughly, ensuring bubbles do not form [2].

    - Talent adding the FRET probe solution into each test and control well.

    - SCREEN: 2,4,2-Shaking-SCREEN.mp4. 00:00-00:44. Video Editor: Please remove 00:29-00:40.

  - Then, incubate the DNase samples containing the ratiometric FRET probe at 37 degrees Celsius using either a water bath or a temperature-controlled incubator for various durations depending on the strength of the DNase activity [1].

    - Talent placing the 96-well plate containing samples into a water bath or incubator set to 37 degrees Celsius.

  - Terminate all incubation reactions by adding 100 microliters of 250 millimolar Tris-Hydrochloric acid buffer at pH 8 to each well. This step also equalizes the pH across all samples [1].

    - Talent pipetting 100 microliters of Tris-HCl buffer into each well of the 96-well plate.

- Fluorometric Signal Detection, Calculation of FRET Ratios, and Cleavage Verification

  - Immediately after pH equalization, use a spectrofluorometer to record the emission spectra of both donor and acceptor fluorophores [1]. Perform measurements directly in the 96-well plate by exciting the donor fluorophore at 488 nanometers and simultaneously detecting emissions at 525 nanometers for the donor and 580 nanometers for the FRET-derived acceptor signal [2].

    - Talent placing the 96-well plate into the spectrofluorometer.

    - SCREEN: 3.1.2-Scan-Settings-SCREEN.mp4 00:00-00:26.

  - Use the recorded fluorescence intensities to assess DNase activity for each sample [1].

    - SCREEN: 3.2.1-Excel-SCREEN-UPDATED-REPLACEMENT.mp4 00:08-00:15, 00:30-00:39.

  - Then, calculate the FRET ratio using the given formula [1].

    - TEXT on PLAIN BACKGROUND:

RFRET = Donor emission intensity at 525 nmAcceptor emission intensity at 580 nm

  - Verify the cleavage of the FRET probe by loading 15 microliters of each sample onto a 20% denaturing polyacrylamide gel [1].

    - Talent pipetting 15 microliters of sample onto a 20% denaturing polyacrylamide gel.

  - Run the gel for 1 hour at 100 volts [1]. Finally, place the gel on the tray of the documentation system equipped with a high-resolution color camera [2] and acquire gel images [3].

    - Talent setting the gel apparatus to 100 volts and starting the run.

    - Talent placing the gel into the tray of the system.

    - Talent clicking the display screen to capture images. Videographer: Please record the screen for this shot.

# Results

- Results

  - Denaturing polyacrylamide gel analysis revealed that [1] incubation of the FRET probe with leukocyte elastase inhibitor or LEI (L-E-I) at pH 4.8 for 24 hours led to its cleavage, producing distinct fluorescein amidite or FAM (Fam) and tetramethylrhodamine or TAM (tam)-labeled fragments of different lengths [2].

    - LAB MEDIA: Figure 2A.

    - LAB MEDIA: Figure 2A. Video Editor: Highlight the 3rd image from the left (LEI).

  - The size and labeling of the cleavage products indicated specific cuts near the apexes of both hairpins in the probe, consistent with the unique fragmentation pattern produced by LEI [1], and distinct from that of DNase II [2].

    - LAB MEDIA: Figure 2A, 2B. Video Editor: Highlight the 3rd image from the left (LEI) in 2A.

    - LAB MEDIA: Figure 2B, 2C. Video Editor: Highlight 2C.

  - Emission spectra of LEI-treated FRET probes showed [1] maximal signal change at pH 5.2 [2], followed by pH 4.8 [3] and then pH 5.5 [4], indicating peak DNase activity in this acidic range [5], with no significant activity in the control sample [6].

    - LAB MEDIA: Figure 2D.

    - LAB MEDIA: Figure 2D. Video editor: Highlight the curve for pH 5.2.

    - LAB MEDIA: Figure 2D. Video editor: Highlight the curve for pH 4.8.

    - LAB MEDIA: Figure 2D. Video editor: Highlight the curve for pH 5.5.

    - LAB MEDIA: Figure 2D.

    - LAB MEDIA: Figure 2D. Video editor: Highlight the curve for Control.

  - Quantitative analysis of FRET ratios confirmed that [1] LEI exhibited significantly higher activity at pH 5.2 than at pH 4.8 [2], and both were markedly higher than at pH 5.5 [3], with activity dropping sharply above pH 5.5 [4].

    - LAB MEDIA: Figure 2E.

    - LAB MEDIA: Figure 2E. Video editor: Highlight the bars at pH 5.2 and pH 4.8 in the top graph (LEI).

    - LAB MEDIA: Figure 2E. Video editor: Highlight the bars at pH 5.2, pH 4.8, and 5.5 in the top graph (LEI).

    - LAB MEDIA: Figure 2E. Video editor: Highlight the bars at pH 5.8, 6.0, 6.2 and 6.5 in the top graph (LEI).