# Submission ID #: 67342

# Scriptwriter Name: Sulakshana Karkala

# Project Page Link: https://review.jove.com/account/file-uploader?src=20539358

# Title: A Rehabilitation Program of Exoskeleton-Assisted Body Weight-Supported Treadmill Training with Non-Immersive Virtual Reality for Stroke Patients

# Authors and Affiliations:

# Zhengwei Chen, Qing Li, Yi Zheng, Houqiang Zhang, Lixia Chen

# Department of Rehabilitation Medicine, Peking Union Medical College Hospital

# Corresponding Authors:

Lixia Chen			(clx727@126.com)

# Email Addresses for All Authors:

Zhengwei Chen 		(chenzhengwei@pumch.cn)

Qing Li 			(liqingwawj@sina.com)

Yi Zheng 			(1152343246@qq.com)

Houqiang Zhang		(1391502491@qq.com)

Lixia Chen			(clx727@126.com)

## Author Questionnaire

1. Microscopy: Does your protocol require the use of a dissecting or stereomicroscope for performing a complex dissection, microinjection technique, or something similar?  NO

2. Software: Does the part of your protocol being filmed include step-by-step descriptions of software usage?  Yes, all done

3. Filming location: Will the filming need to take place in multiple locations?   No

Current Protocol Length

Number of Steps:  14

Number of Shots:  36

# Introduction

Videographer: Obtain headshots for all authors available at the filming location.

REQUIRED: What is the scope of your research? What questions are you trying to answer?

  - Zhengwei Chen: We explored whether combining exoskeleton-assisted treadmill training with game-based VR could enhance dual-task performance and balance in stroke survivors during early rehabilitation.

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B.roll:3.7.2-3.7.3

What are the current experimental challenges?

  - Zhengwei Chen: It remains challenging to maintain patient engagement throughout intensive sessions, especially in early stroke recovery when fatigue and attention deficits are common.

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B.roll:.3.7.1

What significant findings have you established in your field?

  - Zhengwei Chen: Our study suggests that this combined approach may help stroke patients better manage balance during walking.

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B.roll:4.1

What advantage does your protocol offer compared to other techniques?

  - Zhengwei Chen: This approach enhances both physical recovery and cognitive engagement, providing a richer and more effective rehab experience.

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B.roll:4.2

How will your findings advance research in your field?

  - Zhengwei Chen: Our findings support integrating multimodal technologies in stroke rehab and provide preliminary evidence to guide future randomized controlled trials.

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera.

Videographer: Obtain headshots for all authors available at the filming location.

Ethics Title Card
This research has been approved by the Institutional Review Board (IRB) at Peking Union Medical College Hospital

# Protocol

- Donning and Adjusting the Weight-Supported Exoskeleton System

Demonstrator: Zhengwei Chen

  - To begin, turn both handles on the robotic arm counterclockwise to loosen them [1]. Pull the exoskeleton outward to clear the treadmill runway and create space for the patient [2].

    - WIDE: Talent rotates both handles counterclockwise on the robotic arm, focusing on hand movement and handle mechanics.

    - Talent pulls the exoskeleton away from the treadmill.

  - Guide the patient onto the treadmill runway from the rear ramp to the front [1]. For patients that are unable to walk, assist them in entering with a wheelchair and position them at the front [2].

    - Talent guiding a patient onto the treadmill runway.

    - Talent assisting a patient in a wheelchair at the front of the runway.

  - Lower the harness of the suspension system using the remote control [1]. Adjust the harness to sit flush with or slightly below the patient’s torso to ensure proper alignment [2].

    - Talent uses the remote control to lower the harness from the suspension system.

    - Shot of the harness being aligned with the patient’s torso, verifying correct positioning.

  - Then unstrap the harness to begin dressing the patient [1]. If the patient is standing, apply the unfastened harness to the torso from behind and secure the straps around the torso and thighs [2]. Ensure all straps are comfortably tightened [3].

    - Talent unfastens all straps of the harness to prepare for donning.

    - Talent applies the harness from behind and fastens the torso and leg straps.

    - Shot of leg and torso straps being snugly adjusted for comfort and safety.

  - If the patient is in a wheelchair, lift the torso slightly away from the backrest [1]. Thread the unfastened harness around the torso and secure the straps comfortably [2]. Wrap the leg straps around the thighs and fasten them [3].

    - Talent lifts the patient’s torso to create space behind the backrest.

    - Shot of the harness being threaded and strapped around the patient’s torso while seated.

    - Shot of the leg straps being secured snugly around the thighs.

  - Now, elevate the weight-supported system to bring the patient into a standing position [1-TXT]. Use the remote control to adjust the weight reduction as needed and observe the data on the unit screen [2]. Raise the patient slightly while ensuring that their feet do not hang [3].

    - Talent presses the remote to elevate the suspension system. TXT: Stop when harness tightens

    - Talent adjusting weight levels with a remote. 
Videographer: Please pan to the unit’s screen

    - Shot of the patient being raised slightly.

  - If the patient is in a wheelchair, gradually elevate them into a standing position using the suspension system [1]. Once upright, remove the wheelchair from the runway [2]. Adjust the weight reduction using the remote control as needed [3].

    - Shot of the Suspension system being used to lift the patient gently from the seated to the standing position.

    - Talent removes the wheelchair from the treadmill area.

    - Talent adjusting weight reduction with the remote.

- Donning and Operating the Exoskeleton

  - Reset the exoskeleton by pushing it inward from its opened position [1]. Then, rotate both handles clockwise to engage the immobilization device [2].

    - Talent pushes the exoskeleton inward until it is positioned over the treadmill.

    - Close-up of both handles being rotated clockwise into the locked position.

  - Press down on the folded and suspended exoskeleton to shift it from a seated to a standing configuration [1]. Then instruct the patient to lean back against the exoskeleton’s torso support [2] and attach the thoracic anchorage straps around the patient’s chest securely [3].

    - Talent presses down on the top of the suspended exoskeleton to transform its position into standing.

    - Patient leans back slowly into the support structure.

    - Talent fastens thoracic straps across the patient's torso, checking for proper fit.

  - Adjust the height of the exoskeleton to align the motor axis of the arms with the patient’s hip and knee joints [1]. Tighten the belts to a comfortable level [2]. Then secure the thigh and calf straps, ensuring a snug and safe fit [3].

    - Talent adjusts the height control.

    - Talent tightens the waist belt with a firm pull and checks for comfort.

    - Shot of thigh and calf straps being wrapped and locked into place.

  - For exoskeleton operation, launch the control software on the computer [1]. Enter the patient’s basic information into the system [2].

    - SCREEN: 3.4.1-3.4.2.mov	00:03-00:12

    - SCREEN: 3.4.1-3.4.2.mov	00:18-00:38
Video Editor: Please speed up the video

  - Adjust treatment parameters within the software [1]. Set the walking speed to 1.5 kilometers per hour, duration to 20 minutes, and apply the default joint mobility settings for both hips and knees [2]. Then click on Start to begin therapy [3].

    - SCREEN: 3.5.1-3.5.3.mov	00:00-00:08

    - SCREEN: 3.5.1-3.5.3.mov.	00:09-00:16

    - SCREEN: 3.5.1-3.5.3.mov	00:17-00:23

  - Next, launch the ZEPU (Zee-poo) Gait Training and Evaluation software on the computer [1]. Select the Game option to activate the VR training mode [2].

    - SCREEN: 3.6.1-3.6.2.mov	00:00-00:04

    - SCREEN: 3.6.1-3.6.2.mov	00:04-00:10

  - Guide the patient during exoskeleton-assisted movement [1]. When one leg is in the swing phase, instruct the patient to actively control it [2]. When the leg is ready for propulsion, instruct the patient to forcefully propel it and perform hip flexion [3].

    - Talent guiding the patient during exoskeleton-assisted movement.

    - Talent pointing out swing phase and encouraging the patient to activate muscles accordingly.

    - Talent pointing out propulsion phase and encouraging the patient to activate muscles accordingly.

# Results

- Results

  - The Berg Balance Scale score showed a statistically significant increase from 43.88 to 48.38 after 4 weeks of treatment [1].

    - LAB MEDIA: Table 3 and Figure 3A	Video Editor: Please emphasize the row corresponding to BBS(score) in Table 3 and the light grey column corresponding to “Post” in Figure 3

  - The Timed Up-and-Go test time decreased from 21.88 seconds to 17.63 seconds, indicating improvement without statistical significance [1]. The Functional Independence Measure score increased from 92.75 to 98.75, reflecting a clinically positive trend though not statistically significant [2].

    - LAB MEDIA: Table 3 and Figure 3B	Video Editor: Please emphasize the row corresponding to TUG (score) in Table 3 and the light grey column corresponding to “Post” in Figure 3

    - LAB MEDIA: Table 3 and Figure 3C