# Submission ID #: 67810

# Scriptwriter Name: Poornima G

# Project Page Link: https://review.jove.com/account/file-uploader?src=20682098

# Title: Surgical Technique for the Implantation of a Wireless Telemetry Device on Pigs’ Backs

# Authors and Affiliations:

# Angel Moctezuma-Ramirez, Jordana Goulet, Abdelmotagaly Elgalad

# Center for Preclinical Surgical & Interventional Research, The Texas Heart Institute

# Corresponding Authors:

# Abdelmotagaly Elgalad		aelgalad@texasheart.org

# Email Addresses for All Authors:

Angel Moctezuma-Ramirez		amoctezuma@texasheart.org

# Jordana Goulet			goulet@livemail.uthscsa.edu

# Abdelmotagaly Elgalad		aelgalad@texasheart.org

## Author Questionnaire

1. We have marked your project as author-provided footage, meaning you film the video yourself and provide JoVE with the footage to edit. JoVE will not send the videographer. Please confirm that this is correct.

√ Correct

2. Microscopy: Does your protocol require the use of a dissecting or stereomicroscope for performing a complex dissection, microinjection technique, or something similar?  No

3. Software: Does the part of your protocol being filmed include step-by-step descriptions of software usage?  No

4. Proposed filming date: To help JoVE process and publish your video in a timely manner, please indicate the proposed date that your group will film here: 15th Sep 2025

When you are ready to submit your video files, please contact our Content Manager, Utkarsh Khare.

Current Protocol Length

Number of Steps:  23

Number of Shots:  48

# Introduction

Authors: We can have only upto 5 interview statements in the video. Hence other questions have been deleted.

REQUIRED: What is the scope of your research? What questions are you trying to answer?

  - Angel Moctezuma-Ramirez: We develop and refine different preclinical surgical and interventional models for heart failure and mechanical circulatory support, aiming to advance device design and implantation techniques in advanced cardiovascular research.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: 2.3.1

What are the current experimental challenges?

  - Angel Moctezuma-Ramirez: Maintaining long-term model stability, ensuring reproducible surgical techniques, and integrating complex monitoring systems without compromising animal welfare remain key experimental challenges in our work.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: 2.5.1

What research gap are you addressing with your protocol?

  - Angel Moctezuma-Ramirez: We address the challenge of streaming and recording biopotentials in chronic experiments when the groin and abdomen cannot be used due to other procedures.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: 3.2.1

What advantage does your protocol offer compared to other techniques?

  - Abdelmotagaly Elgalad: This technique improves welfare by placing the device on the back, reducing discomfort, allowing natural movement, providing reliable continuous data, and maintaining a low complication rate.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: 3.4.1

What research questions will your laboratory focus on in the future?

  - Abdelmotagaly Elgalad: We will focus on improving experimental design, maximizing throughput without compromising animal welfare, and identifying methods to collect high-quality data efficiently.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: 4.2.1

Ethics Title Card

This research has been approved by The Texas Heart Institute’s Institutional Animal Care and Use Committee

# Protocol

- Preparation of the Pig for the Procedure

Demonstrators: Technicians, Scrub Tech, Clinical Veterinarian, AMR/AE

Authors, please provide the names of the people

  - To begin, place the anesthetized pig on the surgical table in the ventral position in the operating room [1-TXT] and start an intravenous infusion of Plasmalyte at a rate of 10 to 15 milliliters per hour [2-TXT].

    - WIDE: Talent positioning the pig ventral-side down on the surgical table. TXT: Anesthesia induction: 3% Isoflurane

    - Talent connecting the Plasmalyte infusion line and setting the IV pump to 10 to 15 milliliters per hour. TXT: Maintenance of anesthesia: 0.5 - 3% Isoflurane

  - Using sterile gauze soaked in 70 percent isopropyl alcohol, clean the intended surgical incision site on the left upper back and the surrounding shaved area [1-TXT]. Allow the alcohol to air-dry completely [2].

    - Talent using gauze soaked in isopropyl alcohol to clean the surgical site in outward strokes. TXT: Area: ~ 15 cm x 15 cm; Wipe from the center to outwards

    - Shot of the cleaned and dried skin surface.

  - Perform a sterile surgical scrub using povidone-iodine scrub solution and sterile gauze [1]. Apply the scrub in firm, circular motions starting from the incision line and moving outward in expanding circles [2]. Continue scrubbing for at least 5 minutes, replacing gauze as necessary [3].

    - Talent pouring povidone-iodine scrub onto sterile gauze.

    - Talent scrubbing the surgical site in circular outward motions.

    - Shot of changing the scrub pad.

  - Apply povidone-iodine solution using sterile gauze or sponge sticks, moving from the center outward [1] and allow the final paint layer to air-dry without wiping it off [2].

    - Talent applying povidone-iodine solution with a sponge stick to the prepped skin.

    - Shot showing the iodine layer dried on the skin surface.

  - Now, isolate the prepared surgical area using sterile surgical drapes [1].

    - Talent carefully placing sterile surgical drapes around the cleaned and painted incision area.

  - Identify the anatomical landmarks forming a triangle, including the spine as the upper border, the scapula as the left border, and the lower edge of the trapezius muscle as the right border [1].

    - Talent palpating and pointing out each anatomical landmark on the pig’s upper back.

  - Next, using a number 10 blade, make a 2-inch longitudinal incision approximately 2 inches lateral to the spine [1].

    - Talent making a precise longitudinal incision on the pig’s back with a number 10 scalpel blade.

  - After incising the epidermis, use an electrocautery to dissect through the subcutaneous tissue and divide the trapezius muscle [1]. Take care to divide all fascial layers [2].

    - Talent using electrocautery to separate subcutaneous layers.

    - Close-up of the tool dividing all fascial layers with controlled precision.

  - Once the inner fascial layer is divided, use blunt dissection with the index and middle fingers to create a pocket [1]. Ensure the tip of the scapula is reachable with the fingertips [2] and confirm that the tip of the scapula is palpable at the pocket’s extent after dissection [3].

    - Talent inserting fingers and spreading the tissue to form a pocket.

    - Talent touching and reaching the scapula tip with fingers.

    - Shot of the scapula tip at the pocket boundary.

  - Check that the telemetry device fits inside the pocket with minimal tension on the skin incision [1] and then remove the device from the pocket [2].

    - Talent placing the telemetry device into the pocket and assessing fit.

    - Talent removing the device after size verification.

  - Mix 1 gram of vancomycin with 50 milliliters of 0.9 percent saline [1]. Place approximately 10 milliliters of this mixture into the pocket and let it sit for 1 to 2 minutes [2]. Remove excess solution using a gauze pad [3].

    - Talent mixing vancomycin powder with saline in a sterile vial.

    - Talent using a syringe to add 10 milliliters of solution into the pocket.

    - Talent blotting excess liquid from the pocket with gauze.

- Lead Placement and Device Positioning

Demonstrator: Angel Moctezuma-Ramirez, Abdelmotagaly Elgalad

  - Use a 14 French angiocath to place the wires as per orientation requirements [1].

    - Talent preparing a 14 French angiocath and lead wires for insertion.

  - Insert the angiocath in a lateral to medial, outer to inner direction under the muscle, guiding it toward the pocket [1]. Advance the needle until only one-third remains visible [2]. Remove the needle and thread the lead through the catheter from the tip toward the hub [3].

    - Talent inserting the angiocath under the muscle from lateral to medial.

    - Close-up of the needle advancement until one-third remains outside.

    - Talent threading the lead wire through the catheter.

  - Leave between 1 and 2 inches of lead wire exposed to prevent subcutaneous migration during the placement of additional leads [1] and remove the catheter after securing the lead [2].

    - Talent pointing to 1 to 2 inches of exposed lead wire.

    - Talent removing the catheter while keeping the lead in place.

  - Now, insert one lead of any colour among yellow, red, green, or blue into position [1]. Immediately follow it with the corresponding transparent lead inserted on the contralateral side of the pocket [2].

    - Talent inserting a yellow lead on one side of the pocket.

    - Talent inserting the corresponding transparent lead on the opposite side.

  - Insert each color-transparent pair together, one pair at a time, to mirror their arrangement on the telemetry device and avoid confusion during setup [1]. These leads form differential pairs for ECG channels, supporting bipolar signal acquisition and minimizing electrical noise [2]. After all lead pairs are placed, insert the black ground lead last to complete the configuration [3].

    - Talent placing the lead pair and looking at the telemetry device layout.

    - Shot of multiple colored and transparent lead pairs placed.

    - Close-up of black ground lead being inserted last.

  - Next, pull the wires gently so the tips of the leads are just beneath the skin surface [1]. Make sure no part of any lead is visible externally [2].

    - Talent gently adjusting wire lengths so lead tips are barely subdermal.

    - Close-up confirming absence of lead exposure at the skin level.

  - Then, gather all the wires into a loop, using the hand as a guide [1] and secure them together with heavy silk ties to create a bundled group [2].

    - Talent looping wires into a neat bundle using their hand.

    - Talent tying the bundle securely with heavy silk suture.

  - Place the telemetry device and the lead bundle into the created pocket [1]. Ensure the device sits flat and is oriented properly. One side of the device is labeled to face out [2].

    - Talent placing the bundled wires and device inside the pocket.

    - Close-up of label confirming correct orientation: “This side facing out.”

  - Now, adjust the device so that its lateral edge lies at least 1 to 2 centimeters medial to the palpable caudal edge of the scapula [1]. Gently move the pig’s left forelimb through its full range of motion to ensure that the device does not rub or impinge against the scapula [2].

    - Talent adjusting device position relative to the scapula.

    - Talent flexing and extending the pig's forelimb while observing device position.

  - Confirm that the device is successfully transmitting data to the receiver and connected computer [1]. Successful transmission is defined by accurate heart rate readings, rhythm analysis, and QRS complex identification [2].

    - Screen recording or monitor display showing telemetry signals being received.

    - Close-up of waveform display highlighting heart rate and QRS complexes.

  - Next, ensure complete hemostasis at the surgical site [1]. Close the muscle layer using a 1-0 (1-oh) Vicryl suture [2] and apply approximately 10 milliliters of the vancomycin solution over it. Allow 1 to 2 minutes for absorption [3].

    - Shot of inspecting the area for active bleeding.

    - Talent closing the muscle layer with 1-0 Vicryl in interrupted or continuous stitches.

    - Talent applying vancomycin solution over the sutured muscle.

  - Finally, close the subcutaneous layer with 2-0 (2-oh) Vicryl [1]. Close the skin incision using surgical staples [2] and reinforce closure with tissue glue [3].

    - Talent suturing the subcutaneous tissue with 2-0 Vicryl.

    - Talent applying skin staples to close the skin.

    - Talent applying a line of tissue glue over the incision.

# Results

- Results

  - Successful electrocardiogram transmission was achieved in all 40 pigs, demonstrating consistent long-term signal acquisition from the implanted telemetry devices [1].

    - LAB MEDIA: Table 1. Video editor: Highlight the row “Successful ECG transmission” and the value “40” under “Number of Pigs”.

  - No major bleeding events occurred during the implantation procedure, indicating the safety of the fascial pocket dissection and vascular handling techniques [1].

    - LAB MEDIA: Table 1. Video editor: Highlight the row “Major bleeding” and the value “0” under “Number of Pigs”.

  - Infections were observed in 2 pigs early in the study and were successfully managed, representing a 5% complication rate prior to protocol improvements [1].

    - LAB MEDIA: Table 1. Video editor: Highlight the row “Infection” and the value “2” under “Number of Pigs”.

  - All 40 animals survived to the 180-day endpoint, confirming the long-term viability of the implantation approach [1].

    - LAB MEDIA: Table 1. Video editor: Highlight the row “Survival to 180 days” and the value “40” under “Number of Pigs”.

  - Representative electrocardiogram tracings taken on postoperative day 30 showed well-defined P waves, consistent QRS complexes, and minimal baseline drift, indicating high-fidelity signal acquisition [1].

    - LAB MEDIA: Figure 4. Video editor: Zoom in on the first 4 set of wavy lines from the top

  - The lower channels in the telemetry output confirmed stable core body temperature [1] and a continuous 100% signal quality rating throughout the recording segment [2].

    - LAB MEDIA: Figure 4. Video editor: Highlight the Flat line at the mark of “35.86”.

    - LAB MEDIA: Figure 4. Video editor: Highlight the flat line at the bottomat the mark of 100.