# Submission ID #: 67837

# Scriptwriter Name: Sulakshana Karkala

# Project Page Link: https://review.jove.com/account/file-uploader?src=20690333

# Title: Unilateral Lung Volume Analysis Using Micro-CT for Enhanced Assessment of Pulmonary Fibrosis in Preclinical Models

# Authors and Affiliations:

Xihui Huang1,2*, Shu Xia1*, Yiqian Yang1*, Chengcheng Lei3*, Penghui Yang1, Shuang Li1, Jianyi Niu1, Jian Wu3, Qun Luo1, Zhenli Fu3

1State Key Laboratory of Respiratory Diseases, National Clinical Research Center for Respiratory Diseases, Guangzhou Institute of Respiratory Health, the First Affiliated Hospital of Guangzhou Medical University

2Guangzhou National Laboratory

3Second Department of Elderly Respiratory, Guangdong Provincial People's Hospital (Guangdong Academy of Medical Sciences), Southern Medical University, Guangdong Provincial Geriatric Institute

*These authors contributed equally to this work

# Corresponding Authors:

Jian Wu				(sywujian@scut.edu.cn)

Qun Luo				(luoqunx@163.com)

Zhenli Fu 				(fuzhenli@gdph.org.cn)

# Email Addresses for All Authors:

Xihui Huang				(hxh513141@163.com)

Shu Xia                        (xiashu0327@163.com)

Yiqian Yang				(yangyiqian03@163.com)

Chengcheng Lei			(may2021@163.com)

Penghui Yang			      (yph0913@gird.cn)

Shuang Li				(18707487685@163.com)

Jianyi Niu				(niujianyi001@163.com)
Jian Wu				(sywujian@scut.edu.cn)

Qun Luo				(luoqunx@163.com)

Zhenli Fu 				(fuzhenli@gdph.org.cn)

## Author Questionnaire

1. We have marked your project as author-provided footage, meaning you film the video yourself and provide JoVE with the footage to edit. JoVE will not send the videographer. Please confirm that this is correct.

√ Correct

2. Microscopy: Does your protocol require the use of a dissecting or stereomicroscope for performing a complex dissection, microinjection technique, or something similar?  NO

3. Software: Does the part of your protocol being filmed include step-by-step descriptions of software usage?  Yes

Authors: Please create screen capture videos of the following shots labeled as SCREEN, create a screenshot summary, and upload the files to your project page as soon as possible: https://review.jove.com/account/file-uploader?src=20690333

SCREEN: 3.2.2, 4.7.1-4.7.2

4. Proposed filming date: To help JoVE process and publish your video in a timely manner, please indicate the proposed date that your group will film here: 06/30/2025

When you are ready to submit your video files, please contact our China Location Producer, Yuan Yue.

Current Protocol Length

Number of Steps:  21

Number of Shots:  52

# Introduction

REQUIRED: What is the scope of your research? What questions are you trying to answer?

  - Zhenli Fu: Our group has been dedicated to basic and translational research in organ fibrosis, with a recent focus on developing early diagnostic approaches and precision therapies for pulmonary fibrosis.

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B.roll:5.1

What research gap are you addressing with your protocol?

  - Zhenli Fu: We developed a micro-CT-based unilateral ventilation lung volumetry approach enabling precise pulmonary fibrosis evaluation, potentially transforming preclinical intervention assessment.

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera.

What advantage does your protocol offer compared to other techniques?

  - Xihui Huang: Unilateral bleomycin instillation induces precise left lung injury in mice, ensuring comparable pre-treatment severity between groups, quantified via micro-CT for comprehensive intervention evaluation.

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B.roll:2.4

What research questions will your laboratory focus on in the future?

  - Xihui Huang: Our group will develop AI-assisted automated fibrotic lesion segmentation algorithms to enhance quantitative assessment accuracy.

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera.

Ethics Title Card

This research has been approved by the Institutional Animal Care and Use Committee (IACUC) at Guangzhou Medical University

# Protocol

- Unilateral Pulmonary Intratracheal Instillation

Demonstrator: Xihui Huang

  - To begin, observe the respiratory rate of the anesthetized mouse and test the limb withdrawal reflex [1]. Place the mouse in a supine position on a plastic board angled between 70 and 90 degrees [2] and restrain it by the incisors and limbs [3].

    - WIDE: Talent observing a deeply anesthetized mouse, noting respiratory rhythm and performing limb pinch test.

    - Talent placing the anesthetized mouse in a supine position on the angled board.

    - Talent tying the mouse using incisor loop and limb restraints.

  - Use sterile blunt-ended forceps to gently retract the tongue [1]. Insert a custom 22-gauge blunt needle into the trachea using a small animal laryngoscope to visualize the epiglottis and oropharynx structures [2].

    - Talent opening the mouth and carefully pulling the tongue forward with forceps.

    - Talent positioning the laryngoscope and guiding the needle down the trachea.

  - Rotate the board counterclockwise to negative 70 degrees to align the left main bronchus parallel to the main bronchus [1]. Advance the needle until resistance is felt [2].

    - Talent rotating the board counterclockwise to negative 70 degrees.

    - Talent advancing the needle through the trachea until it stops.

  - Rotate swiftly to a 30-degree angle [1] and infuse 40 microliters of bleomycin solution [2]. Then remove the needle [3].

    - Talent rotating the board to a 30 degree angle.

    - Talent injecting bleomycin solution.

    - Talent withdrawing the needle from the trachea.

  - Hold the mouse in position for at least 30 seconds to ensure proper drug distribution [1]. Then allow the mouse to recover prone on a warming blanket [2].

    - Talent stabilizing the mouse on the angled board for 30 seconds.

    - Talent placing the mouse on a warming blanket in prone position and monitoring its recovery.

- Diagnostic Imaging in Murine Models

  - Perform micro-computed tomography scanning on mice for 3 weeks after bleomycin instillation [1]. Remove the mice from the induction box [2] and place them supine on the scanning table with limbs extended and body aligned along the midline [3].

    - Talent preparing mice for imaging session.

    - Talent removing a mouse from the induction box.

    - Talent placing the mouse on the micro-CT table, aligning its limbs and body.

  - Set the isoflurane maintenance concentration in the scanning table to 1% [1]. Then set the micro-CT scanning mode parameters with transverse field of view 43 millimeters, resolution of 2K by 2K, pixel size 0.042 millimeter, and slice thickness 0.042 millimeter [2]. After scanning, place the mice on a warming blanket until they awaken [3].

    - Talent setting the isoflurane maintenance concentration.

    - SCREEN: Show the scanner interface with the specified scan mode parameters being input.
AUTHORS: Please provide this file. The files you provided were incorrectly named. Screen capture files had information for steps 3.3 onwards. Also provide SC file for 4.7.1-4.7.2

    - Talent gently transferring mice to a warming blanket post-scan.

  - Next, launch the 3D Slicer program to begin post-processing for unilateral selective lung segmentation and volume analysis [1]. Click Load to import imaging data in DICOM (die-com) format. Display axial, coronal, and sagittal images in the main interface [2].

    - SCREEN: 67837_screenshot_2.mp4	00:00-00:04	.

    - SCREEN: 67837_screenshot_1.mp4	00:00-00:23

  - Adjust sagittal view parameters to show hidden options then choose Rename current volume, and enter new name [1]. In the Segment Editor, click Add to create a segmentation for the right lung. Adjust threshold range to [-1000, -350] (minus-one-thousand-and-minus-Three-fifty) Hounsfield Units and enable Use for Masking [2].

    - SCREEN: 67837_screenshot_2.mp4	00:00-00:08	.

    - SCREEN: 67837_screenshot_2.mp4	00:09-00:23

  - Now use the Paint tool with sphere brush to segment the desired lung regions [1]. Select the Island tool and choose Keep Selected Island. Highlight the right lung, then click Show 3D to display the 3D model in the visualization panel [2].

    - SCREEN: 67837_screenshot_3.mp4	00:00-00:12	.

    - SCREEN: 67837_screenshot_3.mp4	00:13-00:22, 00:32-00:47

  - In the Terminology panel, change the 3D model's color to blue [1].

    - SCREEN: 67837_screenshot_4.mp4	00:00-00:12

  - After repeating the steps to segment the left lung [1], process the axial chest CT images to calculate ventilated lung volume [2] , then generate a 3D model [3].

    - SCREEN: 67837_screenshot_5.mp4	00:00-00:21	.

    - SCREEN : 67837_screenshot_5.mp4	03:25-03:37.

    - SCREEN : 67837_screenshot_5.mp4	03:38-03:51.

  - Now change the 3D model color of left lung to yellow [1]. Set the opacity of the right lung segmentation to 0.2 [2]. Then export the 3D model as required [3].

    - SCREEN: 67837_screenshot_6.mp4	00:04-00:21.

    - SCREEN: 67837_screenshot_6.mp4	00:22-00:29.

    - SCREEN: 67837_screenshot_6.mp4.	00:33- 00:41

  - Capture screenshots of 3D view or slice images and save them by clicking on Annotation Screenshot, selecting image format then modifying the save path [1]. Use Segment Statistics module to calculate lung volume in cubic millimeters and analyze normo-aerated regions [2].

    - SCREEN: 67837_screenshot_7.mp4	00:00-00:16	.

    - SCREEN: 67837_screenshot_7.mp4	00:26-00:51

- Masson's Trichome Staining of Murine Lung Samples

Demonstrator: Xihui Huang

  - Fix the lung sample with 4% paraformaldehyde at room temperature overnight [1].  Dehydrate the fixed sample through a series of graded ethanol [2].

    - Talent placing lung sample in fixative.

    - Talent transferring the fixed samples to a series of graded ethanol.

  - Then embed the samples in paraffin [1]. Prepare 5-micrometer thick slices using a rotary microtome [2].

    - Shot of the paraffin embedded sample.

    - Talent using rotary microtome to prepare paraffin slices.

  - After deparaffinizing and rehydrating the sections, rinse them in distilled water [1]. Next, treat the samples with preheated Bouin fluid at 56 to 60 degrees Celsius for 1 hour or overnight [2].

    - Talent placing deparaffinized and rehydrated sections in distilled water.

    - Talent transferring slides between solutions and incubator.

  - Stain the slices, after washing, with Weigert's iron hematoxylin for 10 minutes [1]. Then rinse under warm tap water for 10 minutes [2], before immersing the sections in Biebrich scarlet-acid fuchsin for 10 to 15 minutes [3].

    - Talent places the slices in Weigert’s iron hematoxylin solution.

    - Shot of the slices being placed under warm tap water.

    - Shot of the slices being transferred to Biebrich scarlet-acid fuchsin solution.

  - Differentiate with phosphomolybdic-phosphotungstic acid for 10 to 15 minutes until collagen-rich areas become clear [1]. Then transfer the slices to aniline blue for 5 to 10 minutes [2], before rinsing in distilled water [3]. Differentiate the slices in 1% acetic acid for 2 to 5 minutes [4].

    - Talent performing acid treatment and monitoring stain change.
AUTHORS: Please try to capture the change in the collagen-rich areas of the sections, if possible

    - Talent transferring the slices to aniline blue.

    - Shot of slices being placed in distilled water.

    - Shot of 1% acetic acid being added to the slices.

  - Next, dehydrate slides through graded ethanol [1]. Clear the dehydrated section in xylene [2]. Finally, mount the sections with resinous medium [3].

    - Talent placing the sections in graded ethanol.

    - Shot of the dehydrated sections being placed in xylene.

    - Talent mounting the section with resin.

  - For staining machine learning analysis, acquire high-resolution digital images of stained sections using a whole-slide scanner [1]. Analyze collagen composition with orbit image analysis software to minimize observer bias [2].
AUTHORS: Please provide this file.

    - SCREEN: High-resolution images of the stained sections are being seen.

    - SCREEN: Show Orbit software processing collagen segmentation and quantitative output.

# Results

- Results

  - The left lung volume in the bleomycin-treated group was significantly reduced compared to the saline-treated control group [1].

    - LAB MEDIA: Figure 10A, C  Video Editor: Please highlight the BLM image of 10A and BLM column of 10 C

  - Collagen deposition in the lung tissue, indicated by Masson's Trichrome staining, was significantly elevated in the bleomycin-treated group [1].

    - LAB MEDIA: Figure 10B,D. Video Editor: Please highlight the BLM image of 10B and BLM column of 10 D