# Submission ID #: 67878

# Scriptwriter Name: Poornima G

# Project Page Link: https://review.jove.com/account/file-uploader?src=20702838

# Title: Analyzing Platelet Subpopulations by Multi-Color Flow Cytometry

# Authors and Affiliations:

# Zoi Laspa, Anne-Katrin Rohlfing, Meinrad P. Gawaz

# Department of Cardiology and Angiology, University Hospital Tübingen, University of Tübingen

# Corresponding Authors:

# Zoi Laspa			zoi.laspa@med.uni-tuebingen.de

# Meinrad P. Gawaz		meinrad.gawaz@med.uni-tuebingen.de

# Email Addresses for All Authors:

# Anne-Katrin Rohlfing		anne-katrin.rohlfing@med.uni-tuebingen.de

# Meinrad P. Gawaz		meinrad.gawaz@med.uni-tuebingen.de

# Zoi Laspa			zoi.laspa@med.uni-tuebingen.de

## Author Questionnaire

1. Microscopy: Does your protocol require the use of a dissecting or stereomicroscope for performing a complex dissection, microinjection technique, or something similar?  NO

2. Software: Does the part of your protocol being filmed include step-by-step descriptions of software usage?  Yes,all done

3. Filming location: Will the filming need to take place in multiple locations?   NO

Current Protocol Length

Number of Steps:  23

Number of Shots:  40 (3 SC)

# Introduction

Videographer: Obtain headshots for all authors available at the filming location.

  - Zoi Laspa: Our research uses multicolor flow cytometry to identify and characterize platelet subpopulations, aiming to understand their roles, activation patterns, and clinical relevance in thrombosis, inflammation and disease-specific contexts.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: 2.3.1

What significant findings have you established in your field?

  - Zoi Laspa: The established multi-color flow cytometric assay enabled to observe that hemin, a platelet agonist, induces a distinct activation pattern and, compared to collagen-related peptide, generates a strong procoagulant, chemokine receptor-positive subpopulation.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: 2.5.1

What research gap are you addressing with your protocol?

  - Zoi Laspa: The multi-color assay enables the simultaneous measurement of platelet activation, adhesion, aggregation and apoptotic potential of a single platelet within a population of isolated human platelets in a single tube.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: 5.1.1

Videographer’s Note: Head shot was done for Zoe Laspa and Anne-Katrin Rohlfing. Meinrad P. Gawaz was not available

Videographer: Obtain headshots for all authors available at the filming location.

# Ethics Title Card

This research has been approved by the Ethics Committee of the Medical Faculty of the Eberhard Karls University and the University Hospital of Tübingen

# Protocol

NOTE: LAB MEDIA/SCREEN/SCOPE timestamps for protocol were added at the postshoot stage. Please contact the postshoot note integrator (Sulakshana Karkala) for queries regarding lab media

- Isolation of Human Platelets from Whole Blood

Demonstrator: Zoi Laspa

  - To begin, prepare all the reagents required for the procedure [1].

    - WIDE: Talent arranging all the reagents on the bench.

  - Now, mix 10 milliliters of 10X Tyrode’s buffer with 90 milliliters of distilled water [1]. Add 0.1 gram of glucose [2] and adjust the pH of the full volume with HEPES to reach 7.4 [3]. First, set the pH of 10 milliliters of the buffer to 7.4 [4], and then adjust the pH of the remaining 90 milliliters with 1 normal hydrochloric acid to 6.5 [5].

    - Talent mixing 10 milliliters of 10X Tyrode’s buffer with 90 milliliters of distilled water.

    - Talent dissolving glucose into the buffer.

    - Talent adjusting the buffer pH using HEPES while monitoring with a pH meter.

    - Talent dividing the buffer into 2 containers.

Added shot 2.2.5: Talent adjusting the buffer pH using 1 N HCL.

  - For blood sample collection, prepare a 10-milliliter syringe for each donor with 2 milliliters of ACD anticoagulant [1]. Let the syringe equilibrate at room temperature [2-TXT].

    - Talent drawing 2 milliliters of ACD buffer into one 10 milliliter syringes.

    - Talent placing the syringes on a tablet on the bench. TXT: Alternatively, incubate at 37 °C

  - After collecting donor blood in a pre-warmed ACD syringe [1], slowly transfer the ACD-treated blood into a 15-milliliter reaction tube [2-TXT].

    - Talent picking up the syringe with blood.

    - Talent carefully transferring the blood from the syringe into a labeled 15 milliliter reaction tube. TXT: Use the blood immediately to avoid platelet pre-activation

  - Centrifuge the ACD-treated blood at 209 g for 20 minutes at room temperature without using the brake [1]. Meanwhile, prepare 25 milliliters of Tyrode's-HEPES buffer, pH 6.5, in a 50-milliliter reaction tube [2].

    - Talent placing the tube of ACD blood into the centrifuge, screws on cap and closing the lid.

    - Talent pouring 25 milliliters of Tyrode’s-HEPES buffer into a clean 50 milliliter tube and labeling it.

  - After centrifugation, gently transfer the upper platelet-rich plasma layer into the tube containing Tyrode’s-HEPES buffer [1].

    - Talent pipetting the upper layer of platelet-rich plasma and transferring it into the prepared tube.

  - To prevent contamination, leave approximately 1 milliliter of platelet-rich plasma above the buffy coat and erythrocyte layer [1].

    - Close-up of plasma left above the buffy coat.

  - Then, centrifuge the suspension at 430 g for 10 minutes at room temperature to pellet the platelets [1] and carefully resuspend the platelet pellet in 200 to 300 microliters of Tyrode’s buffer adjusted to pH 7.4 [2].

    - Talent placing the tube into the centrifuge and setting parameters.

    - Talent gently resuspending the visible platelet pellet in buffer using a pipette.

  - Measure the platelet count using a cell counter [1] before adjusting it to 200,000 platelets per microliter with Tyrode' s-HEPES buffer at pH 7.4 [2].

    - Talent loading a sample into a cell counter.

    - Talent pipetting Tyrode’s-HEPES buffer to platelets and mixing.

- Selection and Titration of Antibodies for Flow Cytometry

  - Adjust the platelet count to 1 × 10⁶ platelets per sample[1].

    - Talent picking up the sample tube from the bench. 
AUTHOR’S NOTE: This shot was not filmed. The platelet count was already adjusted in 2.9.2

  - For receptors that do not require activation prior to staining, including CD61, CD42b, CD41, CXCR7, and CXCR4, prepare one unstained and one stained sample per antibody [1]. For receptors or markers that require activation, such as PAC1, CD63, CD62P, Annexin V (annexin-5), and Zombie NIR, activate the sample with 10 micrograms per milliliter of CRP-XL and prepare both activated and non-activated samples [2].

    - Talent labeling tubes as “stained” and “unstained” samples.

    - Talent adding the platelets to the tubes.

  - To activate the platelets, incubate them with 10 micrograms per milliliter of CRP-XL for 30 minutes [1]. Then, incubate the isolated platelets with varying concentrations of the fluorochrome-conjugated antibody or dye for 30 minutes [2]. Then, add 300 microliters of Annexin Binding Buffer to each sample to dilute it [3].

    - Talent placing platelet samples aside on the bench.

    - Talent pipetting antibody to the sample tube.

    - Talent adding Annexin Binding Buffer to sample and inverting the tube.

  - Measure 10,000 events for each sample using a flow cytometer [1].

    - Talent loading sample into the flow cytometer.

  - Calculate the stain index for each antibody to determine the optimal separation between positive and negative populations [1].

    - SCREEN: 67878_3.5.1.mp4		00:00-00:20.

  - Plot the stain index against antibody concentrations to identify the optimal concentration for each antibody [1].

    - SCREEN: 67878_3.6.1.mp4		00:00-00:20
Video Editor: Please sequentially highlight each graph, if possible

- Staining for Flow Cytometry, Compensation Setup and Fluorescence Minus One Control (FMO) Measurement

  - For staining, dissolve hemin powder in 1.4 molar sodium hydroxide to create a 3-molar solution [1]. Heat the solution to 96 degrees Celsius for 5 minutes at 450 revolutions per minute [2]. Dilute it with distilled water to obtain a 3 millimolar stock solution [3].

    - Talent adding sodium hydroxide solution to hemin powder.

    - Talent placing the hemin solution on a heated shaker set to 96 degrees Celsius and 450 revolutions per minute.

    - Talent diluting the heated solution with distilled water.

  - After adjusting the platelet count to 1 million platelets per sample, activate the platelets as demonstrated earlier [1].

    - Talent placing the samples aside on the bench.

Added shot : 4.2.1 b: Talent placing the samples on the bench, pipetting the platelets (which were adjusted to 1 × 10⁶ platelets per sample) to the tubes and activates one sample with hemin. 
Note: use 4.2.1 B

  - Add 43.3 microliters of antibody cocktail to each sample [1] and incubate them for 30 minutes at room temperature in the dark [2].

    - Talent adding 43.3 microliters of antibody cocktail to tubes.

    - Talent covering the tubes with aluminum foil.

  - After incubation, dilute each sample with 300 microliters of 1X Annexin Binding Buffer composed of 10 millimolar HEPES-sodium hydroxide at pH 7.4 [1-TXT].

    - Talent pipetting Annexin Binding Buffer into each sample tube. TXT: The buffer contains 140 mM NaCl and 2.5 mM CaCl2

  - Measure each stained sample immediately using the flow cytometer [1].

    - Talent loading the prepared samples into the flow cytometer.

  - For compensation with beads, prepare 150 microliters of PBS for each fluorophore and add one drop of beads and 1 microliter of the fluorescence-conjugated antibody [1]. To compensate for Annexin V (annexin-5), use PE/Cy7 (P-E-C-Y-7) beads labeled with anti-human CD62L conjugated to the same fluorophore as Annexin V [2].

    - Talent preparing labeled bead samples in PBS with antibodies for each fluorophore, while adding anti-mouse IgG beads to the tube.
Added shot: 4.6.1.B : Show of all the antibodies

Videographer’s Note: Shot added to show all the antibodies

    - Talent labeling a tube as PE/Cy7 beads.

  - Use amine-reactive compensation beads for the amine-reactive dye by mixing two drops of positive beads and one drop of negative beads in 150 microliters of PBS with 1 microliter of dye [1]. Confirm that the positive beads react and display a signal and the negative beads do not [2].

    - Talent mixing amine-reactive dye compensation sample with both positive and negative beads.

    - SCREEN: 67878_4.7.2.mp4	00:00-00:20
Video Editor: Please highlight the top and bottom charts for positive beads and then do the same for the negative beads .

  - Prepare a fluorescence minus one control for every marker [1-TXT].

    - LAB MEDIA: Table 5.

# Results

- Results

  - The scatter plots showed a clear shift in the platelet population [1], with hemin forming more platelets with low forward scatter than other agonists [2].

    - LAB MEDIA: Figure 4A

    - LAB MEDIA: Figure 4A Video Editor: Highlight the graph for “hemin” at the extreme right

  - Treatment with ADP, thrombin, and CRP-XL resulted in 40 to 60% platelets being CD42b-positive [1], while hemin significantly reduced this population [2]. All platelet agonists had no effect on CD41 and CD61 surface expression [3].

    - LAB MEDIA: Figure 4B left graph. Video editor: Highlight the bars for ADP, thrombin, and CRP-XL showing similar mid-range levels.

    - LAB MEDIA: Figure 4B left graph. Video editor: Highlight the bar for “hemin”

    - LAB MEDIA: Figure 4C,D left panels in both C and D

  - Only hemin significantly increased the CXCR4 and CXCR7 surface expression [1]. In addition, the phosphatidylserine exposure was also enhanced under hemin treatment [2], resulting in an enhancement of Annexin V-positive cells [3].

    - LAB MEDIA: Figure 4H,I Video editor: Highlight the bars for “hemin” in the left graphs

    - LAB MEDIA: Figure 4J Video editor: Highlight the bar for “hemin” in the left graph

    - LAB MEDIA: Figure 4J  Video editor: Highlight the bar for “hemin” in the right graph

  - UMAP (u-map) and PhenoGraph analyses defined 27 distinct platelet clusters, which varied with agonist type [1]. Hemin caused the most dramatic shift in cluster patterns [2].

    - LAB MEDIA: Figure 5A.

    - LAB MEDIA: Figure 5A. Video editor: Emphasize the hemin cluster plot.

  - Resting platelet subpopulations were most preserved with ADP [1] and least with hemin [2].

    - LAB MEDIA: Figure 5B. Video editor: Show the bars for ADP corresponding to pg-08 and pg-13.

    - LAB MEDIA: Figure 5B. Video editor: Show the bars for ‘hemin’ corresponding to pg-08 and pg-13

# Pronunciation Guide:

# 1. Cytometry

# Pronunciation link:
https://www.merriam-webster.com/dictionary/cytometry
IPA: /saɪˈtɑː.mə.tri/
Phonetic Spelling: sigh-tah-muh-tree

# 2. Hemin

# Pronunciation link:
https://www.merriam-webster.com/dictionary/hemin
IPA: /ˈhiː.mɪn/
Phonetic Spelling: hee-min

# 3. Agonist

# Pronunciation link:
https://www.merriam-webster.com/dictionary/agonist
IPA: /ˈæ.ɡə.nɪst/
Phonetic Spelling: a-guh-nist

# 4. Collagen

# Pronunciation link:
https://www.merriam-webster.com/dictionary/collagen
IPA: /ˈkɑː.lə.dʒən/
Phonetic Spelling: kaa-luh-jen

# 5. Apoptotic

# Pronunciation link:
https://www.howtopronounce.com/apoptotic
IPA: /ˌeɪ.pɑːpˈtɑː.tɪk/ or /ˌæp.əpˈtɑː.tɪk/
Phonetic Spelling: ay-pop-tah-tik or ap-uhp-tah-tik

# 6. Tyrode’s (Buffer)

# Pronunciation link:
https://www.howtopronounce.com/tyrode
IPA: /ˈtaɪ.roʊdz/
Phonetic Spelling: tie-rohdz

# 7. HEPES

# Pronunciation link:
https://www.howtopronounce.com/hepes
IPA: /ˈhiː.pɛz/
Phonetic Spelling: hee-pez

# 8. CRP-XL

# Pronunciation link:
https://www.howtopronounce.com/crp-xl
IPA: /siː-ɑːr-piː ɛks-ɛl/
Phonetic Spelling: see-ar-pee eks-el

# 9. Annexin V

# Pronunciation link:
https://www.howtopronounce.com/annexin-v
IPA: /əˈnɛk.sɪn faɪv/
Phonetic Spelling: uh-nek-sin five

# 10. Phosphatidylserine

# Pronunciation link:
https://www.howtopronounce.com/phosphatidylserine
IPA: /ˌfɑːs.fəˌtaɪ.dɪlˈsɪəˌriːn/
Phonetic Spelling: foss-fuh-tie-dil-seer-een

# 11. Fluorochrome

# Pronunciation link:
https://www.howtopronounce.com/fluorochrome
IPA: /ˈflʊə.roʊˌkroʊm/
Phonetic Spelling: floo-roh-krohm

# 12. PE/Cy7

# Pronunciation link:
https://www.howtopronounce.com/pe-cy7
IPA: /piː-iː saɪ-sɛv.ən/
Phonetic Spelling: pee-ee sigh-seven

# 13. CXCR4

# Pronunciation link:
https://www.howtopronounce.com/cxcr4
IPA: /siː-ɛks-siː-ɑːr-fɔːr/
Phonetic Spelling: see-ex-see-ar-four

# 14. CXCR7

# Pronunciation link:
https://www.howtopronounce.com/cxcr7
IPA: /siː-ɛks-siː-ɑːr-sɛv.ən/
Phonetic Spelling: see-ex-see-ar-seven

# 15. UMAP

# Pronunciation link:
https://www.howtopronounce.com/umap
IPA: /ˈjuː.mæp/
Phonetic Spelling: yoo-map

# 16. PhenoGraph

# Pronunciation link:
https://www.howtopronounce.com/phenograph
IPA: /ˈfiː.noʊ.ɡræf/
Phonetic Spelling: fee-noh-graf