# Submission ID #: 67968

# Scriptwriter Name: Sulakshana Karkala

# Project Page Link: https://review.jove.com/account/file-uploader?src=20730288

# Title: Fluorescence-Based Detection of FEN1 Nuclease Activity and Screening of Small-Molecule Inhibitors

# Authors and Affiliations:

# Rongcheng Li1,*, Shui Yang2,*, Yu Sha2,3,*, Xiao Liu1, Jiani Ge1, Dandan Tang1, Rui Liang2, Song-Bai Liu1,2,3

# 1School of Chemistry and Life Sciences, Suzhou University of Science and Technology

# 2Jiangsu Province Engineering Research Center of Molecular Target Therapy and Companion Diagnostics in Oncology, Suzhou Vocational Health College

# 3Jiangsu Province Engineering Research Center of Development and Translation of Key Technologies for Chronic Disease Prevention and Control, Suzhou Vocational Health College

# *These authors contributed equally

# Corresponding Authors:

Song-Bai Liu		(liusongbai@126.com)

# Email Addresses for All Authors:

Rongcheng Li		(lirc97@163.com)

Shui Yang		(ys82@163.com)

Yu Sha			(shayu@wx.szhct.edu.cn)

Xiao Liu		(liuxiao202306@163.com)

Jiani Ge		(gggejian@163.com)

Dandan Tang		(tangdandan0314@163.com)

Rui Liang		(165678437@qq.com)

Song-Bai Liu		(liusongbai@126.com)

## Author Questionnaire

1. Microscopy: Does your protocol require the use of a dissecting or stereomicroscope for performing a complex dissection, microinjection technique, or something similar?  NO

2. Software: Does the part of your protocol being filmed include step-by-step descriptions of software usage?  NO

3. Filming location: Will the filming need to take place in multiple locations?   NO

Current Protocol Length

Number of Steps:  12

Number of Shots:  26

# Introduction

Videographer: Obtain headshots for all authors available at the filming location.

REQUIRED:

  - Rongcheng Li: We developed a fluorescence-based method to detect FEN1 nuclease activity and screen small-molecule inhibitors, advancing cancer diagnostics and targeted therapy research.

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B.roll:4.2

What are the current experimental challenges?

  - Jiahui Du: Current experimental challenges include the complexity and safety concerns of conventional methods, along with the high cost and specialized equipment required for existing fluorescence-based techniques.

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera.

What research gap are you addressing with your protocol?

  - Rongcheng Li: This protocol addresses the shortcomings of traditional radiological methods and high-cost fluorescence techniques, providing a safe, efficient and easy-to-use detection solution.

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera.

What research questions will your laboratory focus on in the future?

  - Songbai Liu: Our laboratory will focus on optimisatiob of a high-throughput screening platform to explore the in vivo applications and mechanisms of FEN1 inhibitors in cancer therapy.

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera.

Videographer: Obtain headshots for all authors available at the filming location.

# Testimonial Questions:

How do you think publishing with JoVE will enhance the visibility and impact of your research?

  - Rongcheng Li: The video demonstrates the fluorescent labelling experimental process, enhances the reproducibility of the method, attracts the attention of cross-field researchers, and expands the scope of application of the technology.

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera.

Can you share a specific success story or benefit you’ve experienced—or expect to experience—after using or publishing with JoVE?

  - Rongcheng Li: It is expected that the experimental videos will reduce laboratory training time, decrease operational error rates, promote academic collaboration, and enhance paper citations due to method transparency.

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera.

Videographer: Please capture the testimonials in both Chinese and in English

# Protocol

- Preparation and Analysis of TAMRA-labeled DNA Substrate for FEN1 Nuclease Activity Assay

Demonstrator: Rongcheng Li

  - To begin, DNA oligonucleotide strands based on the specific cleavage activity of FEN1 (fenn-one) [1]. Mix the single-stranded DNA oligonucleotides in an annealing buffer [2-TXT].

    - WIDE: Talent looking at DNA oligonucleotide strand design on a computer.

    - Talent pipetting DNA oligos into tubes containing annealing buffer. TXT: 5 μL D1 (10 pmol/μL), 10 μL T1 (10 pmol/μL), and 7.5 μL U2 (10 pmol/μL)

  - Add 10 microliters of annealing buffer and deionised water to make a total volume of 50 microliters.[1]. Then heat the reaction mixture at 100 degrees Celsius for 1 minute in a metal bath [2].

    - Talent adding annealing buffer and deionized water.

    - Talent placing the tube into a metal bath preheated to 100 degrees Celsius.

  - Immediately transfer the tube to 72 degrees Celsius and incubate for 10 minutes [1]. Then turn off the heat source [2] and allow the solution to gradually cool to room temperature to form the complete double-stranded DNA substrate [3].

    - Talent transferring the tube to another metal bath set at 72 degrees Celsius.

    - Talent turning off the heat source.

    - Talent leaving the tube on the bench to cool naturally.

  - For the FEN1 nuclease activity assay, first combine 2 microliters of the DNA substrate with FEN1 nuclease in the reaction buffer [1-TXT]. Incubate the reaction mixture in a metal bath at 37 degrees Celsius for 15 minutes [2].

    - Talent pipetting DNA substrate and FEN1 nuclease into a new microcentrifuge tube. TXT: Final volume: 20 µL

    - Talent placing the tube into a metal bath set at 37 degrees Celsius.

  - Add 20 microliters of 2x termination buffer to stop the reaction [1]. Then load 20 microliters of the terminated sample onto a pre-prepared 12% denaturing polyacrylamide gel [2].

    - Talent pipetting 2x termination buffer into the reaction tube.

    - Talent loading the sample into wells of the denaturing polyacrylamide gel.

  - Electrophorese the gel at 100 volts for 60 minutes [1]. Once complete, visualize the electrophoresis results under a fluorescence imaging system [2].

    - Talent starting the gel electrophoresis and setting the voltage.

    - Talent placing the gel in a fluorescence imaging system.

- Screening of Small Molecule Inhibitors of FEN1 Nuclease

  - With a pipette, transfer 2 microliters of dimethyl sulfoxide into a 1.5-milliliter microcentrifuge tube [1]. Add 2 microliters of FEN1 nuclease [2] and 10 microliters of 2X reaction buffer to the tube [3] and 6 microliters of deionized water [4].

    - Talent pipetting dimethyl sulfoxide into a labeled 1.5 milliliter microcentrifuge tube. Videographer’s NOTE: Clip C2159 captures step 3.1.1 - 3.1.4 all together.

Videographer’s NOTE: Clip C2157 is the mixed tube from step 3.1.

    - Talent adding FEN1 nuclease.

    - Talent adding 2x reaction buffer into the same tube.

    - Talent adding microliters of deionized water into the same tube.

  - Mix the solution thoroughly [1]. Then incubate it on ice for 10 minutes [2]. After incubation, pipette 2 microliters of DNA substrate to reach a final reaction volume of 20 microliters [3]. Videographer’s NOTE: Steps 3.2 and 3.5 are repetitive so only filmed with the author once.

    - Talent gently vortexing or flicking the tube to mix.

    - Shot of the tube being placed on ice.

    - Talent adding DNA substrate to the tube.

  - Next, prepare a working dilution of the FEN1 nuclease inhibitor, FEN1-IN-4 (Fen-one-in-Four), by performing a two-fold serial dilution from a 1 molar stock solution in dimethyl sulfoxide [1].

    - Talent using pipette to perform sequential two-fold serial dilutions of the FEN1-IN-4 stock in multiple tubes. Videographer’s NOTE: C2158 is the mixed tube from step 3.3

  - Pipette 2 microliters each of FEN1 nuclease and the diluted FEN1-IN-4 inhibitor into a new 1.5-milliliter microcentrifuge tube [1]. Then pipette 10 microliters of 2x reaction buffer and 4 microliters of deionized water [2].

    - Talent pipetting FEN1 nuclease and diluted inhibitor into a fresh microcentrifuge tube. Videographer’s NOTE: Clip C2156 captures step 3.4.1 and 3.4.2, but was labeled wrong.

    - Talent adding reaction buffer and deionized water into the same tube.

  - Mix gently and incubate the mixture on ice for 10 minutes [1]. Now add 2 microliters of DNA substrate to the tube to make the final volume to 20 microliters [2]. Videographer’s NOTE: Steps 3.2 and 3.5 are repetitive so only filmed with the author once.

    - Talent gently mixing and placing the tube on ice.

    - Talent adding DNA substrate to the reaction tube.

  - Incubate the reaction mixture at 37 degrees Celsius in a metal bath for 15 minutes [1]. Then terminate the reaction by adding 20 microliters of termination buffer [2].

    - Talent placing the tube into a 37 degree Celsius metal bath.

    - Talent adding 2× termination buffer into the tube after incubation. Videographer’s NOTE: Clip C2167 captures step 3.6.2, but was labeled wrong.

# Results

- Results

  - A clear protein band at approximately 45 kilodaltons confirmed successful purification of the FEN1 nuclease from induced expression [1].

    - LAB MEDIA: Figure 2A. Video editor: Please highlight the blue band in lane 6 around the 45 kDa marker.

  - The fluorescein-labeled double-stranded DNA substrate appeared at 80 kiloDaltons, confirming successful synthesis [1] compared to the single-stranded substrate at 59 kiloDaltons [2].

    - LAB MEDIA: Figure 2B. Video editor: Highlight lane 2 showing the lower band at 59 nt labeled D1.

    - LAB MEDIA: Figure 2B. Video editor: Highlight lane 3 showing the higher band at 80 nt labeled SF.

  - FEN1 nuclease cleaved the 80-nucleotide double-stranded DNA substrate in a dose-dependent manner [1], with complete cleavage observed at 0.5 micrograms per microliter [2].

    - LAB MEDIA: Figure 2C. Video editor: Please highlight lane 3 showing intact 80-nt substrate in the SF control.

    - LAB MEDIA: Figure 2C. Video editor: Please highlight lane 5

  - The FEN1-IN-4 compound significantly inhibited the cleavage of double-stranded DNA substrate by FEN1, as shown by the retention of the 80-nucleotide band [1].

    - LAB MEDIA: Figure 2D. Video editor: Please highlight lane 4

  - FEN1-IN-4 reduced FEN1 nuclease activity in a concentration-dependent manner, with a half-maximal inhibitory concentration of 14.03 micromolar [1].

    - LAB MEDIA: Figure 2F. Video editor: Focus on the downward slope of the curve

Pronunciation Guides:

1. FEN1

- Pronunciation link:
https://www.howtopronounce.com/fen1

- IPA: /fɛn wʌn/

- Phonetic Spelling: fen-wuhn

2. Oligonucleotide

- Pronunciation link:
https://www.merriam-webster.com/dictionary/oligonucleotide

- IPA: /ˌɑː.lɪ.ɡoʊˈnuː.kliːˌtaɪd/

- Phonetic Spelling: ah-li-goh-noo-klee-tyd

3. Dimethyl sulfoxide

- Pronunciation link:
https://www.merriam-webster.com/dictionary/dimethyl%20sulfoxide

- IPA: /ˌdaɪˌmɛθəl ˈsʌlfˌoʊˌsaɪd/

- Phonetic Spelling: dye-meth-uhl sull-fox-ide

4. Annealing

- Pronunciation link:
https://www.merriam-webster.com/dictionary/annealing

- IPA: /əˈniː.lɪŋ/

- Phonetic Spelling: uh-nee-ling

5. Polyacrylamide

- Pronunciation link:
https://www.howtopronounce.com/polyacrylamide

- IPA: /ˌpɑː.li.əˈkrɪ.ləˌmaɪd/

- Phonetic Spelling: pah-lee-uh-kril-uh-mide

6. Electrophorese

- Pronunciation link:
https://www.howtopronounce.com/electrophorese

- IPA: /ɪˌlɛk.trə.fəˈriːz/

- Phonetic Spelling: ih-lek-troh-fuh-reez