# Submission ID #: 67971

# Scriptwriter Name: Poornima G

# Project Page Link: https://review.jove.com/account/file-uploader?src=20731203

# Title: Parameterizing V-notch Weir Equations for Flow Monitoring in a Drainage Control Structure

# Authors and Affiliations:

# Sheela Katuwal1, Andrew J. Craig2, Andrew W. Rupiper3, Natalia P. Rogovska1, Gabriel M. Johnson3, Thomas M. Isenhart3, Rob W. Malone1

# 1National Laboratory for Agriculture and the Environment, USDA-ARS, N. University Boulevard

# 2Department of Agricultural and Biosystems Engineering, Iowa State University

# 3Natural Resource Ecology and Management, Iowa State University

# Corresponding Authors:

# Natalia P. Rogovska 		natalia.rogovska@usda.gov

# Email Addresses for All Authors:

Sheela Katuwal		sheela.katuwal@usda.gov

Andrew J. Craig		ajcraig@iastate.edu

Andrew W. Rupiper		arupiper@iastate.edu

Gabriel M. Johnson		gjohnson@iastate.edu

Thomas M. Isenhart		isenhart@iastate.edu

# Rob W. Malone		rob.malone@usda.gov

# Natalia P. Rogovska 		natalia.rogovska@usda.gov

## Author Questionnaire

1. Microscopy: Does your protocol require the use of a dissecting or stereomicroscope for performing a complex dissection, microinjection technique, or something similar?  No

2. Software: Does the part of your protocol being filmed include step-by-step descriptions of software usage?  Yes, all done

3. Filming location: Will the filming need to take place in multiple locations?   No

Current Protocol Length

Number of Steps:  18

Number of Shots:  41 (18 SC)

# Introduction

Videographer: Obtain headshots for all authors available at the filming location.

REQUIRED: What is the scope of your research? What questions are you trying to answer?

  - Sheela Katuwal: We developed a user-friendly online tool that can be used by drainage water management professionals to accurately estimate drainage volume across flow conditions and sizes of water control structures.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: LAB MEDIA: Figure_extra1.tif

What research gap are you addressing with your protocol?

  - Sheela Katuwal: As more edge-of-field and drainage water management conservation practices are designed and implemented, we are attempting to harmonize this data to make comparisons and track performance of these practices.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: LAB MEDIA: Video_extra.MP4

What advantage does your protocol offer compared to other techniques?

  - Andrew W.  Rupiper: Our protocol provides an easy-to-use online tool that should allow other researchers to quickly understand the flow regime and how to calculate the flow rate in their system.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: 2.6.1

How will your findings advance research in your field?

  - Andrew W. Rupiper: Our findings will improve our ability to understand the performance of systems that require monitoring of flow. This work will also improve tracking loss of nutrients in drainage water.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: 3.5.1

What research questions will your laboratory focus on in the future?

  - Andrew W. Rupiper: Our results provide a uniform method for monitoring drainage flows by researchers and agencies. Future efforts will include different sizes and configurations of control structures to ensure the accuracy of the tool across applications.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: 3.2.2

Videographer: Obtain headshots for all authors available at the filming location.

# Protocol

- Setting Up the Drainage Control Structure for a Calibration Experiment

Demonstrator: Andrew J. Rupiper

  - To begin, connect the inlet section of the control structure to the pump assembly, which consists of a pump, a flow meter, and the necessary pipes, fittings, and valves [1]. Then, connect the outlet section of the control structure to the drainage pipe [2].

    - WIDE: Talent connecting the inlet section of the control structure to the pump assembly.

    - Talent securing the connections between the outlet section of the control structure and the drainage pipe.

  - Measure the top width and depth of the V-notch weir using a measuring tape [1], followed by the distance between the tracks where the stoplogs and V-notch weir fit in the drainage control structure [2].

    - Talent measuring the top width of the V-notch weir.  Added shot: Talent measuring the depth of the V-notch weir.

    - Talent using a measuring tape to measure the width between the tracks.

  - Now, insert the stoplogs between the tracks one at a time [1].  Stack additional stoplogs above the existing ones in the control structure until the desired height is reached [2] and place the weir plate firmly above the top stoplog [3].

    - Talent placing a stoplog into the track.

    - Talent stacking more stoplogs above the existing ones.

    - Talent positioning the weir plate on top of the highest stoplog.

  - Now, measure the inner height of the control structure [1] and the distance between the top of the control structure and the top of the V-notch weir using a measuring tape [2-TXT].

    - Talent measuring the inner height of the control structure.

    - Talent measuring the distance between the top of the structure and the top of the V-notch weir.  TXT: Ensure that the zero-marking on the tape aligns with the edge of the structure

  - Calculate the height of the V-notch crest from the bottom of the control structure using the equation [1].

    - SCREEN: 67971_screenshot_1.mp4 00:00-00:10.

  - Then, firmly connect the manometer tube or stilling well to the control structure [1]. Affix a measuring tape to the outside wall of the control structure next to the manometer with the zero value corresponding to the base of the interior of the control structure [2].

    - Talent securing the manometer tube to the structure.

    - Talent aligning and fixing the measuring tape to the exterior of the structure.

- Measuring Flow Rates and Head

  - Start the pump and allow water to fill the inlet chamber, ensuring that water flows through the weir [1].  Adjust the valve in the pump assembly to select a flow rate of approximately 15 to 25 gallons per minute, monitoring the display on the flow meter [2].

    - Talent turning on the pump and water filling the chamber.

    - Talent adjusting the flow rate valve.

  - Allow the flow to stabilize for at least 3 minutes [1] and increase the flow rate if the head is less than 2 inches [2].

    - Shot of display showing stabilized flow.

    - Talent adjusting the valve to increase the flow rate.

  - Once the flow is stable, record the flow rate reading from the flow meter [1]. Take 8 to 10 consecutive readings within 2 minutes and calculate the average flow rate [2].

    - Talent noting down the flow rate from the flow meter display.

    - SCREEN: 67971_screenshot_2.mp4.

  - Now, measure the height of water in the stilling well or manometer tube [1]. If the water level fluctuates, record the high and low readings over 30 seconds and determine the average height [2]. Then, calculate the head using the equation [3].

    - Talent reading the water level in the stilling well.

    - Talent entering data in a computer.

    - SCREEN:  67971_screenshot_3.mp4 00:10-00:17

  - Increase the flow rate by approximately 15 to 20 gallons per minute and wait for it to stabilize to obtain the flow rate and corresponding head [1]. Increase the flow rate at least five more times until the water level reaches near the top of the V-notch [2].

    - Talent adjusting the flow rate and waiting for stabilization. (This step was shot in 3 parts: 3.5.1-A. Shot of previously stabilized flow rate, 3.5.1-B. Talent adjusting the flow rate, and 3.5.1-C. Shot of the stabilized flow rate after adjustment).

    - Shot of the water level near the top of the V-notch.

- Developing the Weir Calibration Equation for Flows Contained Within the V-Notch

  - Enter the values of the head and corresponding flow rates into an Excel spreadsheet [1].

    - SCREEN: 67971_screenshot_4.mp4 00:00-00:09.

  - To plot the measured flow rates and the head, select the data range, then click Insert followed by Charts and Scatter [1].  To fit the power function, right-click on the data points in the chart and select Add Trendline followed by Power, Display Equation on Chart and Display R-squared Value on Chart [2].

    - SCREEN: 67971_screenshot_4.mp4. 00:09-00:18.

    - SCREEN: 67971_screenshot_4.mp4. 00:18-00:22.

- Weir Flow Equation Coefficients for Overtopping Flows Using Own Calibration Equation

  - Click the Weir Flow Equation Coefficients Calculator on the webpage [1]. Select the unit between Metric (SI) (metric S-I) and US Customary units using the radio buttons [2].

    - SCREEN: 67971_screenshot_5.mp4. 00:00-00:07  Video editor: Please highlight the webpage address https://www.ars.usda.gov/midwest-area/ames/nlae/docs/tools-available-from-nlae/

    - SCREEN: 67971_screenshot_5.mp4.00:07-00:10 .

  - In the Size of control structure section, keep the default selection of 6-inch (15-centimeter) as the user provides their own weir equation coefficients [1]. Click Yes in response to the prompt “Do you have your own calibration equation?” [2] and enter the values of parameters a and b corresponding to the equation for flows contained within the V-notch [3].

    - SCREEN: 67971_screenshot_5.mp4. 00:10-00:12.

    - SCREEN: 67971_screenshot_5.mp4. 00:12-00:14

    - SCREEN: 67971_screenshot_5.mp4. 00:14-00:24 .

  - Enter the values of the weir specifications in the Inputs section of the tool [1]. Retrieve the values of coefficients for overtopping flows from the Coefficients for V-notch weir equation table and the plot of flow rate against the head from the Results section of the tool [2].

    - SCREEN: 67971_screenshot_5.mp4. 00:25-00:35.

    - SCREEN: 67971_screenshot_5.mp4. 00:36-00:40 .

- Weir Flow Equation Coefficients for Flows Contained Within the V-Notch and Overtopping Flows Using the Tool

  - Open the tool as described earlier and select the unit and size of the control structure using the radio buttons [1]. Click No in response to “Do you have your own calibration equation?” [2].

    - SCREEN: 67971_screenshot_6.mp4 00:04-00:09.

    - SCREEN: 67971_screenshot_6.mp4 00:09-00:11

  - Enter the values of the weir specifications [1]. Retrieve the values of coefficients for flows with the V-notch weir and overtopping flows from the table [2] and the plot of flow rate vs. (versus) flow depth relative to the V-notch crest in the Results section of the tool [3].

    - SCREEN: 67971_screenshot_6.mp4.  00:12-00:27

    - SCREEN: 67971_screenshot_6.mp4. 00:28-00:30

    - SCREEN: 67971_screenshot_6.mp4. 00:30-00:32

  - Use of weir coefficients provided by the tool for a downstream weir in a multiple-chambered control structure may result in incorrect flow rates at high heads because of increased flow velocities [1].

    - LAB MEDIA: 67971_screenshot_7.mp4

# Results

- Representative Results

  - The calibration of a stainless-steel 45-degree V-notch weir in 15, 20 and 30 centimeters Agri Drain control structures demonstrated that flow rates within the V-notch could be estimated with high accuracy using a dedicated weir equation with root mean square error less than 0.20 liters per second and percentage bias less than 1 [1].

    - LAB MEDIA: Figure 9A-C. Video editor: Highlight the blue data points and line graph

  - The estimated flow rates for overtopping flows, using weir equation coefficients from the Weir Flow Equation Coefficient Calculator tool, were similar to the measured values [1].

    - LAB MEDIA: Figure 9A-C. Video editor: Highlight the yellow data points and line graph .