Submission ID #: 68031

Scriptwriter Name: Poornima G

Project page Link: https://review.jove.com/account/file-uploader?src=20749503

Title: Wireless High-Resolution Surface Electromyography Recording for Facial Muscle Activity Measurement

Q: Filming location: Will the filming need to take place in multiple locations?   NO

Current Protocol Length

Number of Steps:  12

Number of Shots:  35 (19 SC)

# Protocol

AUTHORS: Your protocol demonstrates stepwise software usage. So, we will need you to record the shots labeled as SCREEN using screen recording software. If you use a Mac, QuickTime X also has the ability to record the steps. Please create screen capture videos, a screenshot summary, and upload the files to your project page: https://review.jove.com/account/file-uploader?src=20749503

- Protocol

  - To begin,  carefully remove electrode mask from the case and inspect its shape to ensure a proper fit on the face. [1].

    - WIDE: Talent preparing the mask, checking fit to face

    - 2.1.1. Preparing the mask – CLOSE-UP

  - Place electrode number 11, marked with an eye symbol, near the participant's right eye [1]. Position electrode number 8, marked with a nose symbol, on the nose [2]. Attach electrodes 13 to 16 on the  forehead [3]. Attach  1-2 electrodes on the chin [4]. Now place the reference electrode on the right mastoid [5].

    - Talent positioning electrode 11 near the right eye.

    - Talent placing electrode 8 on the nose.

    - Talent attaching electrodes 13–16 on the forehead.

3.2.3. attaching electrodes 1-2 on the chin

    - Talent positioning the reference electrode on the right mastoid.

3.2.4. additional CLOSE-UP positioning the reference electrode on the right mastoid.

  - Gently press the mask onto the skin [1] and carefully peel off the outer transparent support films one by one to fully attach the mask [2]. Ensure the mask remains in place throughout the process [3].

    - Talent pressing the mask onto the participant's face.

    - Talent peeling off the transparent support films.

2.3.2. peeling off support film from the bottom of the face

3.3.2. peeling of support film from the behind the ear.

    - 2.3.3. SPIN:Close-up of the mask fully attached to the face

  - Remove protective film [1] and insert the electrode array connector card into the DAU's connector socket [2], and secure the DAU (D-A-U) to the tape [3].

    - Talent removing protective film from DAU tape.

    - 1.4.1. Talent inserting the electrode array connector card into the DAU socket.

    - 1.4.1. Talent securing the DAU on tape.

  - Seat the participant comfortably in an upright chair at a fixed distance, such as 70 centimeters, from the computer screen [1].

    - 2.5.1. Participant sitting in the chair at the correct distance from the screen.

  - Verify that all 16 electromyography electrode signals are detected in the application on the tablet [1]. Make sure that prior steps were performed: In the electrodes application, select the DAU ID number and click Confirm to pair the tablet and DAU [2]. Choose electrode mask montage and click Confirm [3]. Now, select the channels to display on the screen with desired filters and click Confirm [4]. After selecting the channels, connect the tablet to the computer using a cable [5] and change the Android tablet universal serial bus (USB) settings from transferring files to USB tethering.

    - SCREEN: Display of the tablet interface showing all 16 EMG electrode signals detected. 1.6.1.mkv 00:00-00:15

    - SCREEN: Opening the electrodes application, selecting the DAU by ID number and clicking Confirm. 1.6.2. mkv 00:01-00:14

2.6.2. WIDE: talent choosing electrode montage on the tablet, checking the channels.

    - SCREEN: Choosing electrode mask montage and clicking Confirm. 1.6.3.mkv 00:02-00:05

    - SCREEN: Selecting channels (1–16) to display on the screen with desired filters (changing EEG to EMG) and clicking Confirm. 1.6.3.mkv 00:05-00:06 and 1.6.4.mkv 00:00-00:20

    - Talent connecting the tablet to the computer using a cable.

    - SCREEN: Talent changing USB setting from transferring files to USB tethering. 1.6.6.mkv 00:06-00:10

  - Open the electrodes application on the Android tablet and press Start session to begin EMG recording [1]. Enter the session details, including Study name, Session name, Site, Operator, Patient ID, Age, Gender, and Remarks [2]. Click Start Recording to capture EMG data [3].

    - SCREEN: Talent pressing Start session on the tablet. 1.7.mkv 00:00-00:02

    - SCREEN: Entering session details in the application. Video editor: Please blur the patient’s personal data while assembling the video after the shoot 1.7.mkv 00:02-00:47 (can fast forward the typing or use already typed information video)

    - SCREEN: Clicking Start Recording. 1.7.mkv 00:47-00:55

  - If calibration is required, the application will prompt the participant to perform basic tasks such as closing eyes or chewing [1]. Click Start to initiate calibration [2]. If recalibration is needed, repeat the process [3]. Once completed, click OK to proceed or click Next to skip [4]. Once calibration completed, click Done  and OK[5]

    - SCREEN: Calibration instructions displayed on the tablet. 1.8.mkv 00:00-00:01 Video editor: Please emphasize the instruction area

    - SCREEN: Talent clicking Start for calibration. 1.8.mkv 00:00-00:30

    - WIDE: Shot of the participant performing calibration tasks.

2.8.3. WIDE/different angle: Shot of the participant performing calibration tasks.

2.8.4. CLOSE-UP: Shot of the participant performing calibration tasks.

    - SCREEN: Clicking OK or Next to proceed. 1.8.mkv 01:37-01:40

    - SCREEN: Clicking Done 1.8.mkv 01:55-02:00

  - On the computer, double-click the experiment file facial_expressions.psyexp (facial-expressions-dot-Psycho-Pie) [1]. Close unnecessary pop up windows [1.2]. Click Run experiment to launch the experiment [2]. When prompted, enter session details such as participant number, session number, and tablet name, then click OK [3]. The session will start by instructions displayed on the computer monitor [4] and annotations displayed on the tablet in events logs[5].

    - SCREEN: Double-clicking the experiment file facial_expressions.psyexp . Video editor: Please blur the file path. 1.9.mkv 00:00-00:10

      - 1.2. Close “readme” and “tip of the Day” windows  1.9.mkv 00:10-00:13

    - SCREEN: Clicking Run experiment. 1.9.mkv 00:13-00:19

    - SCREEN: Entering session details and confirming. 1.9.mkv 00:19-00:32

    - SCREEN: Display of Experiment start on PC

3.9.4. WIDE: experiment start displayed on the screen with participant viewing the screen.

    - SCREEN: Display of beginning of annotations on the Tablet. 1.9.5.nkv 00:21-00:26

  - Have the participant perform 11 facial expressions as instructed by the video guide [1]. After holding each expression for five seconds, give a three-second rest [2]. Observe and guide the participant as needed for accuracy and synchronization with instructions [3].

    - Shot of the participant performing facial expressions. TXT: Repeat each expression 3x consecutively

2.10.1. Different angle: participant performing facial expressions

    - Shot of the participant in the resting position.

    - Talent instructing the participant.

  - Once the experiment is complete, PsychoPy (Psycho-Pie) will automatically stop the session [1]. On the tablet, click END recording to stop EMG data capture [2]. Now, click Yes, stop [3]. A confirmation window will appear, then click OK [4].

    - SCREEN: PsychoPy stopping the experiment automatically.

    - SCREEN: Clicking END recording on the tablet. 1.11.mkv 00:00-00:03

    - SCREEN: Clicking Yes, stop. 1.11.mkv 00:03-00:08

    - SCREEN: Confirmation prompt appearing and clicking OK to finalize the session. 1.11.mkv 00:08-00:13

  - Press the Sync button to synchronize session data with the system's cloud [1]. Once completed press OK [2], quit the application [3-TXT].

    - SCREEN: Clicking Sync to upload session data. 1.12.mkv 00:00-00:10

    - SCREEN: Clicking OK 1.12.mkv 00:10-00:14

    - SCREEN: Talent quitting the application. 1.12.3mkv 00:00-04 TXT: Perform EMG data analysis