Submission ID #: 68036

Scriptwriter Name: Debopriya Sadhukhan

Project Page Link: https://review.jove.com/account/file-uploader?src=20751028

Title: Advanced Glycation End Products Sensitize Human Sensory-Like Neuron Cells to Capsaicin-Induced Calcium Influx

Authors and Affiliations:

Gessica Sabrina de Assis Silva1*; Michelle Cristiane Bufalo2,3*, Marcelo Medina de Souza2*, Carlos DeOcesano-Pereira2, Ana Marisa Chudzinski-Tavassi2,3, Vanessa Olzon Zambelli1,2

1Laboratory of Pain and Signaling, Butantan Institute

2Center of Excellence in New Target Discovery, Butantan Institute

3Innovation and Development Laboratory, Innovation and Development Center, Butantan Institute

*These authors contributed equally

Corresponding Authors:

Vanessa Olzon Zambelli	(vanessa.zambelli@butantan.gov.br)

Email Addresses for All Authors:

Gessica Sabrina de Assis Silva            (g.sasilva.proppg@proppg.butantan.gov.br)

Michelle Cristiane Bufalo	             (michelle.bufalo@butantan.gov.br)

Marcelo Medina de Souza	             (marcelo.souza@butantan.gov.br)

Carlos DeOcesano-Pereira		 (carlos.ocesano@butantan.gov.br)

Ana Marisa Chudzinski-Tavassi	 (ana.chudzinski@butantan.gov.br)

## Author Questionnaire

1. Microscopy: Does your protocol require the use of a dissecting or stereomicroscope for performing a complex dissection, microinjection technique, or something similar?

A: Yes

A: We are able to record movies.

2. Software: Does the part of your protocol being filmed include step-by-step descriptions of software usage?

A: Yes

3. Filming location: Will the filming need to take place in multiple locations?

A: No

Current Protocol Length

Number of Steps:  18

Number of Shots:  30

# Introduction

Videographer: Obtain headshots for all authors available at the filming location.

REQUIRED:

  - Gessica S. A Silva: Increased collagen-derived advanced glycation end products, or AGEs, are consistently linked to painful diseases. This research investigates whether the glycation process sensitizes sensory neurons to capsaicin excitation.

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: LAB MEDIA: Figure 3.

What significant findings have you established in your field?

  - Michelle C. Bufalo: Our results show that collagen glycation increases calcium influx compared to cells treated with normal collagen, suggesting that sensory-like neurons express functional TRPV1 channels and that glycation increases capsaicin excitation.

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: LAB MEDIA: Figure 5B, 5C, 5D, Figure 4.

How will your findings advance research in your field?

  - Marcelo M. de Souza: This research demonstrates that mimicking a pro-nociceptive environment with glycated collagen leads to up-regulation of capsaicin-induced calcium influx, confirming the functionality of TRPV1 channels and suggesting that AGEs may sensitize these channels.

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: LAB MEDIA: Figure 4.

What research questions will your laboratory focus on in the future?

  - Michelle Cristiane Bufalo: Since glycation of collagen occurs with aging and in pathological conditions, this approach may serve as a valuable tool for investigating new molecular targets for treating various degenerative diseases. NOTE: Michelle Cristiane Bufalo delivered this statement.

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera.

Videographer: Obtain headshots for all authors available at the filming location.

# Protocol

- Calcium Influx Imaging

Demonstrator: Marcelo Medina and Michelle Cristiane Bufalo

  - To begin, prepare 1 milliliter of Fluo-8 dye-loading solution by adding 2 microliters of Fluo-8 stock solution to 998 microliters of assay buffer [1-TXT].

    - Talent pipetting 2 microliters of Fluo-8 stock solution into a tube containing 998 microliters of assay buffer. TXT: Assay buffer: HBSS buffer and 10x Pluronic F127 Plus.

  - Pipette 250 microliters of this solution into each plated dish containing sensory-like neuron cells that are previously differentiated and treated with glycated or non-glycated extracellular collagen matrix [1]. Incubate for 30 minutes in a humidified atmosphere of 5% carbon dioxide at 37 degrees Celsius [2].

    - Talent pipetting 250 microliters of prepared Fluo-8 dye-loading solution into a plated dish with sensory-like neuron cells.

    - Talent placing the dish inside the incubator.

  - After 30 minutes, remove the dish from the incubator [1] and keep it at room temperature in the dark for another 30 minutes [2].

    - Talent removing the dish from the incubator.

    - Talent placing the dish on the lab bench in a dark area.

  - For calcium influx imaging, prepare a syringe with 250 microliters of 2-micromolar capsaicin solution [1]. Connect the syringe to a sterile 23-gauge Butterfly scalp vein set [2]. NOTE: The VO has been edited.

    - Talent filling a syringe with 250 microliters of 2 micromolar capsaicin solution.

    - Talent attaching the syringe to a Butterfly type 23 gauge scalp vein set.

    - Talent securing the syringe onto a lab stand using a clamp. NOTE: This shot was not filmed.

  - With extreme caution, place the scalp needle into the Petri dish positioned on the confocal microscope stage without touching the bottom [1]. NOTE: The VO has been edited.

    - Talent placing the Petri dish on the microscope stage. NOTE: This shot was not filmed.

    - SCOPE: Talent carefully positioning the scalp needle into the Petri dish without making contact with the bottom. NOTE: The videographer filmed this shot.

  - Identify a field containing more than 20 cells [1] and adjust the focus using bright field light [1].

    - SCREEN: 68036_Screenshot_2.6.1 and 2.6.2: 00:01-00:18

    - SCREEN: 68036_Screenshot_2.6.1 and 2.6.2: 00:25-00:33

  - Start Live mode with a 488-nanometer laser [1] and adjust the focus and illumination parameters [2].

    - SCREEN: 68036_Screenshot_1.mp4 00:03-00:10.

    - SCREEN: 68036_Screenshot_1.mp4 00:11-00:20.

  - Start the acquisition recording [1]. After the baseline period of t = 0 to t = 60 seconds, gently push the syringe plunger to release 250 microliters of capsaicin into the Petri dish [2]. NOTE: The VO has been edited.

    - SCREEN: 68036_Screenshot_1.mp4 00:21-00:30.

    - Talent applying 250 microliters of 2-micromolar capsaicin. NOTE: This shot was not filmed, because item 2.8.2 is similar to the item 2.8.3.

    - Talent pushing the syringe plunger gently to release 250 microliters of capsaicin into the Petri dish. NOTE: This shot was modified during the shoot.

- Post-Processing and Data Analysis

  - Open the microscopy software and select Quantification mode to perform the cell analysis [1]. Analyze fluorescence spike cells before and after the capsaicin stimulus [2].

    - SCREEN: 68036_Screenshot_2.mp4 00:03-00:05.

    - SCREEN: 68036_Screenshot_2.mp4 00:06-00:09.

  - Create a Region of Interest or ROI (R-O-I) in each responsive cell, ensuring that the entire cell area is included [1-TXT].

    - SCREEN: 68036_Screenshot_2.mp4 00:15-00:29, 00:52-01:05. TXT: Responsive cells: Cells that display a fluorescence spike after capsaicin Video Editor: Speed up the video as needed.

  - Export the data as a CSV (C-S-V) file for further analysis [1].

    - SCREEN: 68036_Screenshot_2.mp4 01:10-01:19.

  - Alternatively, perform the analysis using the freely available FIJI (Fiji) software [1].

    - SCREEN: To be provided by authors: Opening FIJI software and displaying the home interface. NOTE: Removed this as OBS studio window is visible here.

  - To perform the analysis with FIJI (Fiji) software, open the image files by selecting File, followed by Import, Bio-Formats, and then choose the relevant file [1].

    - SCREEN: 68036_Screenshot_3.mp4 00:02-00:16.

  - Use the Magnifying Glass tool to zoom in [1] and the Freehand Selection tool to draw an ROI around the responsive cells [2].

    - SCREEN: 68036_Screenshot_3.mp4 00:17-00:20.

    - SCREEN: 68036_Screenshot_3.mp4 00:21-00:33.

  - Press the T key to add it to the ROI Manager [1-TXT]. NOTE: The VO has been edited.

    - SCREEN: 68036_Screenshot_3.mp4 00:33-00:35. TXT: Repeat this for all cells of interest

  - Then, go to the main menu and choose Analyze. Select Mean Gray Value and click OK [1]. Go to the ROI Manager window and select all ROIs [2]. NOTE: The VO has been edited.

    - SCREEN: 68036_Screenshot_3.mp4 00:56-01:08.

    - SCREEN: 68036_Screenshot_3.mp4 01:08-01:12.

  - Select More, followed by Multi Measure. A new window displaying fluorescence intensity measurements across multiple frames will appear [1]. NOTE: The VO has been edited.

    - SCREEN: 68036_Screenshot_3.mp4 01:13-01:21.

  - Navigate to File, Save As, and export the results as a CSV file [1]. NOTE: The VO has been edited.

    - SCREEN: 68036_Screenshot_3.mp4 01:22-01:42.

  - Now, import the CSV file into the Google Sheet [1]. Normalize the intensity value for each time point [2]. Calculate average intensity as F₀ (F-zero) for all images captures between 0 to 60 seconds [3]. Calculate F(t) (F-T), the average of the maximum fluorescence intensity as observed after the stimulus [4].

    - SCREEN: 68036_Screenshot_4.mp4 00:05-00:18.

    - SCREEN: 68036_Screenshot_4.mp4 00:36-01:13. Video Editor: Speed up the video.

    - SCREEN: 68036_Screenshot_4.mp4 01:20-01:34.

    - SCREEN: 68036_Screenshot_4.mp4 01:48-02:01.

  - Calculate the ΔF/F0 (Delta F over F-zero) for each ROI using the given equation [1,2]. Now, paste the data in the Graphpad (Graph-pad) Prism software and select scatter plot to visualize the data [3].

    - SCREEN: 68036_Screenshot_4.mp4 03:18-03:40.

    - TEXT ON PLAIN BACKGROUND: ΔF/F₀ = (F(t) - F₀) / F₀

    - SCREEN: 68036_Screenshot_4.mp4 03:47-03:49, 04:00-04:03, 04:08-04:11, 04:17-04:30, 04:36-04:40. Video Editor: Too many timestamps are added for this shot to avoid showing the OBS studio window.

# Results

- Representative Results

  - β-III (beta three) tubulin expression in undifferentiated and differentiated cells is presented in this figure [1]. From the results, it is evident that differentiated cells exhibited neuron-like morphology with elongated neurites [2], while undifferentiated cells retained a rounded shape [3].

    - LAB MEDIA: Figure 3.

    - LAB MEDIA: Figure 3. Video Editor: Highlight the right image.

    - LAB MEDIA: Figure 3. Video Editor: Highlight the left image.

  - Differentiated cells also showed an increased expression of TRPV1 (T-R-P-V-one) compared to undifferentiated cells [1].

    - LAB MEDIA: Figure 4. Video Editor: Highlight B and the dark green bar in C.

  - Cells treated with glycated collagen exhibited a higher capsaicin-induced calcium influx compared to normal collagen-treated cells [1].

    - LAB MEDIA: Figure 5B, 5C, 5D. Video Editor: Highlight the bottom two images in 5B, red graph in 5C, and the longer bar in 5D.

Pronunciation Guides:

1. Fluo-8

Pronunciation link:
https://www.howtopronounce.com/fluo-8
IPA: /fluː.oʊ eɪt/
Phonetic Spelling: floo-oh eight

2. Pluronic F127

Pronunciation link:
https://www.howtopronounce.com/pluronic
IPA: /plʊˈrɑː.nɪk/
Phonetic Spelling: ploo-rah-nik
Note: "F127" is said as "eff one twenty-seven"

3. Capsaicin

Pronunciation link:
https://www.merriam-webster.com/dictionary/capsaicin
IPA: /ˈkæpˌseɪsɪn/
Phonetic Spelling: kap-say-sin

4. Butterfly (scalp vein set)

Pronunciation link:
https://www.howtopronounce.com/butterfly-needle
IPA: /ˈbʌtərˌflaɪ/
Phonetic Spelling: buh-ter-fly

5. Confocal (Microscope)

Pronunciation link:
https://www.howtopronounce.com/confocal
IPA: /ˌkɑːnˈfoʊkəl/
Phonetic Spelling: kon-foh-kul

6. FIJI (Fiji software)

Pronunciation link:
https://www.howtopronounce.com/fiji
IPA: /ˈfiː.dʒi/
Phonetic Spelling: fee-jee

7. ROI (Region of Interest)

Pronunciation link:
https://www.howtopronounce.com/roi
IPA: /ɑːr.oʊˈaɪ/
Phonetic Spelling: ar-oh-eye

8. ΔF/F₀ (Delta F over F-zero)

Pronunciation link (Delta):
https://www.merriam-webster.com/dictionary/delta
IPA (Δ): /ˈdɛltə/
Phonetic Spelling: del-tuh
"F over F-zero" is pronounced as: eff over eff zero

9. TRPV1

Pronunciation link:
https://www.howtopronounce.com/trpv1
IPA: /tiː ɑːr piː viː wʌn/
Phonetic Spelling: tee-ar-pee-vee-one

10. β-III Tubulin

Pronunciation link for "tubulin":
https://www.merriam-webster.com/dictionary/tubulin
IPA: /ˈtuːbjəlɪn/
Phonetic Spelling: too-byuh-lin
"β-III" is pronounced as beta three