List and Parts of Movie to the Final Video Edition – JoVe June-25

Giulianno Molina

Orientation Protocol

- Sialendoscopy Technique

  - Now, place the patient in the supine position under general anesthesia, with the orotracheal tube on the side opposite to the affected gland [1].

    - Patient being positioned supine on the surgical table, with the anesthesia setup visible and orotracheal tube placed on contralateral side.

  - Dilate the salivary gland papilla progressively with dilators until it fits the appropriate size of the sialendoscope [1] and then insert the sialendoscope once proper size is achieved for clear duct visualization [2].

    - To be provided by authors: Talent using sequential dilators on the salivary papilla.

    - To be provided by authors:Talent inserting the sialendoscope gently into the duct.

  - Attach a 20-milliliter syringe filled with sterile 0.9 percent saline solution or isotonic water to the sialendoscope irrigation system [1].

    - Talent connecting the pre-filled syringe to the irrigation inlet of the sialendoscope.

  - Once the duct is distended, gently navigate the sialendoscope into it without using major maneuvers or twists [1].

    - To be provided by authors: Talent advancing the sialendoscope smoothly into the duct, maintaining gentle control.

  - Using the sialendoscope, inspect the main, secondary, and tertiary ducts in detail [1-TXT].

    - To be provided by authors: View through the sialendoscope exploring different levels of the duct system. TXT: Remove mucus plugs with saline irrigation

  - After cleaning, locate the salivary stone or stones inside the duct and prepare for stone capture maneuvers [1].

    - To be provided by authors: Clear ductal view revealing one or more salivary stones.

- Standardization of Basket Use in Sialendoscopy

  - Using the sialendoscope, inspect all duct extensions during the initial phase, checking for the presence of stenosis and evaluating it across the main, secondary, and tertiary ducts [1].

    - To be provided by authors: View through the sialendoscope examining various branches of the duct system, looking for signs of stenosis.

  - Observe the appearance and location of any duct stenosis and estimate its size in comparison to the main duct [1]. Also, look for stones located beyond the stenosis and assess the nature of the saliva, whether milky or clear [2].

    - To be provided by authors: Show the stenotic region to the main duct.

    - To be provided by authors: Close-up view showing the presence of a stone and the character of the surrounding saliva.

  - After the initial inspection, examine the main, secondary, and tertiary ducts directly to locate stones, making note of their exact positions within the ductal network [1].

    - To be provided by authors: Visual tracking of stone location throughout different duct levels.

  - Observe the size of each sialolith carefully and document whether the stones occupy the full duct diameter or are smaller [1].

    - Talent looking at the screen and pointing something. Authors, please simulate the image on the screen

  - Identify whether the stones are single or multiple, and plan the approach accordingly as a single or multiple attempt retrieval [1].

    - To be provided by authors: show multiple stones within the duct.

  - Flush the duct with sterile 0.9 percent physiological saline solution while maneuvering the sialendoscope to observe the stone mobility, and classify them as fixed, minimally mobile, or freely mobile [1].

    - To be provided by authors: Real-time movement of stones in response to irrigation, with visible fluid flow and commentary on mobility.

  - Select the appropriate basket type 3-wire, 4-wire, or 6-wire, based on the size and mobility of the stone, ensuring a better chance of successful capture [1].

    - To be provided by authors: Talent choosing a specific basket type from a sterilized tray.

- Different Approaches for the Technique

  - After completing the prior evaluation, place the tip of the basket directly against or very close to the anterior part of the sialolith for the Type A frontal approach [1]. Open the wires gently [2] and flush with sterile saline solution while attempting to trap the stone as it is carried into the basket [3].

    - To be provided by authors: Shot of Basket tip positioned in front of the sialolith, aligned for frontal engagement.

    - To be provided by authors: Shot of  Basket wires opening.

    - To be provided by authors: Shot of gentle saline flush moving the stone into the open wires.

  - Select the Type B side-to-side approach for relatively mobile, single stones smaller than the duct diameter [1]. Position the basket tip beside the stone, aligning the wires laterally [2]. Gently flush with sterile saline solution [3] and use soft lateral movement of the open basket to dislodge and capture the stone [4].

    - To be provided by authors: Shot of Basket tip placed to the side of a small mobile stone within the duct.

    - To be provided by authors: Shot of Gentle opening of wires next to the stone after alignment.

    - To be provided by authors: Shot of flushing saline.

    - To be provided by authors: Shot of Stone being caught by lateral movement and irrigation into the basket.

  - Use the Type C back-to-forward approach for stones that are as large as the duct or for fixed stones with low mobility [1]. Position the basket tip posterior to the sialolith and open the wires behind it [2] and bring the basket forward while flushing gently to wrap around the stone [3]. Once the wires enclose the stone, pull the basket wires to secure the stone [4].

    - To be provided by authors: Shot of Basket tip moved behind a large or fixed sialolith.

    - To be provided by authors: Shot of Opening of basket wires behind the stone.

    - To be provided by authors: Shot of Forward motion of basket and saline flush as wires approach the stone.

    - To be provided by authors: Shot of Wires tightening around the stone and pulling it securely.

  - Perform a final inspection of the main, secondary, and tertiary ducts before completing the procedure to check for residual stones or stenosis [1].

    - To be provided by authors:  Post-removal survey of the entire duct system with clear visualization.

Results

  - Endoscopic stone extraction was successfully performed in 100% of the Pure Sialendoscopy group [1], with all patients also receiving intraductal steroids [2].

    - LAB MEDIA: Table 1. Video editor: Highlight the row “Endoscopic Stone Extraction” with value “132 (100%)”

    - LAB MEDIA: Table 1. Video editor: Highlight the row “Intraductal Steroids” with value “132 (100%)”

  - Submandibular gland involvement was noted in 68.9% of patients [1].

    - LAB MEDIA: Table 1. Video editor: Highlight the row “Submandibular Stones” with value “91 (68.9%)”

  - Most patients presented with pure stones [1], and parotid stones were observed in 31.1% [2].

    - LAB MEDIA: Table 1. Video editor: Highlight the row “Pure Stones” with value “127 (96.2%)”

    - LAB MEDIA: Table 1. Video editor: Highlight the row “Parotid Stones” with value “41 (31.1%)”

  - Single stones were identified in 65.9% of cases, while multiple stones were seen in 34.1% [1].

    - LAB MEDIA: Table 2. Video editor: Highlight the rows “Single” and “Multiple” under “Stones Characteristics”

  - No major complications occurred, and no infections, dehiscence, or stuck baskets were reported [1].

    - LAB MEDIA: Table 2. Video editor: Highlight all rows under “Overall Post-operative Complications”

  - The mean sialendoscopy duration was 62 minutes, ranging from 48 to 98 minutes [1].

    - LAB MEDIA: Table 2. Video editor: Highlight the “Sialendoscopy Time Duration” row showing “62 (average)” and “48–98 (range)”

  - Stones were localized in the main duct in 73.5% of cases, and 26.5% were in secondary or tertiary ducts [1].

    - LAB MEDIA: Table 3. Video editor: Highlight the 2 rows under “Sialolithiasis Localization”

  - Basket types included 4-wire baskets in 68.9% of cases [1], 3-wire in 25.8% [2], and 6-wire in 5.3% [3].

    - LAB MEDIA: Table 3. Video editor: Highlight the row “4-WIRE”

    - LAB MEDIA: Table 3. Video editor: Highlight the row “3-WIRE”

    - LAB MEDIA: Table 3. Video editor: Highlight the row “6-WIRE”

  - The side-to-side technique was used in 35.6% of patients [1], while the back-to-forward technique was applied in 55.3% [2].

    - LAB MEDIA: Table 3. Video editor: Highlight the “Side-to-Side” row

    - LAB MEDIA: Table 3. Video editor: Highlight the row “Back-to-Forward” under “Approach Technique”

Note to authors:

Please upload the raw unedited protocol videos for the shots with the tag “To be provided by authors”.

Please name each file according to the shot number (3-digit-bullet, example, 3.2.1, 3.2.2).

But in case all the shots are in a single long video, that is okay, you may upload the entire video.

When you are ready to submit your video files, please contact our Content Manager, Utkarsh Khare.

In the second case where you upload a long video, you need to indicate the corresponding timestamps against each shot. For example if the action described in shot 3.2.1 is present in the file name xxx.mp4 and is in between timestamp 00:30 to 00:45 then write as following in the finalized script.

Example, shot 3.2.1 Talent performing ........... action.   xxx.mp4 00:30-00:45.

3.2.2 Talent keeping the sample in.................. xxx.mp4 01:00-01:25

Please note that every shot (3-digit step) should have the best corresponding video clip of only 20 to 30 seconds and not more than that, so that the voice narration can match the duration.

Author Answer Video Protocol Files

- Sialendoscopy Technique

1.2.1. To be provided by authors: Talent using sequential dilators on the salivary papila

1.2.1. Provided in Video# 1: timestamps 00:17-00:021 and 1:27-1:38

1.2.2. To be provided by authors:Talent inserting the sialendoscope gently into the duct

1.2.2. Provided in Video# 2: timestamps 00:02-00:21

1.4.1. To be provided by authors: Talent advancing the sialendoscope smoothly into the duct, maintaining gentle control.

1.4.1.  Provided in Video# 3: timestamps 00:05-00:18   AND   Video# 4:  timestamps 00:01-00:30

1.5.1. To be provided by authors: View through the sialendoscope exploring different levels of the duct system. TXT: Remove mucus plugs with saline irrigation

1.5.1.  Provided in Video #5: timestamps:  04:31-05:44

1.6.1. To be provided by authors: Clear ductal view revealing one or more salivary stones.

1.6.1.  Provided in Video #6: timestamps:  05:34-06:03

2. Standardization of Basket Use in Sialendoscopy

2.1.1.   To be provided by authors: View through the sialendoscope examining various branches of the duct system, looking for signs of stenosis.

2.1.1.  Provided in Video #7: timestamps:  07:36-08:18

2.2.1. To be provided by authors: Show the stenotic region to the main duct.

2.2.1.  Provided in Video #8: timestamps:  00:26-00:49

2.2.2. To be provided by authors: Close-up view showing the presence of a stone and the character of the surrounding saliva.

2.2.2.  Provided in Video #9: timestamps:  00:58-01:15

2.3.1 To be provided by authors: Visual tracking of stone location throughout different duct levels.

2.3.1.  Provided in Video #10: timestamps:  27:05-27:46

2.4.1 Talent looking at the screen and pointing something. Authors, please simulate the image on the screen

2.4.1.  Provided in Video #11: timestamps:  00:00- 00:32

2.5.1 To be provided by authors: show multiple stones within the duct.

2.5.1.  Provided in Video #12: timestamps:  40:13- 41:08

2.6.1 To be provided by authors: Real-time movement of stones in response

2.6.1.  Provided in Video #12: timestamps:  24:15- 24:40

2.7.1. To be provided by authors: Talent choosing a specific basket type from a sterilized tray.

2.7.1.  Provided in Video #13: timestamps:  00:00- 00:08  AND Video#14: timestamps: 00:00-00:08  AND Video#15: timestamps: 00:00-00:08  AND  Video#16: timestamps: 00:00-00:06

3. Different Approaches for the Technique

Type A frontal

3.1.1. To be provided by authors: Shot of Basket tip positioned in front of the sialolith, aligned for frontal engagement.

3.1.1.  Provided in Video # 17: timestamps:  01:22- 01:30

3.1.2.  To be provided by authors: Shot of  Basket wires opening.

3.1.2.  Provided in Video # 18: timestamps:  01:07- 01:19

3.1.3.  To be provided by authors: Shot of gentle saline flush moving the stone into the open wires.

3.1.3.  Provided in Video # 19: timestamps:  26:00- 26:46

Type B side-to-side

3.2.1.  To be provided by authors: Shot of Basket tip placed to the side of a small mobile stone within the duct.

3.2.1.  Provided in Video # 20: timestamps:  30:56- 31:08

3.2.2. To be provided by authors: Shot of Gentle opening of wires next to the stone after alignment.

3.2.2.  Provided in Video # 20: timestamps:  31:07- 31:21

3.2.3. To be provided by authors: Shot of flushing saline.

3.2.3.  Provided in Video # 20: timestamps:  33:41- 34:05

3.2.4. To be provided by authors: Shot of Stone being caught by lateral movement and irrigation into the basket.

3.2.4.  Provided in Video # 20: timestamps:  46:00- 47:09

Type C back-to-forward

3.3.1. To be provided by authors: Shot of Basket tip moved behind a large or fixed sialolith.

3.3.1.  Provided in Video # 21: timestamps:  05:27- 05:50

3.3.2. To be provided by authors: Shot of Opening of basket wires behind the stone.

3.3.2.  Provided in Video # 21: timestamps:  07:26- 07:43

3.3.3. To be provided by authors: Shot of Forward motion of basket and saline flush as wires approach the stone.

3.3.3.  Provided in Video # 21: timestamps:  08:06- 08:30

3.3.4. To be provided by authors: Shot of Wires tightening around the stone and pulling it securely.

3.3.4.  Provided in Video # 21: timestamps:  08:31- 09:15