# Submission ID #: 68124

# Scriptwriter Name: Sulakshana Karkala

# Project Page Link: https://review.jove.com/account/file-uploader?src=20777868

# Title: DNA-Barcode-Based Multiplex Immunofluorescence Imaging to Analyze FFPE Specimens from Genetically Reprogrammed Murine Melanoma

# Authors and Affiliations:

# Sachin S. Surwase1-3*, Xin Ming M. Zhou3-4*, Kathryn M. Luly1-3, Qingfeng Zhu5-7, Niki Talebian6, Robert A. Anders5-9, Jordan J. Green1-3,5,8-12, Stephany Y. Tzeng1-3, Joel C. Sunshine1,3-6,8,9

# 1Department of Biomedical Engineering, Johns Hopkins University

# 2Translational Tissue Engineering Center, Johns Hopkins University

# 3Johns Hopkins Translational ImmunoEngineering Center, Johns Hopkins University

# 4Department of Dermatology, Johns Hopkins University

# 5Department of Oncology, Johns Hopkins University

# 6Department of Pathology, Johns Hopkins University

# 7Convergence Institute, Johns Hopkins University School of Medicine

# 8Bloomberg Kimmel Institute for Cancer Immunotherapy, Johns Hopkins University School of Medicine

# 9Sidney Kimmel Comprehensive Cancer Center, Johns Hopkins University School of Medicine

# 10Institute for Nanobiotechnology, Johns Hopkins University

# 11Departments of Neurosurgery and Ophthalmology, Johns Hopkins University School of Medicine

# 12Departments of Materials Science & Engineering and Chemical & Biomolecular Engineering, Johns Hopkins University

*These authors contributed equally to this work

# Corresponding Authors:

Joel C. Sunshine			(joelsunshine@jhmi.edu)

# Email Addresses for All Authors:

Sachin S. Surwase			(ssurwas1@jhu.edu)

Xin Ming M. Zhou			(xzhou125@jh.edu)

Kathryn M. Luly			(kluly1@jhmi.edu)

Qingfeng Zhu				(qzhu6@jhmi.edu)

Niki Talebian                                           (ntalebi1@jh.edu)

Robert A. Anders			(raa@jhmi.edu)

Jordan J. Green			(green@jhu.edu)

Stephany Y. Tzeng			(stzeng1@jhmi.edu)

Joel C. Sunshine			(joelsunshine@jhmi.edu)

## Author Questionnaire

1. Microscopy: Does your protocol require the use of a dissecting or stereomicroscope for performing a complex dissection, microinjection technique, or something similar?  NO

2. Software: Does the part of your protocol being filmed include step-by-step descriptions of software usage?  Yes, all done

3. Filming location: Will the filming need to take place in multiple locations?   Yes, Same building, different floors.

Current Protocol Length

Number of Steps:  29

Number of Shots:  56

# Introduction

Videographer: Obtain headshots for all authors available at the filming location. Also obtain the names of the authors who deliver the introduction statements

  - Joel Sunshine: We present a protocol for developing and validating a multiplex immunofluorescence panel to study mouse FFPE tissues, and demonstrate its utility by studying samples from a melanoma model treated with nanoparticles delivering plasmid DNA encoding immunologic signals for reprogramming the tumor microenvironment.

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B.roll:2.4

What research gap are you addressing with your protocol?

  - Joel Sunshine: The field was missing rigorously developed and validated multiplex immunofluorescence protocols compatible with mouse formalin fixed paraffin embedded (FFPE) tissues.

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B.roll: 2.3

What advantage does your protocol offer compared to other techniques?

  - Sachin S. Surwase :  FFPE preserves tissue morphology long-term, is easy to handle and store, and enables creation of TMAs for viewing multiple specimens on a single slide.

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera.

How will your findings advance research in your field?

  - Xin Ming M. Zhou: Spatial proteomics imaging will enhance the visualization and analysis of the complex TME, ultimately aiding in the prediction of immune responses, which can contribute to the development of better therapeutics.

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera.

What research questions will your laboratory focus on in the future?

  - Joel Sunshine: We will be using this protocol to enable improved spatial profiling of mouse tumor models across many more tumor types and in different immune organs.

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera.

Videographer: Obtain headshots for all authors available at the filming location.

# Protocol

- Formalin-Fixed Paraffin-Embedded (FFPE) Tissue Staining and Imaging for Multiplex Analysis

Demonstrator: Sachin S. Surwase

Videographer’s Note: File B070_05051346_C026.mov was a false start on the camera and so was omitted from the upload

  - To begin, bake the FFPE (F-F-P-E) tissue slides at 60 degrees Celsius, overnight [1-TXT]. The next day, cool the slides at room temperature for 10 minutes before starting deparaffinization and rehydration [2].

    - WIDE: Talent placing FFPE tissue slides in a 60°C oven. TXT: FFPE: Formalin-Fixed Paraffin-Embedded

    - Talent removes the slides from the oven and places it on a tabletop.

  - Next, incubate the slides in xylene solution twice for 5 minutes each to remove paraffin [1]. Then rehydrate the tissue by moving the slides through 100% ethanol, twice, for 5 minutes each [2].

    - Talent placing slides in xylene for deparaffinization.

    - Talent transfers the slides to 100% alcohol. 
Videographer’s Note: Take 2 was better

  - Place the slides sequentially in 90%, 70%, 50%, and 30% ethanol solutions for 5 minutes each [1]. Wash the slides twice with distilled water for 5 minutes each [2]. Perform antigen retrieval [3] then use fresh bleaching solution to bleach the  tissue was twice for 45 min minute intervals [4] before performing antibody staining [5].

    - Talent sequentially immersing slides in different ethanol concentrations.

    - Talent placing the slides in distilled water.

Added Shot: Talent performing the antigen retrieval step.

Added Shot: Talent performing the tissue bleaching steps twice.
Videographer’s Note: Added shots have 2 shots each. First added shot is labelled 2.3.2 a,b and 2nd one is 2.3.2 c,d

    - Shot of antibody-stained slides.
Videographer’s Note: Last take is best

  - Next, remove the plastic chamber and store the collected antibody cocktail at 4 degrees Celsius to fix the stain [1]. Wash the slides twice in staining buffer for 2 minutes each [2].

    - Talent removing the plastic chamber and storing antibody cocktail.

    - Talent placing slides in staining buffer.

  - Incubate the slides in a dish containing 40 milliliters of post-staining fixing solution for 10 minutes [1-TXT]. Then rinse them with PBS three times for 2 minutes each [2].

    - Talent transferring slides to post-staining fixation solution. TXT: Post-Staining Fixing Solution: 4 mL 16% PFA, 36 mL storage buffer

    - Talent placing the slides in PBS.

  - Now transfer the slides into ice-cold methanol for 5 minutes before rinsing with PBS as before [1]. Apply 200 microliters of final fixative solution to each slide under a humidity chamber [2-TXT].

    - Talent immersing slides in ice-cold methanol.

    - Talent applying fixative solution to slides. TXT: Fixative solution: 20 µL/tube + 1 mL of PBS

  - After a 20-minute incubation at room temperature, wash the slides in PBS again, for 3 cycles of 2 minutes each [1].

    - Talent transfers the slides to PBS.

  - If imaging immediately, wipe the slide around the tissue using a lint-free tissue [1]. Press the flow cell for 30 seconds using flow cell assembly equipment [2].

    - Talent wiping slides.

    - Talent pressing the flow cell on the slide.

  - If imaging later, store the slide without applying the flow cell in the storage buffer at 4 degrees Celsius [1]. When ready to image, transfer the slides from storage buffer to PBS for 10 minutes before applying the flow cell [2].

    - Talent places the slide in storage buffer and keeps it at 4 degrees Celsius.

    - Talent transferring slides from storage buffer to PBS before imaging.

  - Next, prepare running and DMSO buffers based on the desired imaging cycles [1]. Prepare the reporter stock solution required for the total number of imaging cycles [2].

    - Talent preparing buffers. 
AND
TEXT ON PLAIN BACKGROUND:
Low DMSO (1:4) Buffer: 1-part DMSO with 4-parts running buffer
High DMSO (9:1) Buffer: 9-parts DMSO with 1-part running buffer
Video Editor: Please show both shots side by side

    - Shot of labelled prepared reporter stock solution.

  - Pipette 250 microliters of reporter stock solution and 5 microliters of each reporter into labelled black or amber 1-milliliter microcentrifuge tubes [1].

    - Talent pipetting reporter stock solutions into each tube.

Added Shot: Talent adding the 5 microliters of reporters into the tubes.

  - Transfer the solution to a black 96-well plate and seal it with adhesive foil [1].

    - Shot of solution being transferred to a black 96-well plate and sealed with adhesive foil.

  - Now, start the imaging device and use the instrument manager to set the parameters and exposure times [1]. Place the reporter plate in the device [2] and then place the slide in the microscope holder to begin imaging [3]. Acquire images of stained tissues [4]. 
NOTE: VO has been edited to accommodate the added shots

Added shot: Talent showing setting of exposure times.

AND

    - SCREEN: 68124_Section2_Screenshot_1.mp4	00:04-00:33
Video Editor: Please show both shots side by side
Added shot: Talent placing the reporter plate in the device
Added shot: Talent placed the slide in the microscope holder and began imaging

    - SCREEN: 68124_Section2_Screenshot_2.mp4	01:02-01:11

- Digital Pathology Analysis and Proteomics Data Normalization

Demonstrator: Xin Ming M. Zhou

  - Install the latest version of digital pathology analysis software [1]. Click Create Project and select a destination folder for the project space [2].

    - Talent installing/launching the latest version of the digital pathology analysis software.

    - SCREEN: 68124_Section3_Screenshot_1_1080p.mp4	00:00-00:14

  - Next, click on Add images followed by Choose files, then navigate to the QPTIFF (Q-P-tiff) file produced from multiplex immunofluorescence imaging [1]. Set the image type to Fluorescence, keep default settings, and click Import [2].

    - SCREEN: 68124_Section3_Screenshot_2_1080p.mp4	00:00-00:11

    - SCREEN: 68124_Section3_Screenshot_2_1080p.mp4	00:17-00:27

  - If using QuPath (Q-path), double-click the new image to open a workspace, then press File and Save periodically to track project changes [1]. Toggle marker visibility and viewing settings using the Brightness & Contrast tool in the toolbar [2].

    - SCREEN: 68124_Section3_Screenshot_3_1080p.mp4	00:00-00:11

    - SCREEN: 68124_Section3_Screenshot_3_1080p.mp4	00:15-00:29

  - Use the brush or wand tool to draw an annotation around the entire tissue section [1]. Define this annotation as Full_Tissue (Full-tissue) while excluding overlying skin or regions that should not be analyzed [2]. Hold the Alt key to create negative annotations or shrink boundaries [3].

    - SCREEN: 68124_Section3_Screenshot_4_1080p.mp4. 	00:00-00:15

    - SCREEN: 68124_Section3_Screenshot_4_1080p.mp4. 	00:16-00:24

    - SCREEN: 68124_Section3_Screenshot_4_1080p.mp4. 	00:26-00:34

  - Now choose the annotation by clicking the Annotations tab and pressing Objects > followed by Annotations... (Annotations) then click Duplicate selected annotations [1]. Turn on the SOX10 (socks-ten) channel, then shrink the duplicated annotation to define the Tumor region using Alt + brush/wand (Alt-plus-Brush-wand) tool [2].

    - SCREEN: 68124_Section3_Screenshot_5_1080p.mp4. 	00:00-00:08

    - SCREEN: 68124_Section3_Screenshot_5_1080p.mp4. 	00:18-00:24, 00:27-00:48

  - Now, select the Full_Tissue annotation, then go to Objects followed by Annotations... and Expand annotations [1]. Set the Expansion radius to 1 micrometer and click Run [2].

    - SCREEN: 68124_Section3_Screenshot_6_1080p.mp4. 	00:00-00:07

    - SCREEN: 68124_Section3_Screenshot_6_1080p.mp4. 	00:08-00:14

  - Rename this new annotation as Full_Tissue_Expansion [1]. Select the Tumor Annotation, right-click, and select Insert in hierarchy [2].

    - SCREEN: 68124_Section3_Screenshot_7_1080p.mp4. 	00:10-00:16

    - SCREEN: 68124_Section3_Screenshot_7_1080p.mp4. 	00:16-00:22

  - Re-select the Tumor annotation, then go to Objects , Annotations...  and now click Make inverse [1].  Define this new annotation as Stroma and repeat for all tissue sections [2].

    - SCREEN: 68124_Section3_Screenshot_8_1080p.mp4. 	00:00-00:08

    - SCREEN: 68124_Section3_Screenshot_8_1080p.mp4. 	00:09-00:20

  - To run cell segmentation, click on the Image tab to find the pixel width and image height [1].

    - SCREEN: 68124_Section3_Screenshot_9_1080p.mp4	00:00-00:12.

  - Download the StarDist (Star-Dist) extension from GitHub (Git-hub) [1]. Import the qupath-extension-stardist-[version].jar (Q-path-extension-stardist-version-dot-jar) file into QuPath [2]. Also download the StarDist groovy script files and model file from GitHub [3].

    - SCREEN: 68124_Section3_Screenshot_10_1080p.mp4. 	00:00-00:13

    - SCREEN: 68124_Section3_Screenshot_10_1080p.mp4. 	00:14-00:24

    - SCREEN: 68124_Section3_Screenshot_10_1080p.mp4. 	00:25-00:47

  - For each tissue section, select both Tumor and Stroma annotations [1]. In the settings bar, click Automate followed by Script Editor to open the script interface [2]. Open the appropriate StarDist cell segmentation script corresponding to the image pixel size [3]. When prompted, select stardist_cell_seg_model.pb (stardist-cell-seg-model-dot-P-B) for cell segmentation [4].

    - SCREEN: 68124_Section3_Screenshot_11_1080p.mp4	00:00-00:05.

    - SCREEN: 68124_Section3_Screenshot_11_1080p.mp4	00:06-00:10.

    - SCREEN: 68124_Section3_Screenshot_11_1080p.mp4	00:11-00:20

    - SCREEN: 68124_Section3_Screenshot_11_1080p.mp4	00:25-00:34.

  - After cell segmentation, save the image. Then sequentially click on Measure,  Export Measurements, select the Corresponding image(s), and set Export type as Cells and Separator as .csv (Dot-C-S-V) [1]. Choose an output file location, then click Populate to include relevant metrics [2].

    - SCREEN: 68124_Section3_Screenshot_12_1080p.mp4. 	00:00-00:21

    - SCREEN: 68124_Section3_Screenshot_12_1080p.mp4. 	00:22-00:32

  - For each lineage marker to be used in clustering or phenotyping, select one mean value to export [1-TXT].

    - SCREEN: 68124_Section3_Screenshot_13_1080p.mp4 	00:00-00:26
 TXT: Export Nucleus: Mean for nuclear markers and Cytoplasm:Mean for cytoplasmic markers

  - For normalization of proteomic data, open the exported CSV file and truncate the column headings to retain only the marker name [1].

    - SCREEN: 68124_Section3_Screenshot_14_1080p.mp4 	00:00-00:18

  - After installing the latest version of R and RStudio (R-Studio), download the Marker Normalization.R  (Marker-Normalization-dot-R) script from GitHub and run it to filter out cells based on nuclear size [1].

    - SCREEN: 68124_Section3_Screenshot_15_1080p.mp4 	00:00-00:27

  - Perform minimum-maximum normalization for each marker [1-TXT]. Set the lowest Mean Fluorescence Intensity or MFI (M-F-I) value in the range to 0 and the 99.7th percentile MFI value to 1 [2]. Clip any intensities above the 99.7th percentile to 1 [2]. Once complete, save the new CSV file with the normalized data [3].

    - SCREEN: 68124_Section3_Screenshot_15_1080p.mp4 	00:02-00:20 TXT: Set lowest MFI to 0 and 99.7th percentile to 1; Clip intensities above 99.7th percentile to 1

    - SCREEN: 68124_Section3_Screenshot_15_1080p.mp4 	00:21-00:34

# Results

- Representative Results

  - Antibody-DNA barcode conjugation was confirmed by protein gel electrophoresis, showing additional bands at the heavy chain region [1].

    - LAB MEDIA: Figure 4. Video editor: please highlight the “Heavy chain” band

  - Multiplex immunofluorescence imaging of B16F10 (B-sixteen-F-ten) flank tumors treated with 4-1BBL/IL-12 (four-one-B-B-L-I-L-twelve) nanoparticles and systemic anti-PD1 (Anti-P-D-one) showed distinct immune marker expression across tumor sections [1].

    - LAB MEDIA: Figure 5.

  - FlowSOM (flow-some) clustering identified a wider range of marker expression intensities compared to Seurat phenotyping, with a maximum intensity difference of approximately 0.7 versus 0.5 [1]. FlowSOM phenotyping also classified a higher number of macrophages into M1 and M2 subtypes [2].

    - LAB MEDIA: Figure 6A and B. Video editor: please show B first then A

    - LAB MEDIA: Figure 7. Video editor: please highlight the purple areas of the FlowSOM phenotyping image

  - Quantification of macrophage densities showed that FlowSOM detected higher densities of both M1 and M2 macrophages compared to Seurat [1], while Seurat classified more macrophages as "other" [2].

    - LAB MEDIA: Figure 8A-B. Video editor: please highlight the blue columns

    - LAB MEDIA: Figure 8C Video editor: please highlight the red column

  - Spatial analysis revealed that M2 macrophages and natural killer cells had the highest average minimum distances following treatment [1], and M1 macrophages were more prevalent around CD8 (C-D-Eight) T cells [2].

    - LAB MEDIA: Figure 9A. Video editor: Emphasize the columns of Target Cell Type corresponding to M2 and NK (blue columns)

    - LAB MEDIA: Figure 9B Video editor: Please highlight the CD8-M1 row

Pronunciation Guide:

- Immunofluorescence
Pronunciation link:
https://www.merriam-webster.com/dictionary/immunofluorescence
IPA: /ˌɪmjənoʊflʊˈrɛsəns/
Phonetic Spelling: ih-myoo-noh-floo-REH-sns

- Paraffin
Pronunciation link:
https://www.merriam-webster.com/dictionary/paraffin
IPA: /ˈpærəfɪn/
Phonetic Spelling: PAIR-uh-fin

- Murine
Pronunciation link:
https://www.merriam-webster.com/dictionary/murine
IPA: /ˈmjʊraɪn/
Phonetic Spelling: MYUR-ine

- Melanoma
Pronunciation link:
https://www.merriam-webster.com/dictionary/melanoma
IPA: /ˌmɛləˈnoʊmə/
Phonetic Spelling: mel-uh-NOH-muh

- QPTIFF
Pronunciation link: No confirmed link found
IPA: /kjuː-piː-tɪf/
Phonetic Spelling: KYOO-pee-tiff

- QuPath
Pronunciation link: No confirmed link found
IPA: /ˈkjuːpæθ/
Phonetic Spelling: KYOO-path

- SOX10
Pronunciation link: No confirmed link found
IPA: /sɑks tɛn/
Phonetic Spelling: SOCKS-ten

- Stroma
Pronunciation link:
https://www.merriam-webster.com/dictionary/stroma
IPA: /ˈstroʊmə/
Phonetic Spelling: STROH-muh

- StarDist
Pronunciation link: No confirmed link found
IPA: /stɑr-dɪst/
Phonetic Spelling: STAR-dist

- Proteomics
Pronunciation link:
https://www.merriam-webster.com/dictionary/proteomics
IPA: /ˌproʊtiˈɑmɪks/
Phonetic Spelling: proh-tee-AH-miks

- Electrophoresis
Pronunciation link:
https://www.merriam-webster.com/dictionary/electrophoresis
IPA: /ɪˌlɛktrəfəˈrisɪs/
Phonetic Spelling: ih-lek-troh-fuh-REE-sis

- FlowSOM
Pronunciation link: No confirmed link found
IPA: /floʊ-sɑm/
Phonetic Spelling: FLOH-sahm

- Seurat
Pronunciation link: No confirmed link found
IPA: /ˈsɜːrɑː/ (based on bioinformatics usage; differs from French painter /sɜːˈrɑː/)
Phonetic Spelling: SUR-rah

- Macrophages
Pronunciation link:
https://www.merriam-webster.com/dictionary/macrophage
IPA: /ˈmækrəˌfeɪdʒ/
Phonetic Spelling: MAK-roh-fayj

- CD8
Pronunciation link: No confirmed link found
IPA: /ˌsiːˌdiːˈeɪt/
Phonetic Spelling: SEE-DEE-eight

- Nanoparticles
Pronunciation link:
https://www.merriam-webster.com/dictionary/nanoparticle
IPA: /ˈnænoʊˌpɑrtɪkəlz/
Phonetic Spelling: NAN-oh-par-ti-klz

- Plasmid
Pronunciation link:
https://www.merriam-webster.com/dictionary/plasmid
IPA: /ˈplæzmɪd/
Phonetic Spelling: PLAZ-mid

- Tissue Microarray (TMA)
Pronunciation link (Tissue):
https://www.merriam-webster.com/dictionary/tissue
Pronunciation link (Microarray):
https://www.howtopronounce.com/microarray
IPA: /ˈtɪʃuː ˈmaɪkroʊəˌreɪ/
Phonetic Spelling: TISH-oo MY-kroh-uh-ray

- Antigen
Pronunciation link:
https://www.merriam-webster.com/dictionary/antigen
IPA: /ˈæntɪdʒən/
Phonetic Spelling: AN-tih-jen