# Submission ID #: 68141

# Scriptwriter Name: Sulakshana Karkala

# Project Page Link: https://review.jove.com/account/file-uploader?src=20783053

# Title: Investigating the Potential of Singly Curved Thin Piezoelectric Transducers for Energy Harvesting and Structural Health Monitoring

# Authors and Affiliations:

# Aleena V Krishnanunni1, Naveet Kaur2, Suresh Bhalla3

# 1Kerala State Electricity Board Limited

# 2CSIR-Central Road Research Institute, Delhi

# 3Civil Engineering Department, Indian Institute of Technology (IIT) Delhi

# Corresponding Authors:

Naveet Kaur 			(nkaur@crri.res.in)

# Email Addresses for All Authors:

Aleena V Krishnanunni 	(vkaleena@gmail.com)

Naveet Kaur 			(nkaur@crri.res.in)

Suresh Bhalla 			(sbhalla@civil.iitd.ac.in)

## Author Questionnaire

1. Microscopy: Does your protocol require the use of a dissecting or stereomicroscope for performing a complex dissection, microinjection technique, or something similar? No

2. Software: Does the part of your protocol being filmed include step-by-step descriptions of software usage? No

3. Filming location: Will the filming need to take place in multiple locations? Yes, 12 km apart

Current Protocol Length

Number of Steps:  14

Number of Shots: 27

# Introduction

Videographer: Obtain headshots for all authors available at the filming location.

Authors: Please note that only 5 statements may be presented. They have been edited for brevity

REQUIRED: What is the scope of your research? What questions are you trying to answer?

  - Aleena V Krishnanunni: The main scope of our research is to comprehensively evaluate our newly invented sensor concrete vibration energy harvester for energy harvesting as well as SHM as compared to the existing sensor concrete vibration sensor.

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B.roll:3.1

What are the most recent developments in your field of research?

  - Suresh Bhalla: Recent advances include low-cost sensors like concrete vibration sensor and concrete vibration energy harvester that replace expensive accelerometers for full 3D modal analysis, integrated seamlessly with AI, ML, and UAV technologies for real-life applications.

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B.roll:2.2

What technologies are currently used to advance research in your field?

  - Naveet Kaur: In our view, the futuristic sensing technologies are AI-ML integrated piezo, fibre optic and vision-based sensing.  Sensor fusion and Internet of Things holds a great promise for future.

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera.

What are the current experimental challenges?

  - Aleena V Krishnanunni: The development of stand-alone PEH powered autonomous miniature cloud centric data acquisition systems qualifying IoT sensing in true sense are a challenge.

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera.

What significant findings have you established in your field?

  - Suresh Bhalla: Our concrete vibration energy harvester invention, with a novel geometry-driven design, dramatically boosts energy harvesting and excels in SHM by capturing both global and local vibrations in real-life RC structures.

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera.

Videographer: Obtain headshots for all authors available at the filming location.

# Testimonial Questions (OPTIONAL):

# Videographer:

- Please ensure that all testimonial shots are captured in a wide-angle format, while also maintaining sufficient headspace, given that the final videos will be rendered in a 1:1 aspect ratio.

- Also, kindly note that testimonial statements will be presented live by the authors, offering their spontaneous perspectives.

How do you think publishing with JoVE will enhance the visibility and impact of your research?

  - Naveet Kaur (authors will present their testimonial statements live): When I first came to know about this journal, I was delighted to know that something like this journal also exists. We strongly believe that the our work published in this journal will have definitely have wider outreach among our peers and will surely pave way for new collaborations in the future.

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera.

Can you share a specific success story or benefit you’ve experienced—or expect to experience—after using or publishing with JoVE? (This could include increased collaborations, citations, funding opportunities, streamlined lab procedures, reduced training time, cost savings in the lab, or improved lab productivity.)NK

  - Suresh Bhalla: (authors will present their testimonial statements live): Visual form always offers quicker understanding. Hence, we expect more derivative research stemming out.

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera.

# Protocol

- Energy Harvesting and Storage Using Piezoelectric Transducers

Demonstrator: Aleena V Krishnanunni

  - To begin, cast a M5628-P2(M-Five-Six-two-Eight-P-Two) variant of the piezoelectric material in two cement mortar cylinders each with a diameter of 100 millimeters and a thickness of 30 millimeters [1].

    - WIDE: Talent holding a cast M5628-P2 piezoelectric patch in a mould.

AUTHORS: Please use an existing cast. This is only a representative action and does not need to be shown till completion.

  - Then cast a reinforced concrete beam using M30 grade concrete [1-TXT]. During casting, tie the concrete vibration energy harvester and the concrete vibration sensor to the top layer compression reinforcement of the beam, ensuring that their top surfaces align with that of the beam [2].

    - Talent casting a reinforced concrete beam using M30 grade concrete.TXT: Dimensions: 1500 mm [L] × 300 mm [B] × 100 mm [D]

    - Talent tying the CVS and CVEH to the reinforcement and ensuring flush alignment with the top surface.
AUTHORS: Please use an existing cast. This is only a representative action and does not need to be shown till completion.

  - Confirm that the electrical connections of the piezoelectric transducers remain protected throughout the casting process. Then, position the reinforced concrete beam in a supported setup with an effective span of 1100 millimeters for experimental testing [1].

    - Shot of the experimental setup
AUTHOR PLEASE UPLOAD THE LAB MEDIA TO YOUR PROJECT PAGE FOR THIS SHOT

  - Next, position a shaker at mid length of the beam [1].

    - Talent positioning the shaker at mid length of the beam

  - Now connect the concrete vibration sensor and concrete vibration energy harvester outputs to two channels of a oscilloscope [1]. Fix an accelerometer with a sensitivity of 100 millivolts per gram at the mid-span of the beam [2]. Then connect it to an ICP(I-C-P) amplifier [3].

    - Talent connects CVS and CVEH to 2 channels of a oscilloscope.

    - Talent installing the accelerometer at mid-span of the beam.

    - Talent connecting the accelerometer to an ICP amplifier.

  - Excite the reinforced concrete beam using the shaker in sweep mode and conduct the frequency domain analysis of the responses from the concrete vibration sensor, concrete vibration energy harvester and accelerometer [1-TXT].

    - Shot of the experimental setup. TXT: Excite beam between 25 Hz to 85 Hz; Step Interval: 15 Hz; Keep shifting shaker position to L/3 and L/6

  - Next, position an eccentric rotary-type shaker at a quarter of the beam length from the nearest support [1]. Connect and set up a speed controller to regulate the shaker [2].

    - Talent positioning the shaker at one-fourth the beam’s length from the nearest support.

    - Talent connecting and setting the speed controller.

  - To analyze the power generated by piezo transducers, first prepare a power measurement circuit by connecting two resistors, R1 (R-one)and R2 (R-Two), in series across the piezo sensors [1-TXT]. Connect a digital multimeter across R1 to monitor the output voltage V2 (V-two)[2]. Then connect the digital multimeter to a laptop for data acquisition [3].
Authors: Please create screen capture videos of the shots labeled as SCREEN, create a screenshot summary, and upload the files to your project page as soon as possible:https://review.jove.com/account/file-uploader?src=20783053

    - Talent assembling the power measurement circuit with labeled R1 and R2 resistors.TXT: R1 = 335 kΩ and R2 = 340 kΩ

    - Talent connecting a multimeter across R1.

    - SCREEN: Show multimeter connected to laptop displaying live voltage readings.

  - Start the shaker to induce vibrations in the reinforced concrete beam and adjust the acceleration using the speed controller [1]. Using the multimeter, record the output voltage V2 from each sensor [2-TXT]. Then calculate the power generated by each sensor at the different acceleration levels [3].

    - Talent turning on the shaker and gradually increasing the speed.

    - SCREEN: Show multimeter capturing voltage output for each sensor.TXT: Repeat vibration test for 2 or more shaker accelerations

    - TEXT ON PLAIN BACKGROUND:
									(3)

Where i is the current flowing through the circuit.

  - To assess the power storage potential of the piezo transducers, setup the experimental apparatus without the power measurement circuit [1]. Replace it with a simple full-bridge rectifier circuit [2].

    - Shot of prepared experimental setup.

    - Talent disconnecting the resistor network and attaching a rectifier module to the sensors.

  - Start the shaker and induce vibrations in the reinforced concrete beam[1]. Adjust the acceleration of the shaker using the speed controller [2].

    - Shot of the shaker being started and the RC beam undergoing vibrations.

    - Talent adjusting speed of the shaker with the speed controller.

  - Using the multimeter, measure and record the output voltage Vc (V-C)from the rectifier circuit for both sensors [1]. Calculate the total energy stored and the average power stored in the capacitor for both sensors using the given equations [2].

    - SCREEN: Show multimeter measuring rectified voltage from both sensors.

    - TEXT ON PLAIN BACKGROUND:

Where C is the capacitance (1000 μF), and Tc is the total charging time of the capacitors

  - For structural health monitoring, connect an LCR meter to the laptop for the measurement of electrical admittance signature [1].  Place a digital thermometer to ensure that a uniform room temperature is maintained throughout the experiment [2].

    - Talent connecting the LCR meter with the sensors

    - Talent placing the digital thermometer

  - Excite the sensors by applying a harmonic electric potential across them at high frequencies in sweep mode with the LCR meter [1]. Now induce damages in the beam by drilling holes one by one [2].

    - Shot of the laptop showing the readings. AND
TEXT ON PLAIN BACKGROUND:
Use two frequency ranges:
Case 1 : Coarser frequency range of 1 kHz to 1000 kHz with a step interval of 1 kHz 
Case 2: Finer frequency range of 100 kHz to 300 kHz with a step interval of 100 Hz

Video Editor: Please play both shots side by side

    - Shot of a hole being drilled into the beam.

# Results

- Results

# Authors: Please note that the current results section exceeds the 200-word limit. If you wish to add new results, kindly replace existing content to stay within the word count

  - In the frequency domain, the peak frequency was identified at approximately 55 Hertz for both transducers and the accelerometer [1], confirming the natural frequency of the beam [2].

    - LAB MEDIA: Figure 6C. Video editor: Highlight the peak of both the red dashed and black curves around 55

    - LAB MEDIA: Figure 6D. Video editor: Emphasize the single large peak in the orange plot around 55 on the frequency axis.

  - In sinusoidal excitation, CVEH generated a consistently higher voltage than CVS over the same time interval [1-TXT].Across various excitation frequencies and shaker positions, CVEH consistently produced a higher open circuit voltage than CVS, particularly at one-third and halfway positions from the beam support [2].

    - LAB MEDIA: Figure 7A. Video editor: Highlight the red dashed waveform (Curved)
TXT: CVEH: Concrete Vibration Energy Harvester; CVS: Concrete Vibration Sensor

    - LAB MEDIA: Figure 7B. Video editor: Emphasize the bars corresponding to shaker at L/3 and L/2

  - The power generated by CVEH increased substantially with acceleration, with power ratios rising from 8.36 at 15.19 meters per second squared [1], to 13.01 at 21.68 meters per second squared [2].

    - LAB MEDIA: Figure 8A. Video editor: Highlight the red waveform (CVEH)

    - LAB MEDIA: Figure 8C. Video editor: Highlight the red waveform (CVEH)

  - At a mid-point acceleration of 4.89 meters per second squared, CVEH charged the capacitor in 236 seconds [1], compared to 335 seconds by CVS [2].

    - LAB MEDIA: Figure 9A. Video editor: Highlight the point on the red curve (CVEH) reaching the dashed line at 236 s.

    - LAB MEDIA: Figure 9A. Video editor: Highlight the black curve (CVS) reaching 1.6 volts later at a time close to 335 seconds.

  - At a higher acceleration of 8.84 meters per second squared, CVEH stored 8.65 microwatts of power [1], approximately 1.68 times that of CVS at 5.14 microwatts [2].

    - LAB MEDIA: Figure 9C. Video editor: Highlight the taller striped bar labeled 8.6 under “Curved” for 8.84 m/s².

    - LAB MEDIA: Figure 9C. Video editor: Highlight the shorter solid brown bar labeled 5.1 under “Straight” for 8.84 m/s².

  - For structural health monitoring, deviation from baseline admittance signatures was more pronounced in the CVEH [1] than in the CVS, especially after successive damage stages [2].

    - LAB MEDIA: Figure 10A. Video editor: Emphasize the dashed curves for “1st damage” , “2nd damage” and  “3rd damage”

    - LAB MEDIA: Figure 10C. Video editor: Emphasize the dashed curves for “1st damage” , “2nd damage” and  “3rd damage”

  - The root mean square deviation values between healthy and damaged stages were consistently higher in the CVEH than in the CVS for both conductance [1] and susceptance [2].

    - LAB MEDIA: Figure 12A. Video editor: Highlight the first three brown bars under “Curved” for 1st, 2nd, and 3rd damage stages showing values of 57, 77, and 34.

    - LAB MEDIA: Figure 12B. Video editor: Highlight the corresponding bars under “Curved” showing RMSD values of 67, 91, and 40.