Submission ID #: 68153

Scriptwriter Name: Sulakshana Karkala

Project page Link: https://review.jove.com/account/file-uploader?src=20786713

Title: Biomechanical Method for Simulated Fall on an Outstretched Hand in a Human Cadaveric Model

Current Protocol Length

Number of Steps: 22

Number of Shots: 46

# Protocol

Ethics Title Card
Anonymized human cadaveric specimens were obtained through the Anatomy Gifts Registry (Hanover, MD).

Protocol

  - To begin, collect fresh frozen forearm specimens that have been disarticulated at the elbow from an anatomical donation registry [1-TXT]. Place the specimen on an absorbent pad to thaw at room temperature for 18 to 20 hours [2].

    - WIDE: Talent retrieving fresh frozen forearm specimens from storage. TXT: Store arms at - 80 °C until testing     Video: 68153_Video_01 (00:10-00:30)

    - Talent placing the specimen on an absorbent pad to thaw. 68153_Video_02 (00:05-00:13)

  - After thawing, remove any packaging and absorbent pads from the specimen [1]. Place the specimen on an absorbent pad for dissection [2-TXT].

    - Talent removing packaging and absorbent pads from the specimen. Video: 68153_Video_03 (00:16-00:56)

    - Talent placing the specimen on an absorbent pad.TXT: Dispose items that come in contact with the specimen in biohazard bags   68153_Video_03 (00:16-00:56)

  - Next, with a ruler and marker, measure and mark the location three inches proximally from the styloid process of the ulna [1]. Use a scalpel, surgical scissors, and rat-tooth forceps, and remove all soft tissue proximal to the marked location to expose the radius and ulna [2-TXT].

    - Talent measuring and marking the specimen with a ruler and marker. Video: 68153_Video_04 (00:03-00:22)

    - Talent using scalpel, surgical scissors, and rat-tooth forceps to remove soft tissue. TXT: Maintain all tissue and bone removed in accordance with cadaveric registry company until properly disposed    Video: 68153_Video_04 (00:22-03:35) and 68153_Video_05 (00:04-00:17)

  - Measure and mark the location six inches proximally from the styloid process of the ulna on the exposed radius and ulna [1]. Then, dissect and remove the radius and ulna sections proximal to this location [2]. Place the proximal end of the exposed radius and ulna into a table vise and tighten [3]. With an oscillating saw or handsaw, cut the exposed bone at the six-inch mark [4].

    - Talent measuring and marking six inches from the styloid process of the ulna. Video: 68153_Video_06 (00:03-00:30)

    - Talent dissecting and removing the radius and ulna sections.   NOTE: See videos for steps 1.4.3 and 1.4.4

    - Talent securing the specimen in a table vise. Video: 68153_Video_07 (00:02-00:59)

    - Talent cutting the exposed bone using an oscillating saw. Video: 68153_Video_08 (00:00-00:15) and 68153_Video_09 (00:03-00:34)

  - For specimen mounting, first seal the bottom of a three-inch by three-inch by three-inch square aluminum extrusion section with aluminum foil and laboratory tape [1]. Place the specimen against the wrist mounting fixture and secure it using trigger clamps [2-TXT].

    - Talent sealing the bottom of the aluminum extrusion with foil and lab tape. Video: 68153_Video_10 (00:00-01:59)

    - Talent positioning the forearm specimen against the wrist mounting fixture. TXT: Ensure the wrist mounting fixture is set at a 15° angle 68153_Video_11 (00:00-00:16), 68153_Video_12 (00:08-00:14), and 68153_Video_13 (00:00-00:05)

  - Position the radius and ulna so that they cross each other, mimicking a pronated hand position [1]. Center the specimen with the proximal end of the radius and ulna approximately 0.25 inches from the bottom of the extrusion [2].

    - Shot of the radius and ulna in a crossed position, resembling Figure 2C. Video: 68153_Video_14 (00:00-00:06)

    - Shot of the specimen centered in the extrusion. 68153_Video_15 (00:00-00:05) and 68153_Video_16 (00:00-00:06)

  - Now, mix self-curing acrylic per manufacturer specifications [1-TXT]. Fill the extrusion to 0.25 to 0.50 inches from the top [2].

    - Talent mixing self-curing acrylic. TXT: Mixing ratio: 1 scoop powder to 10 mL liquid; Total : 12 scoops powder, 120 mL liquid per specimen
68153_Video_17 (00:00-01:00), 68153_Video_18 (00:00-00:28), and 68153_Video_19 (00:00-00:21)

    - Talent pouring the acrylic into the extrusion. Video: 68153_Video_20 (00:20-00:48) and 68153_Video_21 (00:00-00:07)

  - After the acrylic has cured for 15 minutes, proceed for biomechanical testing [1].

    - Shot of the cured self-curing acrylic ready for testing. 68153_Video_22 (00:00-00:12) and 68153_Video_23 (00:00-00:21)

  - For biomechanical testing, place the biaxial stage on the material testing machine stage [1]. Secure the bottom portion with four M10 (m-ten) screws and the top portion with four M8 (m-eight) screws [2]. Attach the extrusion holder to the top portion of the biaxial stage using three M8 screws [3].

    - Talent placing the biaxial stage on the material testing machine stage.  Video: 68153_Video_24 (00:00-00:51) and 68153_Video_25 (00:00-00:49)

    - Shot of the bottom portion being secured with four M10 screws and the top being secured with four M8 screws. Video: 68153_Video_24 (00:00-00:51) and 68153_Video_25 (00:00-00:49)

    - Talent attaching the extrusion holder to the biaxial stage. Video: 68153_Video_26 (00:00-00:51)

  - Then fix the 10 kilonewton load cell to the actuator with six M10 screws [1]. Secure a three-inch by three-inch loading platen to the load cell with six M8 screws [2]. Now, insert the load and torsion transducers into the material testing machine frame [3] and turn on the machine [4].

    - Talent securing the load cell to the actuator. Video: 68153_Video_27 (00:00-01:44)

    - Talent attaching the loading platen to the load cell. Video: 68153_Video_28 (00:00-01:15)

    - Talent inserting transducers into the testing machine. Video: 68153_Video_29 (00:00-00:26)

    - Talent turning on the material testing machine. Video: 68153_Video_30 (00:15-00:25)

  - Launch the material testing machine software and perform the required system calibration of the load cell and set system limits [1]. Raise the actuator to its highest position to allow full use of the range of motion [2].

    - SCREEN: 68153_Screenshot_01 .MOV: 00:03-00:04, 00:18-00:23, 68153_Screenshot_02.MOV: 00:05-00:11

    - Talent raises the actuator to its highest position.   68153_Video_31 (00:00-00:20

  - Place the square extrusion section of the mounted specimen into the extrusion holder on the material testing machine stage [1]. Then tighten the two M4 set screws to secure the specimen [2].

    - Talent securing the specimen into the extrusion holder. Video: 68153_Video_32 (00:00-00:19)

    - Shot of the tightened M4 set screws. Video: 68153_Video_32 (00:00-00:19)

  - Move the biaxial stage to align the specimen with the actuator from left to right [1]. Align the specimen from front to back so that the loading platen covers the breadth of the palm [2-TXT].

    - Talent adjusting the biaxial stage for alignment. Video: 68153_Video_33 (00:00-00:41)

    - Shot of the specimen centered under the loading platen. TXT: Use the distal palmar crease as an anatomical landmark for alignment

Video: 68153_Video_34 (00:00-00:05)

  - Now, lower the material testing machine frame so that the loading platen is nearly touching the palm of the specimen [1]. Monitor the measured load in the software to ensure no force is applied to the specimen [2].

    - Shot of the testing frame being lowered to nearly touch the specimen palm.  Video: 68153_Video_35 (00:00-00:13) and 68153_Video_36 (00:07-00:12)

    - SCREEN: 68153_Screenshot_03.MOV  (00:00-00:05)

  - Review the testing setup before fracture induction to ensure that all screws on the plate are fully tightened [1]. Now, collect the fracture induction testing data at a Sampling Rate of 100 hertz [2]. Use an Absolute Ramp Function to pre-load the specimen at a Loading rate of 0.33 millimeters per second until a force value of two newtons is reached [2-TXT].  NOTE: The VO is edited for the deleted shot

    - Talent inspecting the testing setup.Video: 68153_Video_37 (00:00-00:13)

    - SCREEN: Navigate to data collection settings and set sampling rate to 100 hertz.  NOTE: Not required, not filmed

    - SCREEN: Video: 68153_Screenshot_04.MOV  (00:03-00:04) TXT: At 2 N, the system will autobalance to display compressive displacement

  - Next, use the Absolute Ramp Function to pre-load the specimen at 0.33 millimeters per second until a force of 50 newtons is reached [1]. Move away from the specimen at the same rate until a force of 10 newtons is reached [2]. Inspect the specimen before resuming or canceling the test when prompted [3].

    - SCREEN: 68153_Screenshot_04.MOV (00:04-00:22)

    - SCREEN: 68153_Screenshot_04.MOV : (00:22-00:25)

    - SCREEN: 68153_Screenshot_05.MOV  (00:00-00:05)

  - Ensure that the specimen wrist remains in extension with the loading platen in contact [1-TXT]. Click Resume in the pop-up prompt window [2].

    - Talent verifying wrist positioning during testing. TXT: If wrist moves into flexion, cancel test and restart Video: 68153_Video_38 (00:00-00:13)

    - SCREEN: 68153_Screenshot_05,MOV (00:05-00:06)

  - Then use the Absolute Ramp Function to load the specimen at a rate of 3.3 millimeters per second until failure occurs, with an End-of-ramp load value of 5000 newtons, adjusted for the load cell capacity [1].

    - SCREEN: 68153_Screenshot_04.MOV (00:36-00:44)

  - After fracture induction, raise the material testing machine frame and loosen the set screws [1]. Remove the specimen and cured self-curing acrylic from the aluminum extrusion [2].

    - Talent raising the machine frame post-test. Video: 68153_Video_39 (00:00-00:26)

    - Talent removing the fractured specimen from the extrusion. Video: 68153_Video_39 (00:26-00:40)

  - Collect a radiograph of the wrist to confirm successful fracture induction [1]. Analyze the force-displacement graph to assess stiffness, yield point, maximum force, fracture force, work to maximum force, and work to fracture force before further testing or disposal [2].

    - Shot of a Radiograph of fractured wrist.

    - SCREEN: 68153_Screenshot_06.MOV (00:00-00:08) and 68153_Screenshot_07.PNG (Image)