# Submission ID #: 68167

# Scriptwriter Name: Sulakshana Karkala

# Project Page Link: https://review.jove.com/account/file-uploader?src=20790983

# Title: Determination of Mitochondrial Morphology in Live Cells Using Confocal Microscopy

# Authors and Affiliations:

# Manna Lin1,2, Jiakang Wang1, Zihong Xian1, Chunyuan Zeng1, Jiangping Xu1

# 1NMPA Key Laboratory for Research and Evaluation of Drug Metabolism & Guangdong Provincial Key Laboratory of New Drug Screening, School of Pharmaceutical Sciences, Southern Medical University

# 2Central Laboratory, Southern Medical University

# Corresponding Authors:

Jiangping Xu				jpx@smu.edu.cn

# Email Addresses for All Authors:

Manna Lin				linmanna@i.smu.edu.cn

Jiakang Wang				wjk0125@i.smu.edu.cn

Zihong Xian				linus233@i.smu.edu.cn

Chunyuan Zeng			3190080011@i.smu.edu.cn

Jiangping Xu				jpx@smu.edu.cn

## Author Questionnaire

1. Microscopy: Does your protocol require the use of a dissecting or stereomicroscope for performing a complex dissection, microinjection technique, or something similar?  No.

2. Software: Does the part of your protocol being filmed include step-by-step descriptions of software usage?  Yes, all done

3. Filming location: Will the filming need to take place in multiple locations?   No.

Current Protocol Length

Number of Steps:  15

Number of Shots:  31

# Introduction

Videographer: Obtain headshots for all authors available at the filming location.

REQUIRED: What is the scope of your research? What questions are you trying to answer?

  - Manna Lin: Our research focuses on basic neuropharmacology. Specifically, we study how drug molecules work. We aim to understand the relationship between potential drug compounds and mitochondrial function in Parkinson's disease.

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera.

What research gap are you addressing with your protocol?

  - Manna Lin: Prior studies reveal long-standing live-cell analysis challenges even with manufacturer guidelines. To solve this, we outline a detailed, step-by-step protocol for assessing mitochondrial morphology in live cells.

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B.roll:4.2

What advantage does your protocol offer compared to other techniques?

  - Jiakang Wang: Our protocol uses a simple mitochondrial stain, fluorescence imaging, and open-source software. This reduces costs, increases accessibility, and makes it easier to use than complex alternatives like electron microscopy.

    - INTERVIEW: Named Talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B.roll:2.7

Videographer: Obtain headshots for all authors available at the filming location.

# Protocol

- MPP⁺ Stimulation and MitoTracker Staining of SH-SY5Y Cells After

Demonstrator: Manna Lin

  - To begin, obtain cultures of SH-SY5Y (S-H-S-Y-Five-Y) cells grown in flasks containing DMEM/F12 (D-M-E-M-F-Twelve), supplemented with 10 percent FBS [1].

    - WIDE: Talent taking out a culture flask from the humidified incubator. TXT: DMEM/F12: Dulbecco's Modified Eagle Medium/Nutrient Mixture F-12

  - Use 0.05 percent trypsin solution to detach the cells for 1 minute [1]. Then centrifuge the cells at 100 g for 3 minutes before passaging [2].
2.1.2. Talent adding trypsin solution to the flask.

    - Talent placing the tube in a centrifuge.

  - Next, suspend the SH-SY5Y cells in DMEM/F12 (D-M-E-M-F-Twelve) medium supplemented with 1% FBS [1]. Then seed the cells onto confocal dishes with glass bottoms and maintain them overnight [2].

    - Talent suspend SH-SY5Y cells in DMEM/F12.

    - Talent seeding the cells into confocal dishes.

  - For MPP+ (M-P-P-Plus) stimulation, replace the culture medium with fresh medium [1] with or without 1-methyl-4-phenylpyridinium for 24 hours [2].

    - Talent aspirating the old medium. TXT: MPP+ : 1-methyl-4-phenylpyridinium iodide

    - Talent adding MPP+ solution, and placing the dishes back in the incubator.

  - For mitoTracker staining, dilute MitoTracker stock solution with prewarmed DMEM-F12 medium to obtain a 50 nanomolar working concentration [1]. Keep the staining solution protected from light throughout the procedure [2].

    - Talent pipetting and mixing the MitoTracker solution into a tube with DMEM-F12.

    - Talent covering the staining solution tube with aluminum foil.

  - Remove the culture medium from the confocal dishes [1]. Wash the cells twice with fresh DMEM/F12 [2].

    - Shot of the culture medium being aspirated from the confocal dishes.

    - Talent adding fresh medium to the dishes, swirly it and removing the medium.

  - Then pipette 1 milliliter of the MitoTracker working solution into the dish [1]. Incubate for 15 minutes at 37 degrees Celsius in the dark [2].

    - Talent adding MitoTracker solution to dishes.

    - Talent placing them in a dark incubator.

  - Now, remove the MitoTracker working solution [1] and wash the dishes twice with DMEM/F12 [2]. After the last wash, add 1 milliliter of DMEM/F12 to each dish [3]. Incubate in the cell incubator before imaging [4].

    - Talent aspirating the dye.

    - Shot of DMEM-F12 being added to the dish.

    - Talent adding fresh medium.

    - Talent placing dishes back into the incubator.

- Live Imaging of Mitochondrial Dynamics in SH-SY5Y Cells

  - To preequilibrate the confocal microscope for imaging, under the acquisition panel, adjust the light path parameters by setting the excitation and emission wavelengths [1]. Set the pinhole size to 1.2 airy units, pixel scan size to 1,024 by 1,024, pixel dwell time to 2.4 microseconds with averaging of 2 [2].

    - SCREEN: 68167_screenshot_3.1.1	00:00-00:16.

    - SCREEN: 68167_screenshot_3.1.2	00:00-00:14

  - Now use the halogen light to focus on the cells using a plan apochromatic 60X objective lens with a numerical aperture of 1.4, minimizing phototoxicity [1]. Adjust the laser power and high voltage just below the saturation level, then set the zoom factor to 3 and capture the images [2].

    - Talent operating the microscope.

    - SCREEN: 68167_screenshot_3.2.2	00:00-00:30.

  - Move the region of interest to focus on other cells under the microscope, then capture images without changing any settings [1].

    - SCREEN: 68167_screenshot_3.3.1	00:10-00:35 .

  - To enhance image quality prior to binary transformation and skeletonization, first open the image [1]. Select an area of a single cell using the ROI (R-O-I) tool [2].

    - SCREEN: 68167_screenshot_3.4.1	00:00-00:10 .

    - SCREEN: 68167_screenshot_3.4.2	00:00-00:11 .

  - Now sequentially click on Process , Filters , Unsharp Mask function to enhance sharpness [1], then use Process and Enhance Contrast to reduce noise amplification [2]. Click on Process, Filters and the Median option to remove salt and pepper noise [3].

    - SCREEN: 68167_screenshot_3.5.1	00:00-00:13.

    - SCREEN: 68167_screenshot_3.5.2	00:00-00:10 .

    - SCREEN: 68167_screenshot_3.5.3	00:00-00:14 .

  - To convert the image to binary, click on Process followed by Binary and choose the Make Binary option [1]. Skeletonize the binary image by clicking Process, Binary and Skeletonize [2]. 
3.6.1. SCREEN: 68167_screenshot_3.6.1	00:00-00:10 .
3.6.2. SCREEN: 68167_screenshot_3.6.2	00:00-00:10 .

  - Then group all skeleton pixels by selecting Analyze followed by Skeleton and Analyze Skeleton (2D/3D) (Skeleton-2-D-3D) function [1]. Simplify the analysis by using the MiNA (ˈmiːnə) tool to calculate nine descriptive parameters in the skeletonized image [2].

    - SCREEN: 68167_screenshot_3.7.1	00:00-00:10.

    - SCREEN: 68167_screenshot_3.7.2	00:00-00:16

# Results

- Results

  - The fluorescence intensity of mitochondrial staining increased progressively with MitoTracker Red CMXRos concentration from 25 nanomolar to 200 nanomolar [1], but at 200 nanomolar, substantial cytoplasmic background staining was observed [2].

    - LAB MEDIA: Figure 1A. Video editor: Highlight the increasing brightness of the red signal across the four panels showing 25, 50, 100, and 200 nanomolar.

    - LAB MEDIA: Figure 1A. Video editor: Highlight the cell in the 200 nanomolar panel

  - Mitochondrial morphology was preserved better in DMEM/F12 medium compared to PBS at 1 hour post-staining [1], although DMEM/F12 also showed higher background fluorescence [2].

    - LAB MEDIA: Figure 1B. Video editor: Highlight the panel labeled "DMEM/F12 1 h"

    - LAB MEDIA: Figure 1B. Video editor: Highlight the visibly brighter red background in the "DMEM/F12 0 h" panel compared to "PBS 0 h".

  - Dual-frame averaging significantly improved the signal-to-noise ratio in mitochondrial imaging compared to single-scan mode [1], and increasing pixel dwell time from 2.4 to 10.8 microseconds resulted in brighter images but caused photobleaching [2].

    - LAB MEDIA: Figure 1C. Video editor: Highlight the top left image "average2x Dwell time2.4 Before" panel and then the  top right image "average1x Dwell time2.4".

    - LAB MEDIA: Figure 1C. Video editor: Highlight the intensely bright signal in "average2x Dwell time10.8" and the faded signal in the "After" panel directly below it.

  - Increasing high voltage intensified background noise [1] while increased laser power led to photobleaching [2].

    - LAB MEDIA: Figure 1D. Video editor: Sequentially highlight the images of the top row from left to right

    - LAB MEDIA: Figure 1D. Video editor: Sequentially highlight the images of the bottom row from left to right

  - The MiNA tool produced consistent skeletonization outputs for both dim and bright images, indicating robust segmentation across different fluorescence intensities [1], with the bright image exhibiting 5.5-fold higher intensity than the dim image [2].

    - LAB MEDIA: Figure 1E. Video editor: please show top row first then bottom row.

    - LAB MEDIA: Figure 1E. Video editor: Highlight the “Bright original image skeletonized” then the “Dim original image skeletonized”.

  - MPP+ treatment at 500 micromolar disrupted mitochondrial morphology in SH-SY5Y cells compared to the untreated control [1], resulting in a significantly reduced mitochondrial footprint [2].

    - LAB MEDIA: Figure 2A. Video editor: Show the right side panel images (top and bottom) then show control images

    - LAB MEDIA: Figure 2B. Video editor: Highlight the shorter bar labeled "MPP+ (500 µM)"