# Submission ID #: 68176

# Scriptwriter Name: Poornima G

# Project Page Link: https://review.jove.com/account/file-uploader?src=20793728

# Title: Spatula Montevideo Device for the Vitrification of Mammalian Embryos

# Authors and Affiliations:

# Geraldine Schlapp1*, María Noel Meikle1*, Lucía Goyeneche2, Pedro Dos Santos Neto3, Martina Crispo1

# 1Laboratory Animal Biotechnology Unit, Institut Pasteur de Montevideo

# 2Centro de Esterilidad Montevideo

# 3Fundación IRAUy, Instituto de Reproducción Animal de Uruguay

# *These authors contributed equally

# Corresponding Authors:

# Geraldine Schlapp		gschlapp@pasteur.edu.uy

# Email Addresses for All Authors:

María Noel Meikle		manomeikle@pasteur.edu.uy

Lucía Goyeneche		luciagoye@gmail.com

Pedro Dos Santos Neto 	santosnetopc@hotmail.com

# Martina Crispo		crispo@pasteur.edu.uy

# Geraldine Schlapp		gschlapp@pasteur.edu.uy

## Author Questionnaire

1. We have marked your project as author-provided footage, meaning you film the video yourself and provide JoVE with the footage to edit. JoVE will not send the videographer. Please confirm that this is correct.

√ Correct

2. Microscopy: Does your protocol require the use of a dissecting or stereomicroscope for performing a complex dissection, microinjection technique, or something similar?  Yes

If a dissection or stereo microscope is required for your protocol, please list all shots from the script that will be visualized using the microscope (shots are indicated with the 3-digit numbers, like 2.1.1, 2.1.2, etc.).

SCOPE shots: 2.4.1, 3.3.1, 3.3.2, 3.3.3, 3.5.1, 3.5.2

Authors, please use your microscope camera to film the SCOPE shots and upload the files to your project page as soon as possible: https://review.jove.com/account/file-uploader?src=20793728

3. Software: Does the part of your protocol being filmed include step-by-step descriptions of software usage?  No

4. Proposed filming date: To help JoVE process and publish your video in a timely manner, please indicate the proposed date that your group will film here: 06/02/2025

When you are ready to submit your video files, please contact our Content Manager, Utkarsh Khare.

Current Protocol Length

Number of Steps:  19

Number of Shots:  37 (6 Scope)

# Introduction

REQUIRED: What is the scope of your research? What questions are you trying to answer?

  - Geraldine Schlapp: We generate genetically modified mouse models and then cryopreserve the lines as a backup. For this, we need an easy, low-cost, efficient and reproducible method of vitrifying embryos.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: 2.3.1

What are the most recent developments in your field of research?

  - Geraldine Schlapp: Microinjection or electroporation of CRISPR-Cas9 could be done in vitrified-warmed mouse and livestock zygotes, resulting in a more flexible schedule and reducing the animals in use.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: 2.4.1

What technologies are currently used to advance research in your field?

  - María Noel Meikle: Embryo vitrification uses ultra-rapid cooling to prevent ice crystal formation, preserving cellular integrity. This technique advances cryobiology research, enabling studies on embryo viability, genetic stability, and long-term storage outcomes

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: 3.3.1

What advantage does your protocol offer compared to other techniques?

  - María Noel Meikle: Both the Spatula MVD and the solutions are home-made. The Spatula is sealed with a 0.5 mL straw, taking up minimal space in the nitrogen dewar.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: 3.4.1

Ethics Title Card

This research has been approved by the Institut Pasteur de Montevideo Animal Care and Use Committee (permit number #007-18) and the Institutional Animal Care Committee Fundación IRAUy (protocol #001–2017).

# Protocol

- Spatula Montevideo (MVD) Device Preparation

Demonstrator: María Noel Meikle

  - To begin, cut off the first 10 millimeters and the tip cone of a gel-loader tip [1].

    - WIDE: Talent using scissors or blade to cut off 10 millimeters and the tip cone of the gel-loader tip.

  - Using watchmaker's number 5 forceps, hold the end of the gel-loader tip approximately 1 centimeter from the tip [1] and gently place it into the Bunsen burner flame for 8 to 10 seconds to form a petal-like platform [2].

    - Talent holding the gel-loader tip with forceps 1 centimeter from the end.

    - Talent placing the tip into a Bunsen burner flame and maintaining it to form a flat platform.

  - Now, remove the tip from the flame and continue holding it tightly with the forceps for 5 seconds [1].

    - Talent removing the tip from the flame and maintaining grip with forceps for 5 seconds.

  - Using a stereomicroscope, check that the petal-like platform of approximately 1 square millimeter has formed and that the distal edge is sealed [1].

    - SCOPE: Close-up view through stereomicroscope showing the flat, sealed platform at the tip. Authors: We can show the visuals only if the corresponding narration is added.

  - Repeat the spatula fabrication steps to prepare the required number of spatulas, with each spatula capable of handling up to 40 mouse embryos [1].

    - Shot of multiple spatulas arranged on the work surface.

  - Sterilize the fabricated spatulas using ultraviolet rays [1-TXT] and store them in a sterile tube or box [2].

    - Talent placing the finished spatulas under a UV chamber. TXT: Alternatively, use ethylene oxide for sterilization

    - Talent transferring the sterilized spatulas to a sterile box.

  - On the day of vitrification, label as many rods as spatulas to vitrify with a permanent marker or a liquid nitrogen-resistant sticker label [1].  One by one, retrieve the spatulas, handling them from the opposite end of the platform [2], insert an identification rod into each spatula [3] and place them on the stereomicroscope base [4].

    - Talent labeling rods using permanent markers or stickers

    - Talent picking up sterilized spatulas by the opposite end of the platform.

    - Talent inserting identification rod into a spatula.

    - Talent placing spatulas on the microscope base.

- Embryo Dehydration and Vitrification Process

Demonstrator: Geraldine Schlapp

  - Pour liquid nitrogen into an insulated polystyrene container measuring 10 by 30 by 20 centimeters [1]. Place one 0.5 milliliter straw for each spatula into the container [2].

    - Talent pouring liquid nitrogen into the container.

    - Talent placing straws into the liquid nitrogen.

  - For each spatula, prepare drops of 100 microliters of M2 solution, 20 microliters of PV solution, and 20 microliters of V solution on a 100-millimeter plastic Petri dish [1].

    - Talent pipetting and placing the respective volumes of M2, PV, and V solutions onto the Petri dish.

  - Under the stereomicroscope, place 30 to 40 embryos into the M2 drop [1]. Using a pulled glass capillary attached to an aspirator tube assembly, pre-load the capillary with M2 medium and at least 1 microliter of PV solution [2]. Aspirate the embryos and incubate them in the PV solution drop for 30 seconds [3]. Load the capillary with at least 1 microliter of V solution [4] and transfer the embryos to the V solution drop, incubate for another 30 seconds at room temperature [5].

    - Talent transferring embryos into the M2 drop.

    - Talent preparing the glass capillary, attached to an aspirator assembly, by drawing M2 and at least 1 microliter of PV solution

    - Talent moving embryos into PV solution.

    - Talent loading the glass capillary with 1 microliter of V solution.

    - Talent transferring embryos into V solution.

  - Aspirate the embryos using minimal V solution, approximately 0.1 microliter [1], and quickly place them onto the spatula platform [2].

    - SCOPE: Talent aspirating embryos with the pre-loaded capillary using minimal V solution.

    - Talent transferring embryos quickly onto the spatula platform.

  - Immediately immerse the spatula platform into liquid nitrogen for 5 seconds while holding the spatula by its rod [1]. Retrieve a 0.5 milliliter straw in the container using long stainless-steel tweezers and remove the LN-2 from it [2]. Lift up the spatula a little bit, maintaining it in the LN2 vapor and  seal it with the straw [3].

    - Talent immersing the spatula platform into the liquid nitrogen container.

    - Talent removing LN2 from a straw before sealing to prevent expansion during warming

    - Talent sealing the spatula in a straw using tweezers while maintaining it in nitrogen vapor.

- Embryo Warming Process

Demonstrator: María Noel Meikle

  - On a 100-millimeter plastic Petri dish, place the required sucrose drops for each spatula to be warmed [1] and add three 100 microliter drops of M2 medium for washing [2].

    - Shot of adding the drops onto a plate. TXT: 500 µL drop of 0.5 M sucrose; 50 µL drop of 0.5 M sucrose; 50 µL drop of 0.25 M sucrose

    - Shot of adding three 100 microliter M2 medium drops on the same Petri dish containing sucrose drops.

  - Retrieve the sealed spatulas from the dewar [1] and place them in an appropriate container filled with liquid nitrogen [2].

    - Talent opening the dewar, removing a visotube containing spatulas.

    - Talent transferring the visotube into a container with liquid nitrogen.

  - Now, place the warming plate on the stereomicroscope [1]. One at a time, hold the base of the spatula with stainless-steel tweezers [2]. With a gloved hand, pull out the identification rod and then the straw [3]. Quickly dip the platform containing the embryos into the 500-microliter drop of 0.5 molar sucrose [4].

    - SCOPE: Talent positioning the warming plate on the microscope stage.

    - SCOPE: Talent grasping the spatula by its base using tweezers.

    - SCOPE: Talent pulling out the rod and straw.

    - Talent dipping the platform into the 0.5 molar sucrose drop.

  - Ensure all embryos fall into the sucrose drop; if necessary, gently move the spatula to aid their release [1]. Then, load the embryos into a pulled glass capillary [2] and place them into the 50 microliter drop of 0.5 molar sucrose solution and incubate for 2 minutes [3].

    - Talent adjusting the spatula to release embryos into the solution.

    - Talent collecting embryos with a glass capillary.

    - Talent transferring embryos to the smaller 0.5 molar sucrose.

  - Next, place the embryos into the 50-microliter drop of 0.25 molar sucrose solution and incubate for another 2 minutes [1].

    - SCOPE: Talent moving embryos to the 0.25 molar sucrose.

  - Finally, wash the embryos three times in separate drops of M2 medium to remove sucrose [1] and incubate them under conditions appropriate for the species for at least 1 hour before use [2].

    - Talent transferring embryos through three sequential M2 medium drops.

    - Talent placing embryos in an incubator.

# Results

- Results

  - In both in vivo derived 8-cell and in vitro produced 2-cell mouse embryos, spatula MVD resulted in a significantly higher survival rate compared to the slow freezing method [1], and a significantly higher embryo development rate at 3.5 days post coitum [2].

    - LAB MEDIA: Table 1. Video editor: Highlight the row labeled "Spatula MVD" under the column "Survival rate at 2.5 dpc".

    - LAB MEDIA: Table 1. Video editor: Highlight the "Embryo development rate at 3.5 dpc" value for Spatula MVD row.

  - Despite both cryopreservation methods yielding hatching rates, these were significantly lower [1] than in the control fresh embryo group [2].

    - LAB MEDIA: Table 1. Video editor: Highlight the "Hatching rate at 5.5 dpc" for all three groups.

    - LAB MEDIA: Table 1. Video editor: Highlight the "Hatching rate at 5.5 dpc" for  “Fresh embryos” row.

  - No significant differences were observed in pregnancy and birth rates between vitrified and warmed and fresh 2-cell embryos transferred to recipient mice [1].

    - LAB MEDIA: Table 2. Video editor: Highlight the pregnancy rate and birth rate columns.

  - In sheep embryos, survival and hatching rates at both 3 hours and 24 hours post-warming were similar between Spatula MVD and a commercial device [1], but both were significantly lower than those of the fresh embryo control group in terms of blastocyst development by day 8 [2].

    - LAB MEDIA: Table 3. Video editor: Highlight the “survival rates” columns for both Spatula MVD and Commercial device rows.

    - LAB MEDIA: Table 3. Video editor: Highlight the "Development rate" and "Hatching rate" for the Control group row.

  - Across various genetically modified mouse lines, Spatula MVD vitrification resulted in an average recovery rate of 90.4%, survival rate of 96.8%, pregnancy rate of 80%, and birth rate of 31.5%, with variability likely due to genetic background differences [1].

    - LAB MEDIA: Table 5. Video editor: Highlight the average row at the bottom.