# Submission ID #: 68182

# Scriptwriter Name: Poornima G

# Project Page Link: https://review.jove.com/account/file-uploader?src=20795558

# Title: Real-Time Force Measurement Between Emulsion Droplets During Enzymatic Breakdown

# Authors and Affiliations:

# Chenhui Song1, Yanlong Cai1, Qifei Ma2, Weihong Wang2, Yanxia Zou2, Huaizhou Jin1

# 1Key Laboratory of Quantum Precision Measurement, College of Physics, Zhejiang University of Technology

# 2College of Optical and Electronic Technology, China Jiliang University

# Corresponding Authors:

# Huaizhou Jin		18367198456@163.com

# Email Addresses for All Authors:

Chenhui Song		1401050966@qq.com

Yanlong Cai		840394023@qq.com

Qifei Ma		413519132@qq.com

Weihong Wang	994572339@qq.com

# Yanxia Zou		zouyanxia@163.com

# Huaizhou Jin		18367198456@163.com

## Author Questionnaire

1. Microscopy: Does your protocol require the use of a dissecting or stereomicroscope for performing a complex dissection, microinjection technique, or something similar?  No.

2. Software: Does the part of your protocol being filmed include step-by-step descriptions of software usage?  Yes.

If Yes, we will need you to record using screen recording software.

We recommend using the screen capture program OBS. JoVE’s tutorial for using OBS Studio is provided at this link: https://review.jove.com/v/5848/screen-capture-instructions-for-authors?status=a7854k

As these files are necessary for finalizing your script, please upload all screen-captured video files to your project page as soon as possible.

Videographer: Please record the computer screen for the shots labeled as SCREEN as back-up

3. Filming location: Will the filming need to take place in multiple locations?   No.

Current Protocol Length

Number of Steps:  7

Number of Shots:  26 (17 SC)

# Introduction

Videographer: Obtain headshots for all authors available at the filming location.

REQUIRED: What is the scope of your research? What questions are you trying to answer?

  - Huaizhou Jin: We use optical tweezers to measure real-time forces between individual emulsion droplets during enzymatic digestion to understand the stability during digestion process.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: 2.2.2

What are the current experimental challenges?

  - Huaizhou Jin: During the OT experiment, the digestion process is continuous. The optimal concentration of enzymes are difficult to determine, and if we fail to capture droplets in time, we need to restart the digestion all over again.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: 2.3.3

What significant findings have you established in your field?

  - Huaizhou Jin: The slope and maximum force between individual droplets are indeed tied to the oil type (SFA, MUFA, or PUFA based), digestion time, and enzyme type. It appears for digestion with protease oil type does not affect the digestion process.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: 4.1.1

What research questions will your laboratory focus on in the future?

  - Huaizhou Jin: Possibly an experiment that uses both spectroscopic monitoring and force monitoring to see the link between force and changes of chemical structures.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: 2.3.2

Videographer: Obtain headshots for all authors available at the filming location.

# Protocol

- Setting up the Optical Tweezers

Demonstrators: Chenhui Song, Huaizhou Jin

  - To begin, open the optical tweezers instrument [1]. Add approximately 70 microliters of deionized water to the lower objective lens [2] and insert the sample chamber into the instrument until a click is heard [3].

    - WIDE: Talent opening the optical tweezers instrument.

    - Talent using a pipette to add 70 microliters of deionized water to the lower objective lens.

    - Talent inserting the sample chamber.

  - Place a drop of immersion oil on the top surface of the sample chamber [1]. Then, using the condenser knob, gently lower the condenser lens until it lightly touches the top surface of the sample chamber [2]. Close the optical tweezers instrument to complete the setup before activating the laser [3].

    - Talent applying a drop of immersion oil onto the sample chamber.

    - Talent carefully lowering the condenser lens until contact is made.

    - Talent closing the optical tweezers instrument.

  - Now, turn on the laser [1] and set the power to 100 percent [2]. Adjust the lower knob on the optical tweezers system in a clockwise direction to locate the laser spot, moving from bottom to top [3]. Observe the laser spots three times [4] and set the Z-plane between the second and third sets [5]. Then, reduce the laser power to 30 percent [6].

    - SCREEN: Show the laser control interface and turning on the laser.

    - SCREEN: and set the power to 100 percent.

    - Talent adjusting the lower knob of the optical tweezers system.

    - SCREEN: View showing three laser spots.

    - SCREEN: Setting z-plane between the second and third sets of spots.

    - SCREEN: Show the laser power setting being reduced from 100 percent to 30 percent.

Authors: Please create screen capture videos of the shots labeled as SCREEN, create a screenshot summary, and upload the files to your project page as soon as possible (download the guidelines from the link given in the email): https://review.jove.com/account/file-uploader?src=20795558

- Measurement of the Force-Distance Curve for the Emulsion

  - Use the joystick to capture two droplets of similar size within the channel [1-TXT]. Adjust the distance between the two droplets to approximately 10 micrometers [2]. Fix the x and y positions of the right optical trap [3] and the y position of the left optical trap [4], then mark the droplets with magenta and green boxes, respectively [5].

    - Talent manipulating the joystick to trap two similar-sized droplets. TXT: Adjust the flow rates as needed

    - SCREEN: Show adjusting distance between the droplets.

    - SCREEN: Fixing the x and y positions of the right optical trap .

    - SCREEN: Fixing y position of the left optical trap .

    - SCREEN: Droplets being labeled with magenta and green boxes for right and left respectively.

  - Now, navigate to the Calibration menu [1] and select Measure to begin calibration [2]. If noise levels are excessive, repeat the measurement until satisfactory results are visible [3]. Select Apply to confirm the calibration [4].

    - SCREEN: Show cursor navigating to the Calibration menu.

    - SCREEN: Click on Measure under the calibration panel.

    - SCREEN: cursor hovering over high noise levels in data.

    - SCREEN: Final selection of Apply to confirm the calibration.

  - Next, clear the original data using the appropriate software interface [1] and measure the force-distance using the calibrated optical trap system [2].

    - SCREEN: Show clearing of existing data from the system interface.

    - SCREEN: Measuring the force-distance using the calibrated optical trap system .

  - Finally, add the trypsin solution to the system [1]. Adjust the settings to measure every 10 minutes, starting 10 minutes after adding the trypsin solution [2] to dynamically monitor the emulsions using the optical tweezer system [3].

    - Talent adding trypsin solution into the sample setup.

    - SCREEN: Interface showing setting being made to measure at 10-minute intervals.

    - SCREEN: Data being displayed after trypsin addition.

# Results

- Results

  - Emulsions containing rapeseed oil and whey protein, and milk fat and whey protein, were analyzed using optical tweezers [1]. For rapeseed oil emulsions without trypsin, peak interaction force increased with droplet size, reaching over 100 piconewtons at 8 micrometers [2], compared to approximately 10 piconewtons at 3 micrometers [3]. A similar trend was observed for milk fat emulsion droplets [4].

    - LAB MEDIA: Figure 5.

    - LAB MEDIA: Figure 5C.

    - LAB MEDIA: Figure 5A.

    - LAB MEDIA: Figure 5 D E F.  Video editor: Highlight F

  - In both emulsions, the mean peak interaction force increased with droplet diameter, showing a roughly linear relationship [1].

    - LAB MEDIA: Figure 8A and 8B. Video editor: Sequentially Highlight the bars from left to right to show the bars become taller. Do this for both A and B

  - Droplets with a diameter of 5 micrometers had the highest capture count among all sizes, indicating optimal trapping efficiency [1], while capture frequencies for 3 micrometer and 10 micrometer droplets were the low [2].

    - LAB MEDIA: Figure 9. Video editor: Highlight the tallest bar corresponding to “5” μm on the x-axis.

    - LAB MEDIA: Figure 9. Video editor: Highlight the bars corresponding to 3 μm and 10 μm on the x-axis.

  - After adding trypsin, milk fat emulsion droplets showed a gradual decline in peak interaction force from 31.9 piconewtons at 10 minutes [1] to 6.6 piconewtons at 60 minutes [2].

    - LAB MEDIA: Figure 6 (10min).

    - LAB MEDIA: Figure 6 (60min).

  - In rapeseed oil emulsions, trypsin treatment caused the force-distance curves to decline sharpy, suggesting that the efficiency of enzymatic action is more in the liquid oil phase [1].

    - LAB MEDIA: Figure 7 (40min).