# Submission ID #: 68200

# Scriptwriter Name: Poornima G

# Project Page Link: https://review.jove.com/account/file-uploader?src=20801048

# Title: Multimodal Analysis of Microplastics in Drinking Water using a Silicon Nanomembrane Analysis Pipeline

# Authors and Affiliations:

# Teagan Horan1, Samantha Romanick2, Gregory Madejski3, Ahmet Gurcan2, James McGrath2, James Roussie1, Kathryn Neville1

# 1SiMPore Inc.
2Department of Biomedical Engineering, University of Rochester 
3URnano, University of Rochester

# Corresponding Authors:

# Kathryn Neville					kneville@simpore.com

# Email Addresses for All Authors:

Teagan Horan						thoran@simpore.com

Samantha Romanick					sromanic@ur.rochester.edu

Gregory Madejski					g.madejski@rochester.edu

Ahmet Gurcan						agurcan@u.rochester.edu

James McGrath					james.mcgrath@rochester.edu

James Roussie						jroussie@simpore.com

# Kathryn Neville					kneville@simpore.com

## Author Questionnaire

1. Microscopy: Does your protocol require the use of a dissecting or stereomicroscope for performing a complex dissection, microinjection technique, or something similar?  No

2. Software: Does the part of your protocol being filmed include step-by-step descriptions of software usage?  No

3. Filming location: Will the filming need to take place in multiple locations?   No

Current Protocol Length

Number of Steps:  23

Number of Shots:  51

# Introduction

Videographer: Obtain headshots for all authors available at the filming location.

  - Kathryn Neville: Our research goals are to improve the analysis of microplastics present in multiple sample types, as well as improve the data generated from those particles of interest.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: 2.6.2

What are the current experimental challenges?

  - Kathryn Neville: Current microplastics analytical methods are prone to the introduction of extrinsic contamination. The methods we describe here eliminate transfer steps and alleviate this contamination problem.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: 3.3.1

What research gap are you addressing with your protocol?

  - Teagan Horan: We are providing a protocol utilizing a Silicon Nanomembrane that allows researchers to conduct Multimodal analyses of their particles of interest with increased efficiency and less contamination.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: 5.1.1

What advantage does your protocol offer compared to other techniques?

  - Teagan Horan: The analysis of microplastics is only as effective as the methods utilized; combining optical, electron, and spectroscopic imaging techniques allow for the fullest picture. Silicon nanomembranes enable these multiple analyses.

    - INTERVIEW: Named talent says the statement above in an interview-style shot, looking slightly off-camera. Suggested B-roll: 4.3.2

Videographer: Obtain headshots for all authors available at the filming location.

# Protocol

- Quality Control Procedures and Ultrapure Solution Preparations

Demonstrator: Teagan Horan

  - To begin, don 100 percent cotton laboratory coat and nitrile gloves [1].

    - WIDE: Talent putting on a 100 percent cotton laboratory coat and donning nitrile gloves.

  - Using 99 percent isopropyl alcohol, spray the nitrile gloves [1]. Rub the hands together thoroughly [2] and rinse with approximately 18 megaohm, 0.22 micrometer-filtered water [3-TXT].

    - Talent spraying nitrile gloves thoroughly with 99 percent isopropyl alcohol.  Videographer’s NOTE: 2.2.1 to 2.2.3 were merged into one shot with multiple takes and angles

    - Talent rubbing gloved hands together to spread the isopropyl alcohol evenly.

    - Talent rinsing gloved hands under a stream of filtered water.  TXT: Repeat this process 3x; Use this filtered water throughout

  - Fold a natural fiber delicate task wipe into quarters [1], then spray with 70% isopropyl alcohol [2]. Wipe the hood surface from back to front in long strokes, re-folding the delicate task wipe to an unused surface every two strokes. [3-TXT].

    - Talent folding a delicate task wipe into quarters.   Videographer’s NOTE: 2.3.1 to 2.3.3 were merged into one shot with multiple takes and angles

    - Talent spraying the folded wipe with 70 percent isopropyl alcohol.

    - Talent wiping the laminar flow hood surface from back to front using long, even strokes, and re-folding the wipe.  TXT: Repeat the wiping process if required

  - Now, roll a silicone mat across the surface of the hood to pick up any remaining particles [1]. Spray the silicone roller with 99 percent isopropyl alcohol and scrub using a gloved hand [2]. Rinse the roller with filtered water [3]. After repeating the cleaning process three times, allow the roller to air-dry inside the hood [4-TXT].

    - Talent rolling the mat across the hood surface to pick up debris.  Videographer’s NOTE: 2.4.1 AND 2.4.2 were merged into one shot

    - Talent spraying the silicone roller with 99 percent isopropyl alcohol and scrubbing with a gloved hand.

    - Talent rinsing the silicone roller under filtered water.

    - Talent setting the roller aside in the hood to air-dry. TXT: Re-rinse the nitrile gloves as demonstrated earlier

  - To generate ultrapure water and isopropyl alcohol, fill a 1-liter beaker with the 18 megaohm water under the hood [1]. Prime a 60-milliliter syringe and attached 0.22-micrometer cut-off syringe filter by pushing at least 200 milliliters of filtered water through the syringe and filter assembly [2].

    - Talent filling a 1-liter beaker with filtered water under the hood.

    - Talent adding/drawing the filtered water to a syringe attached to a filter.

  - Then, rinse a glass screw-cap container three times with filtered water [1] and fill the container with syringe-filtered 18 megaohm, 0.22 micrometer-filtered water [2].

    - Talent rinsing a glass screw-cap container three times with filtered water.

    - Talent filling the rinsed container with syringe-filtered water.

  - Repeat the beaker filling, syringe priming, and container rinsing steps using the desired percentage concentration of isopropyl alcohol instead of water to generate ultrapure isopropyl alcohol [1].

    - Talent displaying final isopropyl alcohol obtained after processing.

- Particle Capture from Liquid Samples via Filtration

  - Don personal protective equipment and nitrile gloves [1]. Spray a silicone gasket with ultrapure 99 percent isopropyl alcohol [2] and scrub the gasket with gloved fingers [3]. Then, rinse the gasket with ultrapure water [4-TXT].

    - Talent putting on a PPE and nitrile gloves.

    - Talent spraying a silicone gasket thoroughly with ultrapure 99 percent isopropyl alcohol.

    - Talent scrubbing the silicone gasket with gloved fingers.

    - Talent rinsing the silicone gasket with ultrapure water. TXT: Repeat the IPA wash 3x for each gasket

  - First, generate the process blank. Utilizing the primed syringe, uptake 30 milliliters of ultrapure water and 30 milliliters of air into the 60-milliliter syringe [1]. Screw on a syringe filter [2]. Shake the syringe vigorously [3] and dispense the liquid and air through the filter [4].

    - Talent drawing 30 milliliters of ultrapure water and 30 milliliters of air into the 60 milliliter syringe.

    - Talent screwing a syringe filter onto the syringe.

    - Talent shaking the syringe vigorously to mix.

    - Talent dispensing the contents through the syringe filter.

  - After rinsing 3 times, assemble the filtration apparatus according to the visual assembly graphic [1] [2].

    - Talent assembling the filtration apparatus carefully.

    - LAB MEDIA: Figure 1B Video Editor: Please use split screen to show both the shots of this step

  - Turn on the vacuum to the filtration apparatus to create a negative flow through the filter disk stack [1].

    - Talent turning on the vacuum and confirming negative flow through the stack.

  - To measure the background contamination of the process blank, dispense 50 milliliters of ultrapure water slowly over the nanomembrane in the center of the top disk using the rinsed syringe [1].  Allow the ultrapure water to filter through [2-TXT]. Once the sample is dry, turn off the vacuum [3].

    - Talent dispensing 50 milliliters of ultrapure water over the nanomembrane using the rinsed syringe.

    - Shot of the ultrapure water filtering through the nanomembrane.  TXT: After filtration, keep the vacuum on for 1 min

    - Talent turning off the vacuum once the sample is fully dry. Videographer’s NOTE: along with other Vacuum shots, this was not that captivating so I shot various angles and tried methods to make the shot somewhat interesting.

  - Using clean tweezers, carefully remove the filter disks from the gaskets [1] and place them into a clean, labeled container such as a glass Petri dish or a darkened box [2].  Image the filter disks under microscopy for optical analysis and particle counting [3].

    - Talent removing filter disks carefully with tweezers.

    - Talent placing the filter disks into a clean and labeled container.

    - Talent placing the filter disks under a microscope.

  - For experimental liquid samples, repeat the syringe rinsing process with an additional cleaned gasket and syringe-filter unit [1].  Next, uptake the desired amount of the new sample [2] and dispense the sample slowly over the nanomembrane in the center of the top disk [3].

    - Talent rinsing the syringe. Videographer’s NOTE: A pipette was used here

    - Talent drawing the desired amount of sample into the syringe.

    - Talent dispensing the sample slowly over the nanomembrane.

  - Once sample filtration is complete, rinse the membrane three times with 1 milliliter of ultrapure water [4-TXT].

    - Talent adding the membrane with 1 milliliter of ultrapure water each time. TXT: Vacuum and dry the sample

- Fluorescent Dye Preparation, Staining and Data Acquisition

  - Rinse two glass screw cap containers three times with ultrapure water [1].  Prepare a 0.1 milligram per milliliter solution of Nile Red in ultrapure 99 percent isopropyl alcohol in a clean glass container [2]. Gently invert the container ten times to mix the solution [3].  Filter the Nile Red solution into the second glass screw cap container [4].

    - Talent adding ultrapure water to two glass screw cap containers.

    - Talent adding Nile Red to a clean glass container.

    - Talent gently inverting the glass container.

    - Talent adding the prepared Nile Red solution into the filtration unit placed in a second clean glass screw cap container.

  - Place the filter disk to be stained onto the support frit of the vacuum collection flask [1] and pipette 20 microliters of the 0.1 milligram per milliliter Nile Red solution onto the nanomembrane at the center of the filter disk [2].

    - Talent placing the filter disk carefully onto the support frit of the vacuum collection flask.

    - Talent pipetting and dispensing 20 microliters of Nile Red solution precisely onto the center of the nanomembrane.

  - Incubate the stain on the nanomembrane for 5 minutes [1] and then vacuum filter the stain [2].

    - Talent placing the sample aside.

    - Talent initiating vacuum filtration to remove the stained solution.

  - Rinse the filter disk three times with 1 milliliter of ultrapure 99 percent isopropyl alcohol to remove excess Nile Red stain [1].

    - Talent adding the filter disk with 1 milliliter of ultrapure 99 percent isopropyl alcohol each time.

  - Allow the filter disk to sit on the support frit with the vacuum on for 2 minutes to filter and dry any residual liquid [1]. If it still does not dry after 2 minutes, transfer it to a 70-degree Celsius oven for 2 to 5 minutes using a clean glass Petri dish [2].

    - Shot of the filter disk on the frit under vacuum.

    - Talent transferring the filter disk into a 70-degree Celsius oven.

  - For particle quantification, immobilize the filter disk on a microscope slide using a silicone gasket [1] and move it onto the microscope stage [2].

    - Talent placing the filter disk onto a microscope slide using a silicone gasket.

    - Talent transferring the sample to the microscope stage.

  - Image the nanomembrane using brightfield illumination so that the maximum detected counts are approximately 90% of the detector camera’s maximum range [1].

    - Shot of the talent working at the microscope to capture brightfield images of the nanomembrane.

  - Image the nanomembrane using fluorescent illumination so that the maximum pixel intensities are around 25% of the detector camera’s maximum range [1].

    - Shot of the computer screen showing fluorescent images of the nanomembrane being captured. Videographer’s NOTE: Filming screen was not possible as no real experiment was going on. I filmed various shots of talent at computer screens and scope to be able to utilize for 4.8.1 and 4.9.1

  - Finally, save the acquired images as a 16-bit composite TIFF file [1].

    - Shot of the computer screen showing the images on the software interface being saved in 16-bit composite TIFF format. Videographer’s NOTE: Filming screen was not possible as no real experiment was going on. I filmed various shots of talent at computer screens and scope to be able to utilize for 4.8.1 and 4.9.1

# Results

- Results

  - Bare silicon nitride and gold-coated silicon nitride nanomembranes were suitable for specific analysis types. Bare silicon nitride was suitable for transmission based optical techniques as well as spectroscopy [1], while gold coated silicon nanomembranes were suitable for reflection-based techniques [2].

    - LAB MEDIA: Figure 1. Video editor – emphasize the panel for “Pipeline A”

    - LAB MEDIA: Figure 1. Video editor – emphasize both panels for “Pipeline B” and “Pipeline C”

  - An ideal cascade of data is shown that was generated off a single silicon nanomembrane [1]. Suspected microplastic particles stained with Nile Red indicated that the tested tap water samples had a significantly higher count of particles greater than 20 microns compared to the 8 to 20-micron subfraction [2].

    - LAB MEDIA: Figure 3

    - LAB MEDIA: Figure 3E. Video editor: Emphasize the Dark red bars (20 micrometer).

  - Raman spectra collected with an 830-nanometer laser had a high correlation coefficient on the same particle [1] analysed with optical microscopy [2]. Spectra revealed that the particle was comprised of Polyethylene [3].

    - LAB MEDIA: Figure 3C.

    - LAB MEDIA: Figure 3A.

    - LAB MEDIA: Figure 3C.

  - Scanning electron microscopy revealed detailed morphological features of particles captured on a silicon nanomembrane [1].

    - LAB MEDIA: Figure 3D.

  - Energy-dispersive X-ray spectroscopy analysis showed that the main particle composition was primarily carbon and nitrogen. This, along with a Trypan-blue stain uptake, suggests that the particle is likely organic in origin [1].

    - LAB MEDIA: Figure 3F.

  - Suboptimal sample preparation yielded unclear data. Improperly rinsed Nile Red stain makes particle identification difficult [1] and suboptimal Raman spectra with a low correlation coefficient was obtained, suggesting that the particle's chemical identity cannot be reliably confirmed [2].

    - LAB MEDIA: Figure 5A.

    - LAB MEDIA: Figure 5B

Pronunciation Guide

### 1. Isopropyl

Pronunciation link:
https://www.merriam-webster.com/dictionary/isopropyl
IPA: /ˌaɪsəˈproʊpəl/
Phonetic Spelling: eye-suh-proh-puhl

### 2. Micrometer

Pronunciation link:
https://www.merriam-webster.com/dictionary/micrometer
IPA: /maɪˈkrɑːmɪtər/ (note: this is for the measurement instrument; used in context, it likely refers to a micrometric unit, pronounced /ˈmaɪkroʊˌmiːtər/)
Phonetic Spelling: my-kroh-mee-ter

### 3. Megaohm

Pronunciation link:
https://www.howtopronounce.com/megaohm
IPA: /ˈmɛɡəˌoʊm/
Phonetic Spelling: meh-guh-ohm

### 4. Laminar

Pronunciation link:
https://www.merriam-webster.com/dictionary/laminar
IPA: /ˈlæmɪˌnɑːr/
Phonetic Spelling: la-muh-nar

### 5. Nanomembrane

Pronunciation link:
https://www.howtopronounce.com/nanomembrane
IPA: /ˈnænoʊˌmɛm.breɪn/
Phonetic Spelling: nan-oh-mem-brayn

### 6. Filtration

Pronunciation link:
https://www.merriam-webster.com/dictionary/filtration
IPA: /fɪlˈtreɪʃən/
Phonetic Spelling: fil-tray-shun

### 7. Syringe

Pronunciation link:
https://www.merriam-webster.com/dictionary/syringe
IPA: /səˈrɪndʒ/
Phonetic Spelling: suh-rinj

### 8. Petri

Pronunciation link:
https://www.merriam-webster.com/dictionary/Petri%20dish
IPA: /ˈpiːtri/
Phonetic Spelling: pee-tree

### 9. Spectroscopy

Pronunciation link:
https://www.merriam-webster.com/dictionary/spectroscopy
IPA: /spɛkˈtrɑːskəpi/
Phonetic Spelling: spek-trah-skuh-pee

### 10. Trypan-blue

Pronunciation link:
https://www.howtopronounce.com/trypan-blue
IPA: /ˈtrɪpæn bluː/
Phonetic Spelling: trip-an bloo

### 11. Polyethylene

Pronunciation link:
https://www.merriam-webster.com/dictionary/polyethylene
IPA: /ˌpɑːliˈɛθəˌliːn/
Phonetic Spelling: pah-lee-eth-uh-leen

### 12. Nile Red

Pronunciation link:
https://www.howtopronounce.com/nile-red
IPA: /naɪl rɛd/
Phonetic Spelling: nile red

### 13. Silicon

Pronunciation link:
https://www.merriam-webster.com/dictionary/silicon
IPA: /ˈsɪlɪˌkɑːn/
Phonetic Spelling: sil-ih-kahn

### 14. Raman

Pronunciation link:
https://www.howtopronounce.com/raman
IPA: /ˈrɑːmən/
Phonetic Spelling: rah-mun